3GPP TSG-SA3 Meeting #123	S3-252829

Göteborg, Sweden, 25 – 29 August 2025

Title:	LS on Issues related to paging all devices, and concurrent paging procedures

Release:	Rel-19

Work Item:	Ambient power-enabled Internet of Things (AmbientIoT)

Source:	Ericsson, to be SA3#123

To:	RAN2, RAN3, SA2, CT1

Cc:	-

Contact person:	Mohsin Khan

mohsin.a.khan@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

# 1	Overall description

SA3 has discussed the following issues related to paging all devices, and concurrent paging procedures.

Issue related to paging all devices

According to the Ambient IoT running CR to TS 38.300, R2-2504398, clause 16.x.5.2, it is currently possible to include no paging identifier in the paging request. This corresponds to paging all devices. The option to include no paging identifier is also captured in the running CR R3-253100 to TS 38.413 in clause 9.3.3.xx5 under remit of RAN3, and in documents under SA2 and CT1 remit based on the agreements in RAN2 and RAN3. (Clause 5.2.3 of TS 24.369 under the remit of CT1 allows response to paging based on indication from lower layers as specified in TS 38.391, which corresponds to paging all devices due to the option of indicating to the upper layers that the device is selected based on absent Paging ID field in clause 5.2 of TS 38.391.)

However, this option of paging all devices can cause a denial-of-service attack on the network. Due to SA3's agreement to not authenticate the network towards the AIoT device for inventory, the paging is not protected. Hence, an attacker can replay the paging request, and devices will respond to the paging request. The network will be flooded with responses and not be able to differentiate between intended and not intended responses.

The same issue appears when a large group of devices, e.g. 90% of the devices, is paged.

Consequence: Due to the issue described above, it should not be allowed to page all devices, i.e. include no identifier in the paging request. It should also not be allowed to page large groups of devices.

Issue related to concurrent paging procedures

There is an agreement made in RAN2 (RAN2#130 meeting): "The Rel-19 device always responds to the new service indicated by the received paging message applicable for that device.".

This means that, if a device receives a new paging request during an ongoing procedure, it will abort the ongoing procedure and respond to the new paging. This can cause a denial-of-service attack on the network. An attacker can send new paging requests and thus prevent the device from replying to the genuine network's request. The underlying reason is the same as for the issue related to paging all devices, i.e. due to SA3's agreement to not authenticate the network towards the AIoT device for inventory, the paging is not protected.

Consequence: Due to the issue described above, the agreement at the RAN2#130 should be reverted. When a device receives a new paging, it should not interrupt the ongoing procedure.

# 2	Actions

To RAN2, RAN3, SA2, CT1

ACTION: 	SA3 respectfully requests RAN2, RAN3, SA2 and CT1 to take the above-mentioned issues into account and update their specifications accordingly.

# 3	Dates of next TSG SA WG 3 meetings

SA3#124	13 – 17 October 2025		Wuhan, CN

SA3#125	17 – 21 November 2025		Dallas, US