3GPP TSG-SA3 Meeting #123	S3-252620

# Goteborg, Sweden, 25 – 28 August 2025

Source:	ZTE, China Telecommunications

Title:	SBA interface protection for CP functions deployed in the PNI-NPN operational domain

Document for:	Approval

Agenda item:	6.1.5

Spec:	3GPP <TS/TR><TS/TR number>

Version:	<TS/TR version>

Work Item:	<Work Item>

Comments

This contribution proposes to consider the security of additional CP functions are deployed in the PNI-NPN operational domain. It is nearly the same with the TR 33.757 key issue 2 with some additional statements:

1 If a NF (except AMF or SMF) is compromised by an attacker

2 The mechanism in TS 33.501 Annex AB shall be reused

Proposed Changes

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]				3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]				3GPP TS 33.501: "Security architecture and procedures for 5G system".

[y]				3GPP TR 33.757: "Study on security for a PLMN hosting a Non-Public Network".

* * * Next Change * * * *

6.X	Key Issue #X: SBA interface protection for CP functions deployed in the PNI-NPN operational domain

6.X.1	Key issue details

As in the TR 33.757[y], if NFs are compromised, attackers may utilize compromised NFs to collect topology, send malformed messages or launch DoS attacks.

And in  TS 33.501[x] AB.3, it is recommended to deploy one or more proxy entities at the border between PLMN and PNI-NPN operational domains.

In TS 33.501[x] Annex AB.2, only AMF and SMF are considered,  the Note states:

“Whether additional CP functions are deployed in the PNI-NPN operational domain and its associated additional security controls are  not considered in the present document.”

While additional CP functions may be deployed in the PNI-NPN operational domain, it is necessary to consider whether the proxy entities is suitable for SBA interface protection for additional CP functions.

6.X.2	Security threats

If a NF (except AMF or SMF) is compromised by an attacker, the following problems may occur:

-	The attacker may collect topology information of the PLMN or PNI-NPN operational domains and use the information to direct further attacks at the PLMN or PNI-NPN operational domains.

-	The attacker may send malformed signalling messages to NFs to degrade NFs' ability to process normal signalling messages.

-	The attacker may send messages to the NFs in the opposite domain with wrong NF types according to 3GPP specifications.

-	The attacker may launch DoS attacks to flood and disrupt the availability of NFs in the PLMN operational domains and vice versa.

-	The attacker may initiate unauthorized service operations. Safeguarding access tokens from an attacker is challenging when it crosses the security/trust boundary between the PLMN and PNI-NPN operational domains.

-	A compromised NF in the customer premises may request the NF(s) in a PLMN to consume a service that are not allowed in the PNI-NPN operational domains, and vice versa.

6.X.3	Potential security requirements

5GS should support mutual topology information hiding of the PLMN and PNI-NPN operational domains.

5GS should support the means to block malformed signalling messages sent from NFs (except AMF/SMF/UDM) in the PLMN or PNI-NPN operational domains over trust boundary.

5GS should support the means to block messages with wrong NF types sent from NFs (except AMF/SMF/UDM) in the  PNI-NPN operational domains over the trust boundary according to 3GPP specifications.

5GS should support the means to authenticate and authorize the NFs (except AMF/SMF/UDM) in the  PLMN and PNI-NPN operational domains over the trust boundary.

The 5G system shall support a mechanism for secure exchange of DNS queries/answers, when the dedicated NFs (except AMF/SMF/UDM) are in PNI-NPN operational domains.

5GS should support the means to restrict access to services and information exchanged between  PLMN and PNI-NPN operational domains.

NOTE1:  The mitigation of DoS by compromised NF in 5GS is left for implementation.

NOTE2:  The mechanism in TS 33.501[x] Annex AB shall be reused.

* * * End of Changes * * * *