3GPP TSG-SA3 Meeting #123		S3-252643

Goteborg, Sweden, 25 – 29 Aug. 2025

# Source:	SK Telecom

# Title:	Discussion on update of 6G SID to avoid false base station attack

# Document for:	Discussion

Agenda Item:	6.3

- Decision/action requested

This contribution presents the discussion on the need of update of 6G SID to avoid false base station attack in 6G.

2	References

[1]	TR 33.846 V17.0.0, ‘Study on authentication enhancements in the 5G System’, Release 17.

[2]	TR 33.809 V18.1.0, ‘Study on 5G security enhancements against False Base Stations (FBS)’, Release 18.

[3]	TR 22.870 V0.3.1, ‘Study on 6G Use Cases and Service Requirements’, Release 20.

[4]	TS 24.501 V19.3.0, ‘Non-Access-Stratum (NAS) protocol for 5G System (5GS)’, Release 19.

3	Discussion

In Rel-17, SA3 has carried out study on authentication enhancements in the 5G System [1]. Also, in Rel-18, SA3 has carried out study on 5G security enhancements against False Base Stations [2]. However, these potential attack points from FBS still remains in current 5G network, since there is no follow-up normative work of these topics until now. The biggest reason why normative work of security enhancement against FBS has not been done in 5G is most of candidate solutions require fundamental security protocol changes (e.g., public key infrastructure based base station certification) which cannot be easily applied to legacy 5G infrastructure, UE and USIM.

At the same time, there is also an increasing trend with cyber security threats and attacks over telecom systems, e.g., a recent malware attack over the subscription data management system led to massive subscriber data breach. If this leaked data is used by the potential attacker in combination with the existing attack points of FBS in current 5G specification, it makes more difficult to protect our customer privacy information without any exception.

In contrast to 5G, 6G study in SA3 represents an opportunity to revisit the foundations of security protocol design, free from the constraints of backward compatibility. Also, SA1 already discussed on the potential new requirement for seurity enhancement against FBS for 6G and it is captured in [3].

5.5.3.2 Potential New Requirements needed to support the use case [PR 5.5.3.2-1] Subject to operator policy and regulatory requirements, the 6G system shall support a means for a UE to be able to distinguish a False Base Station from an authentic Base Station.

There are several security enhancements in 5G compared to LTE such as enhanced privacy including permanent identifier encryption (e.g., SUCI: Subscription Concealed Identifier), and avoidance of SUPI paging. However, for enhancing the 6G security from potential FBS attack, we think that at least two security weak points are required to be studied and enhanced as listed below:

1) AS security: radio lower layers protection on target cell selection in both idle and connected mode.

A FBS can induce the UE in either Idle or Connected mode to perform the following procedures, thereby obtaining sensitive information (e.g., SUCI) from the UE:

- Idle Mode

  - Cell Selection

  - Cell Re-selection

- Connected Mode

  - Handover

  - RRC Connection Re-establishment

As long as UE determines its serving cell based solely on signal strength as in now, an attacker can repeatedly attempt the SUCI catching attack since there is no security protection on target cell selection both in idle (e.g., MIB/SIBs) and connected mode (e.g., MAC/MAC-CEs in LTM). Therefore, if there are methods to identify whether a target cell is a FBS during these network procedures, it would be possible to prevent potential information leakage. Several candidate solutions are proposed in [2].

2) NAS security: authentication enhancements

As the UE uses a 5G-GUTI rather than a SUCI in most cases, attacker who want to get a SUCI of victim subscriber needs to force the victim UE to perform the initial connection using SUCI. However, in current 5G NAS specification, the network can send SERVICE REJECT message to UEs without security protection and UE shall process SERVICE REJECT messages without integrity checking (if the 5GMM cause is not #76, #78) [4]. In other words, from this security vulnerability, unauthorized entries (e.g., FBS) can easily force the UE to delete its existing security context and this is used as a starting point for follow-up attacks such as SUCI catching. So, it makes more reasonable to assume that the attacker knows one legitimate SUCI of the victim subscriber.

If we assume that the attacker has pre-knowledge of old legitimate SUCI of the victim subscriber, there are two ways for FBS to detect whether victim subscriber’s presence in a specific area or not. In both cases, the attacker set up a FBS at a coverage with the victim subscriber and actively changes the registration request to this base station using old legitimated SUCI information of victim subscriber as shown in below figure 1.

[Figure 1. Remaining attack points from FBS (False Base Station)]

The first one is SUCI linkablity based on whether the AKA procedure is success or not with a specific SUCI. If some UE makes a registration request to this FBS operated as a relay, the attacker can modify this Registration Request message by replacing the old legitimated SUCI of victim subscriber into the identity field. If the following AKA procedure is success with this old legitimate SUCI of victim subscriber, attacker can know that this is the target subscriber.

As pointed out in [1], this attack cannot by mitigated by hiding only the content of the AKA response, because the attacker can detect from the subsequent messages (e.g. RRC SMC procedure, repetition of the AKA, or RRC release) whether the AKA run was successful or not. Regarding this issue (i.e., Key Issue #2.2: SUCI based attacks), several candidate solutions are proposed in [1].

The second one is SUCI linkablity based on error type (MAC/Synch fail) with a specific SUCI. Same as previous case, the attacker can modify the Registration Request message from UEs by replacing the old legitimated SUCI of target subscriber into the identity field. The network looks up this old legitimated SUCI and responds with an Authentication Request with authentication vectors (RAND, AUTN) that only the target subscriber can properly answer. When the UE receives this Authentication Request, one of two cases may occur:

- the victim UE is actually target subscriber: the victim UE successfully verifies the network and responds with Authentication Response, or Authentication Failure with cause Synch Failure (if SQN value from the network is smaller than SQN value stored in UE).

- the victim UE is not target subscriber: the MAC verification will be failed when executed by any other UE, and as a result an Authentication Failure with MAC failure message is sent.

This issue has been also studied in [1] as “Key Issue #2.1: Linkability by distinguishing MAC failure and synchronization failure” and multiple candidate solutions are proposed.

4	Conclusion

In summary, in 6G, it is necessary to introduce the authenticity verification mechanism such that the UE can verify the authenticity of the base station prior to starting the 6G network procedure both in idle and connected mode. If we miss this precious opportunity (i.e., 6G is commercilized without any security enhancements against FBS), we will have to wait another 10 years to fix it. Therefore, we propose to address potential threats of false base stations in 6G security main study item. For that purpose, we suggest to update the WT #1 of 6G SID to add ‘MIB/SIBs’ since it is a starting point of potential attacks that FBS uses to attract victim UE in idle mode. (red font).

RAN Architecture Security: WT#1: Study security and privacy aspects of 6G RAN architecture. Investigate on threats and security solutions in radio lower layers including protection of MIB/SIBs and MAC/MAC-CEs.

Also, the main security threat on the existing 5G-AKA is the ability to identify a subscriber using the old legitimated SUCI after an arbitrary amount of time has passed since it was captured. In other words, the 5G security scheme is vulnerable to SUCI replay attack since there is no mechanism in the network side to check whether the received SUCI was the previous one or not. Therefore, we propose to keep the WT #5 of 6G SID as it is now, while agreeing to cover false base station attacks relevant to primary authentication within it (i.e., WT#5)

UE to Network Security: WT#5: Study 6G primary authentication including known gaps in current authentication mechanisms and considering digital identities and authentication of both SIM based and SIM-less devices.