3GPP SA- WG3 Meeting # 125AdHoc-e	S3-260006-r3

# Electronic meeting, online, 19 – 23 January 2026

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.250 | CR | 0007 | rev | 01 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to PGW SCAS according to NESASG feedback |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Maint |  |  |  |  |  | Date: |  |  | 2026-01-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Change requests according to GSMA NESAS feedback, to ensure the SCAS is suitable for use in the NESAS scheme. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Editorial update. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.2, 4.2.2.3, 4.2.2.4, 4.2.2.5, 4.2.3.5.1, 4.2.6.3, 4.2.6.4, 4.3.5.1, 4.3.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

#### 4.2.2.2	Per-user based packet filtering

Requirement Name: Per-user based packet filtering

Requirement Reference: TS 23.401 [6], clause 4.4.3.3

Requirement Description: This requirement is identical to per-user based packet filtering (by e.g. deep packet inspection) as specified in TS 23.401, clause 4.4.3.3.

Threat References: TR 33.926 [10], clause B.2.3.1 Failure to assign unique TEID or Charging ID for a session.

Test Case:

Test Name: TC_PER-USER_BASED_PACKET_FILTERING_PGW

Purpose:

Verify that PGW supports a Per-user based packet filtering.

Pre-Conditions:

-	The tester has a privilege to configure the filtering policy on the PGW to make the PGW can filter the packets per-user

-	 Some UE (e.g. UE1 and UE2) are registered on the PGW.

-	The PGW can receive the packets from the UE1 and UE2.

-	A network traffic analyser on the PGW (e.g. tcpdump) is available.

Execution Steps:

1)	The tester configures the different filtering policy for the UE1and the UE2 on the PGW, e.g. the PGW forwards the packets from the UE1 to SGi and drops the packets from the UE2.

2)	The tester sends the packets from the UE1 to the PGW.

3)	The tester sends the packets from the UE2 to the PGW.

4)	The tester checks the filtered packets using the network traffic analyser.

Expected Results:

The PGW can filter the packets per- user according the configured filtering policy, e.g. the PGW forwards the packets from the UE1 to SGi in the step 2 and drops the packets from the UE2 in the step 3.

Expected format of evidence:

Evidence suitable for the interface, e.g. screenshot contains the operation results, pcap file demonstrating that the UE2’s packets are correctly received but unavailable on the SGi interface while the UE1’s packets are correctly sent to SGi.

==============Next change==============

#### 4.2.2.3	Charging ID Uniqueness

Requirement Name: Charging ID Uniqueness

Requirement Reference: TS 32.251 [8], clause 5.1.1

Requirement Description: "Every IP-CAN bearer shall be assigned a unique identity number for billing purposes. (i.e. the Charging Id)" as specified in 3GPP TS 32.251 [8], clause 5.1.1.

Note: A charging ID is not assigned to more than one active IP-CAN bearers at the same time. The reuse of Charging ID is possible after an IP-CAN session has been terminated and the Charging ID related to this IP-CAN session has been released.

Threat References: TR 33.926 [10], clause B.2.5.1 Failure to assign unique TEID or Charging ID for a session

Test Case:

Test Name:  TC_CHARGING_ID_UNIQUENESS_PGW

Purpose:

Verify that the Charging ID value set in the Information Element Bearer Context within a CreateSessionResponse is unique.

Pre-Conditions:

Test environment with P-GW and S-GW, PCRF. PCRF and S-GW may be real nodes or simulated.

The tester is able to trace traffic between the P-GW and the S-GW (real or simulated)

Execution Steps:

1)	The tester intercepts the traffic between the P-GW and the S-GW.

2)	The tester trigger more than one (e.g. at least 10000) consecutive CreateSessionRequest for an Initial UE Attach towards the P-GW (using a real or a simulated S-GW) in order to setup a new IP-CAN bearer.

3)	The P-GW creates a UE/S-GW context and communicates with the PCRF (real or simulated) for QOS and APN resolve. That procedures shall be successfully in order to permit to the P-GW to send back to the S-GW a CreateSessionResponse containing at least :

a)	A Success cause.

b)	The P-GW’s F-TEID for control plane

c)	The PDN Address Allocation (PAA)

d)	A Bearer Contexts Created.

4)	The tester verifies that the Charging ID within Bearer Contexts Created in each generated CreateSessionResponse are different.

Expected Results:

The Charging ID assigned to every IP-CAN bearer requested by different CreateSessionRequest is unique.

Expected format of evidence:

Files containing the triggered GTP messages (e.g. pcap trace).

==============Next change==============

#### 4.2.2.4	TEID UNIQUENESS

Requirement Name: TEID Uniqueness

Requirement Reference: TS 23.060 [9], clause 14.6

Requirement Description: "The TEID is a unique identifier within one IP address of a logical node." as specified in TS 23.060 [9], clause 14.6.

Note: A TEID is not assigned to more than one active GTP tunnel at the same time.The reuse of TEID is possible after a GTP tunnel has been terminated and the TEID related to this GTP tunnel has been released.

Threat References: TR 33.926 [10], clause B.2.5.1 Failure to assign unique TEID or Charging ID for a session

Test Case:

Test Name:  TC_TEID UNIQUENESS_PGW

Purpose:

Verify that the TEID generated for each new GTP tunnel is unique for both control and user plane.

Pre-Conditions:

Test environment with P-GW and S-GW, PCRF. PCRF and S-GW may be real nodes or simulated.

The tester is able to trace traffic between the P-GW and the S-GW (real or simulated)

Execution Steps:

1)	The tester intercepts the traffic between the P-GW and the S-GW.

2)	The tester triggers more than one (e.g. at least 10000) consecutives CreateSessionRequest e.g. for an Initial UE Attach towards the P-GW (using a real or a simulated S-GW) with GTP header TEID set to 0 and F-TEID set to different values.

3)	The P-GW creates a UE/S-GW context and communicates with the PCRF (real or simulated) for QOS and APN resolve. That procedures shall be successfully in order to permit to the P-GW to send back to the S-GW a CreateSessionResponse containing at least :

a)	A Success cause.

b)	The P-GW’s F-TEID for control plane

c)	The PDN Address Allocation (PAA).

d)	A Bearer Contexts Created.

4)	The tester verifies that the F-TEID created for each generated CreateSessionResponse is unique.

Expected Results:

The F-TEID  set into each different CreateSessionResponse  is unique.

Expected format of evidence:

Files containing the triggered GTP messages (e.g. pcap trace).

==============Next change==============

#### 4.2.2.5	Mobility binding

Requirement Name: MN-HA authentication extension validation for mobility binding during trusted non-3GPP access

Requirement Reference: TS 33.402 [11], clause 9.2.1.1

Requirement Description: "The PDN-GW shall validate the MN-HA authentication extension" as specified in               TS 33.402 [11], clause 9.2.1.1.

Threat References:  TR 33.926 [10], 5.3 Generic threats

Test Case:

Test Name: TC_PGW_MIP-AUTH_Non-3GPP

Purpose: To test whether the PGW validates the MN-HA authentication extension correctly.

Pre-Condition:

The UE and PGW are connected in the test environment. UE is simulated.

The tester has access to the S6b interface between the 3GPP AAA Server and the PDN GW.

The tester has access to the MIPv4 FACoA based S2a interface between the PGW and UE.

Execution Steps:

The PGW (home agent for the UE) is active in a 3GPP access network.

The tester captures packets over S6b and S2a interfaces using any packet analyser.

The tester filters the MN-HA Key transported in the authentication and authorization information from the 3GPP AAA server to PGW over S6b interface.

The UE sends a Registration Request message to PGW via the trusted non-3GPP IP access network.

The tester filters the Registration Request sent by UE to PGW and Registration Reply sent by PGW to UE over the S2a interface.

The tester uses the SPI value in Registration Request to identify the MN-HA key to compute the Authenticator value of the authentication extension.

The tester verifies that the computed Authenticator value is same as the Authenticator value in the Registration Request message.

The tester also checks the Reply Code in the Registration Reply message to verify the correctness of the validation at PGW.

Expected Results:

The Reply code is ‘0’ in the Registration Reply message.

Expected format of evidence:

Evidence suitable for the interface, e.g. Screenshot contains the operation results.

==============Next change==============

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 41.001: "GSM Release specifications".

[3]	3GPP TS 33.117: "Catalogue of General Security Assurance Requirements".

[4]	3GPP TR 33.916: "Security assurance scheme for 3GPP network products for 3GPP network product classes".

[5]	3GPP TS 33.401: "3GPP System Architecture Evolution (SAE); Security architecture".

[6]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[7]	3GPP TS 33.102: "3G security; Security architecture".

[8]	3GPP TS 32.251: "Telecommunication management; Charging management; Packet Switched (PS) domain charging".

[9]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[10]	3GPP TR 33.926: "Security Assurance Specification (SCAS) threats and critical assets in 3GPP network product classes".

[11]	3GPP TS 33.402: "3GPP System Architecture Evolution (SAE); Security aspects of non-3GPP accesses".

[x1]	3GPP TS 29.281: “General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U)”.

[x2]	3GPP TS 23.501: “System architecture for the 5G System (5GS)”

[x3]	3GPP TS 22.261: “Service Requirements for the 5G System (5GS)”

[x4]	…

==============Next change==============

##### 4.2.3.5.1 Unpredictable GTP TEID

Requirement Name: Unpredictable GTP TEID

Requirement Reference: TS 29.281 [x1], clause 5.3

Requirement Description:

The TEID created for usage in the GTP-C messages as well as in the GTP-U messages shall be unpredictable  in order to prevent a hacker to inject GTP-U packets with a spoofed TEID into a user’s session (causing e.g. overbilling problems) or to send malicious GTP-C messages to delete an established session (and causing a DoS).

Threat References: TR 33.926 clause 5.3.7.2  Denial of service: Implementation Flaw and clause 5.3.4.x	Tampering: User Traffic Tampering.

Test case:

Test Name: UNPRED_GTP_TEID

Purpose:

To verify that the GTP TEID is unpredictable

Pre-Conditions:

Test environment with P-GW and S-GW, PCRF. PCRF and S-GW may be real nodes or simulated.

The tester is able to trace traffic between the P-GW and the S-GW (real or simulated).

Execution Steps:

- The tester intercepts the traffic between the P-GW and the S-GW.

- The tester triggers 10 consecutives CreateSessionRequest e.g. for an Initial UE Attach towards the P-GW (using a real or a simulated S-GW) with GTP header TEID set to 0 and F-TEID set to different values.

- The tester triggers one CreateSessionRequest, this request shall be for another UE and from another S-GW

- The P-GW creates a UE/S-GW context and communicates with the PCRF (real or simulated) for QOS and APN resolve. That procedures shall be successful in order to permit to the P-GW to send back to the S-GW a CreateSessionResponse containing at least :

  - A Success cause.

  - The P-GW’s F-TEID for control plane

  - The PDN Address Allocation (PAA).

  - A Bearer Contexts Created.

- The tester tries to predict the F-TEID created for the final CreateSessionResponse from the initial 10 F-TEIDs.

Expected Results:

The tester cannot predict the F-TEID in the finalCreateSessionResponse.

Expected format of evidence:

Files containing the triggered GTP messages (e.g. pcap trace) and, if the F-TEID is predictable, a detailed description of how the F-TEID can be predicted.

==============Next change==============

#### 4.2.6.3	IP Address reallocation interval

Requirement Name: IP Address Reallocation Interval

Requirement Reference: TS 23.401 [6], clause 5.3.1, TS 23.501 [x3], clause 5.8.2.2

Requirement Description:

The PGW shall support a mechanism to set an interval between an IP address reallocation.

Test case:

Test Name: TC_ IP-ADDRESS_REALLOCATION_INTERVAL

Purpose:

Verify that the PGW supports an IP address reallocation interval technique.

Pre-Condition:

-	Documentation describing how to configure an IP address reallocation interval.

Execution Steps:

- Configure the IP address reallocation interval to T according to the product documentation.

- Allocate an IP address IP1 to UE1.

- Make UE1 release the IP address IP1.

- Within an interval of T after the release of IP1, make the PGW allocate the IP address IP1 to UE2.

- Attempt the step 4 in more time than T after the release of IP1.

Expected Results:

1)	In execution step 4, the reallocation attempt is rejected.

2)	In execution step 5, the reallocation attempt is accepted.

Expected format of evidence:

A PASS or FAIL.

#### 4.2.6.4	MS/UE-Mutual Access Prevention

Requirement Name: MS/UE-Mutual Access Prevention

Requirement Reference: In accordance with industry best practice

Requirement Description:

The PGW shall support a mechanism to prevent MS/UE-mutual access attacks (e.g. configure a filtering rule to drop all mutual access packets).

Threat Reference: TR 33.926 [10], Annex B

Test case:

Test Name: TC_ MS/UE-MUTUAL_ACCESS_PREVENTION

Purpose:

Verify that the Network Product supports a MS/UE-Mutual Access Prevention technique.

Procedure and execution steps:

Pre-Condition:

-	The PGW has configured two (or more) IP address segments for UEs named IPSeg 1 and IPSeg 2 (e.g. 10.40.0.0/16、10.42.0.0/16).

-	The PGW has 2 different logical or physical Ethernet ports and each port is connected to a host.

-	A PGW analyser on the network product (e.g. tcpdump) is available.

-	A packet analyzer on the UEs is available.

Execution Steps:

1)	The tester configures the PGW to block direct UE to UE traffic according to product documentation.

2)	The tester configures a filtering rule that UEs with IP address in IPSeg 1cannot access to servers with IP address in IPSeg 2 and vice versa.

3)	The PGW allocate the IP1 within the IPSeg 1 to UE 1.

4)	The PGW allocate the IP2 within the IPSeg 2 to UE 2.

5)	The UE1 sends a packet with destination IP Address set to IP3 different from IP1 within the IPSeg 1.

6)	The UE1 sends a packet with destination IP Address set to IP2.

7)	The UE2 sends a packet with destination IP Address set to IP4 different from IP2 within the IPSeg 2.

8)	The UE2 sends a packet with destination IP Address set to IP1.

Expected Results:

Using the network analyser the tester verifies that the packets are correctly received and discarded by the PGW. The tester verifies that the packets are correctly sent by the UE through the packet analyzer on the UEs.

NOTE: 	The IP address segments allocated to UEs are separate from the IP address segments of PDN servers.

Expected format of evidence:

A log from analyser to show the process.

==============Next change==============

#### 4.3.5.1	Traffic separation

Requirement Name: Traffic Separation

Requirement Reference:  TS 22.261 [x3], clause 5.2.12.1

Requirement Description:

The PGW shall support physical or logical separation of O&M and control plane traffic, O&M and user plane traffic, control plane and user plane traffic respectively.

Note1: The security requirement in clause 4.3.5.1 of TS 33.117 (i.e. the physical or logical separation of O&M and control plane traffic) and related test case also applies to the PGW.

Note2: The requirement that is different from TS 33.117[3], clause 4.3.5.1 is that the traffic separation of user plane from O&M and control plane which is considered to be PGW-specific has been take into account in present document.

Test case:

Test Name: TC_TRAFFIC_SEPARATION

Purpose:

To test whether O&M traffic is separated from user plane traffic, control plane traffic is separated from user plane traffic.

Pre-Condition:

The PGW has at least one separate (logical) interface dedicated to O&M traffic and at least two (logical) interfaces for control plane traffic and user plane traffic respectively. The PGW for which the test applies and that fail to meet this precondition fail the test by definition.

Execution Steps:

Execute the following steps:

1.	The tester checks whether the PGW refuses O&M traffic on all interfaces meant for user plane traffic.

2.	The tester checks whether the PGW refuses user plane traffic on all O&M interfaces.

3.	The tester checks whether the PGW refuses control plane traffic on all interfaces meant for user plane traffic.

4.	The tester checks whether the PGW refuses user plane traffic on all control plane interfaces.

Expected Results:

The six tests should be successful, i.e. the PGW refuses traffic in all of the four steps.

Expected format of evidence:

Evidence suitable for the interface, e.g. screenshot contains the operation results.

==============Next change==============

#### 4.3.5.2	User Plane Traffic Differentiation

Requirement Name: User Plane Traffic Differentiation

Requirement Reference: 3GPP TS 23.401 [6], clause 5.10.1 and in accordance with industry best practice

Requirement Description:

“The EPS shall support simultaneous exchange of IP traffic to multiple PDNs through the use of separate PDN GWs or single PDN GW” as specified in 3GPP TS 23.401 [6], clause 5.10.1. According to this, the PGW shall support the user plane traffic differentiation (e.g. enterprise, internet, etc) by setting the specific APNs, and shall support the traffic isolation based on the APNs (e.g. using VPN).

Test case:

Test Name: TC_USER PLANE TRAFFIC_DIFFERENTIATION

Purpose:

1. To test whether the user plane traffics is differentiated by setting the specific APNs.

2. To test whether the traffic is isolated based on the APNs.

Pre-Condition:

The PGW has configured several APNs for the testing, and every APN is configured to associate with specific VPN (e.g. the VPN can be GRE) for user plane traffic. For example, APN1’s traffic is sent and received by VPN1, and APN2’s traffic is sent and received by VPN2.

The PGW for which the test applies and that fail to meet this precondition fail the test by definition.

Execution Steps:

Execute the following steps:

1. The tester intercepts the VPN packets between the P-GW and PDN, as well as the GTP-U packets between P-GW and S-GW/UE;

2. The tester checks triggers APN1’s traffic with the P-GW, then the tester verifies that the tunnel id of the VPN packets sent by the P-GW indicates VPN1 as well as the TEID of the GTP-U packets sent by the P-GW indicates APN1;

3. The tester triggers APN2’s traffic with the P-GW, then the tester verifies that the tunnel id of the VPN packets sent by the P-GW indicates VPN2 as well as the TEID of the GTP-U packets sent by the P-GW indicates APN2;;

4. The tester checks triggers APN1’s traffic with the P-GW, then the tester verifies that the tunnel id of the VPN packets sent by the P-GW does not indicate VPN2 as well as the TEID of the GTP-U packets sent by the P-GW does not indicate APN2;

5. The tester triggers APN2’s traffic with the P-GW, then the tester verifies that the tunnel id of the VPN packets sent by the P-GW does not indicate VPN1 as well as the TEID of the GTP-U packets sent by the P-GW does not indicate APN1.

Expected Results:

The four verification should be successful.

Expected format of evidence:

The APN traffic is sent and received by the right VPN.

==============End of change==============