3GPP TSG-SA3 Meeting #125-e	S3-260001-r3

# Online, , 19th Jan 2026 - 23rd Jan 2026

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.515 | CR | 0013 | rev | 01 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial corrections to 33.515 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | T-Mobile USA |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Maint |  |  |  |  |  | Date: |  |  | 2026-01-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | GSMA NESAS idenified editorial changes to the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update abbreviation list by adding PFCP abbreviation and removing the duplicate TEID abbreviation contained in 21.905. Fixed test headers to align with the template. Corrected the fact that JSON is not a protocol. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Editorial errors would remain in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 4.2.2.1.1, 4.2.2.1.3, 4.2.2.1.4, and 4.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Corrected abbreviations, test headers and format of JSON. |  |  |  |  |  |  |  |  |

==============First change==============

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

CHF	Charging Function

PFCP	Packet Forwarding Control Protocol

SCAS	Security Assurance Specification

SMF	Session Management Function

UDM	Unified Data Management Function

UPF	User Plane Function

==============Next change==============

##### 4.2.2.1.1	Priority of UP security policy

Requirement Name: Priority of UP security policy

Requirement Reference: TS 23.501 [1], clause 5.10.3

Requirement Description: User Plane Security Policy from UDM takes precedence over locally configured User Plane Security Policy as specified in TS 23.501 [1], clause 5.10.3

Threat References: TR 33.926 [4], clause J.2.2.1 Non-compliant UP security policy handling

Test Case:

Test Name: TC_UP_POLICY_PRECEDENCE_SMF

Purpose:

Verify that the user plane security policy from the UDM takes precedence at the SMF under test over locally configured user plane security policy.

Pre-Conditions:

Test environment with AMF and UDM may be simulated.

Both UDM and SMF under test are configured with UP security policy, and the UP security policies are different.

There is no Session Management Subscription data in SMF.

Execution Steps:

1)	The tester triggers PDU session establishment procedure by sending Nsmf_PDUSession_CreateSMContext Request message to the SMF.

2)	The SMF under test retrieves the Session Management Subscription data using Nudm_SDM_Get service from UDM, where the Session Management Subscription data includes the user plane security policy stored in UDM.

3)	The tester captures the Namf_Communication_N1N2MessageTransfer message sent from the SMF under test to the AMF.

Expected Results:

There is a Security Indication IE in the N2 SM information contained in the Namf_Communication_N1N2MessageTransfer message, which is the same with the UP security policy configured in the UDM.

Expected format of evidence:

Evidence suitable for the interface, e.g., Screenshot containing the operational results.

==============Next change==============

##### 4.2.2.1.3	Security functional requirements on the SMF checking UP security policy

Requirement Name: UP security policy check.

Requirement Reference: TS 33.501 [8], clause 6.6.1

Requirement Description: According to TS 33.501 [8], clause 6.6.1, the SMF verifies that the UE's UP security policy received from the target ng-eNB/gNB is the same as the UE's UP security policy that the SMF has locally stored. If there is a mismatch, the SMF sends its locally stored UE's UP security policy of the corresponding PDU sessions to the target gNB. This UP security policy information, if included by the SMF, is delivered to the target ng-eNB/gNB in the Path-Switch Acknowledge message. The SMF logs capabilities for this event and may take additional measures, such as raising an alarm.

Threat References: TR 33.926 [4], clause J.2.2.4, Unchecked UP security policy.

TEST CASE:

Test Name: TC_UP_SECURITY_POLICY_SMF

Purpose:

Verify that the SMF checks the UP security policy that is sent by the ng-eNB/gNB during handover.

Pre-Conditions:

The SMF under test is preconfigured with a UE UP security policy.

Execution Steps:

1.	The tester sends the Nsmf_PDUSession_UpdateSMContext Request message to the SMF under test. A UE UP security policy different than the one preconfigured at the SMF under test is included in the Request message.

2.	The tester captures the Nsmf_PDUSession_UpdateSMContext Response message sent from the SMF under test.

Expected Results:

The preconfigured UE security policy is contained in the ‘n2SmInfo’ IE in the captured Response message.

Expected format of evidence:

Files containing the triggered HTTP messages (e.g. pcap trace).

==============Next change==============

##### 4.2.2.1.4	Charging ID Uniqueness

Requirement Name: Charging ID uniqueness.

Requirement Reference: TS 32.255 [6], clause 5.1.2

Requirement Description: According to TS 32.255 [6], clause 5.1.2:

-	The SMF supports PDU session charging using service based interface.

-	The SMF collects charging information per PDU session for UEs served under 3GPP access and non-3GPP access.

-	Every PDU session is assigned a unique identity number for billing purposes per PLMN. (i.e. the Charging Id).

Threat Reference: TR 33.926 [4], clause J.2.2.3, "Failure to assign unique Charging ID for a session"

TEST CASE:

Test Name: TC_CHARGING_ID_UNIQUENESS_SMF

Purpose:

Verify that the charging ID generated by the SMF for each PDU session is unique.

Pre-Conditions:

Test environment is set up with a Charging Function (CHF), which may be real or simulated, and the SMF under test. The tester is able to capture the traffic between the SMF under test and the CHF.

Execution Steps:

1)	The tester intercepts the traffic between the SMF under test and the CHF.

2)	The tester triggers the establishment of the maximum number of concurrent PDU sessions that the SMF under test can handle.

3)	The tester captures each Charging Data Request [initial] sent from the SMF under test to the CHF, and verifies the charging ID contained in the ‘PDU Session Charging Information’ IE in each Charging Data Request [initial] is unique.

Expected Results:

The charging ID in each Charging Data Request [initial] is unique.

Expected format of evidence:

Files containing the Charging Data Request [initial] messages (e.g. pcap trace).

==============Next change==============

### 4.4.4	Robustness and fuzz testing

The test cases under clause 4.4.4 of TS 33.117 [2] are applicable to SMF.

The interfaces defined for the SMF are defined in clause 4.2.3 of TS 23.501 [1].

According to clause 4.4.4 of TS 33.117 [2], the transport protocols available on the interfaces providing IP-based protocols need to be robustness tested. Following TCP/IP layer model and considering all the protocols over transport layer, for SMF, the following interfaces and protocols are in the scope of the testing:

-	For Nsmf: The TCP, HTTP2 protocols and the format of JSON.

-	For N4: The UDP and PFCP protocols.

NOTE:	There could be other interfaces and/or protocols requiring testing under clause 4.4.4 of TS 33.117 [2]

==============End of change==============