3GPP TSG-SA3 Meeting #125AdHoc-e	draft_S3-260012-r2

Electronic meeting, online, 19 – 23 January 2026

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.117 | CR | 0228 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for 33.117 - Update the execution steps in 4.3.3.1.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Maint |  |  |  |  |  | Date: |  |  | 2026-01-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Non-automatic mounting imposes excessively high requirements, for example the USB is usually automatically mounted on devices, as industry best practice. Propose to change the execution steps and expected results to align with the requirement description. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change “the media device is not automatically mounted and there is no automatic application launch triggered by its insertion” to “the media device is not automatically mounted or there is no automatic application launch triggered by its insertion” in execution steps. Update the expected results. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | - 4.3.3.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

4.3.3.1.3	No automatic launch from removable media

Requirement Name: No automatic launch from removable media

Requirement Reference: In accordance with industry best practice

Requirement Description:

The network product shall not automatically launch any application when removable media device (physical or emulated) such as CD-, DVD-, USB-Sticks or USB-Storage drive is connected. If the operating system supports an automatic launch, it shall be deactivated unless it is required to support availability requirements.

Threat References: TR 33.926 [4], clause 5.3.4.3, External Device Boot

Test Case:

Test Name: TC_NO_AUTO_LAUNCH_FROM_REMOVABLE_MEDIA

Purpose:

To verify that the network product does not launch any applications automatically when a removable media device (physical or emulated) is connected. Any such feature should be deactivated.

Procedure and execution steps:

Pre-Condition

If the network product is provisioned with the necessary physical or emulated ports/drives (CD/DVD drive, USB port, etc.) then the test case applies.

Execution Steps

1.	The tester log in the network product.

2.	For all available physical ports or their emulated substitutes which are externally accessible:

a.	The tester prepares a removable media device (e.g. CD, DVD, USB-Sticks, CD-Image file and/or USB-Storage drives) that contain any kind of autostart file suitable for this port type.

b.	The tester inserts the prepared media device into the network product under test.

3.	The tester verifies that  there is no automatic application launch triggered by media device insertion.

Expected Results:

The network product does not launch any applications to open the contents in the removable media device.

Expected format of evidence:

Evidence can be presented in the form of logs/screenshot/screen-capture on how the network product responds when any removable media device is attached to it (e.g. the output log of the UNIX® mount command before and after insertion of the removable media device).

* * * End of Changes * * * *