3GPP TSG-SA3 Meeting #50 	draft modification of S3-080089

SanYa, China, 25 – 29 February, 2008

| CR-Form-v9.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.203 | CR | 127 |  rev | 1 |  Current version: | 8.1.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Co-existence of authentication schemes: how can IMS network entities enforce that Digest is not used over 3GPP access? |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | BT, Ericsson, Nokia, Nokia Siemens Networks, Orange, TeliaSonera |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code:  | IMS-Sec |  |  |  |  |  |  | Date:  |  | 27/02/2008 |
| Category:  | B |  |  |  |  |  |  | Release:  |  | Rel-8 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) |  |
| Reason for change:  |  | Annex N.1 specifies that “SIP Digest authentication and the requirements in this Annex shall not apply to access networks defined in 3GPP specifications.”. There is an editor’s note which reads: “It is ffs how IMS network entities can enforce this condition. It is envisioned that network-provided P-Access-Network-Info headers, cf. TR 33.803 and TS 24.229, can be used to solve this issue.” |  |  |  |  |  |  |  |  |
| Summary of change:  |  | SA3#49bis approved S3-080007. This CR resolves the above-mentioned editor’s note by pointing to Annex P.3 of S3-080007, where it is stated how the P-CSCF can distinguish REGISTER request relating to SIP Digest from other REGISTER request. The CR does not make use of the P-Access-Network-Info headers. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | SIP Digest authentication may be used over 3GPP-defined access networks. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | Annex N.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

*********************************FIRST CHANGE************************

Annex N (normative):
Enhancements to the access security to enable SIP Digest

## N.1 SIP Digest

SIP Digest authentication and the requirements in this Annex shall not apply to access networks defined in 3GPP specifications. The P-CSCF can enforce this condition by identifying REGISTER requests relating to SIP Digest according to the rules in Annex P.3 of this specification and discarding them when received over an access network defined in 3GPP specifications.

The provisions in Annex N are optional for implementation. The provisions in Annex N are optional for use. However, the use of one of the authentication mechanisms in this specification is mandated.

SIP Digest shall not be used in conjunction with IPsec.

NOTE 1: The use of SIP Digest in conjunction with IPsec, as specified in the main body and in Annex N of this specification, is technically impossible because SIP Digest does not generate session keys for use with IPsec security associations.

An additional scheme for authentication is SIP Digest as specified in RFC 3261 [6]. SIP Digest achieves mutual authentication between the UE and the HN, and is based on HTTP Digest as specified in RFC 2617 [12]. The identity used for authenticating a subscriber is the private identity, IMPI, which has the form of a NAI. The HSS and the UE share a preset secret (e.g., a password) associated with the IMPI. The generation of the authentication challenge shall be done in the same way as specified in RFC 2617 [12] and this document.

It is the policy of the HN that decides if an authentication shall take place for the registration of an additional IMPU that is not part of the already registered set of IMPUs associated with the same IMPI.

If a UE supports SIP Digest as well as further authentication methods, the UE shall proceed as follows:

-	If the access network is of a type defined in 3GPP specifications then the UE shall not select SIP Digest, in accordance with the requirement at the start of this clause.

NOTE 2: The rules listed in TR 33.978 [25] say how a UE can select between IMS AKA and Early IMS.

-	If the access network is of a type not defined  in 3GPP specifications then

-	if both the UE and network support IMS AKA according to the main body or Annex M of this specification, as determined by the use of sip-sec-agree [21], the authentication method shall be IMS AKA;

-	otherwise the authentication method shall be SIP Digest as specified in Annex N of this specification.

******************************END FIRST CHANGE***********************