3GPP TSG-SA WG3 Meeting #54 	S3-090106

- Florence, Italy, 19-23 January 2009

| CR-Form-v9.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.401 | CR |  |  rev | - |  Current version: | 8.2.1 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME | X | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Correction of definition of direction bit DIR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | TeliaSonera |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code:  | SAES |  |  |  |  |  |  | Date:  |  | 19/01/2009 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-8 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) |  |
| Reason for change:  |  | The definition of the direction bit DIR used in ciphering and integrity protection algorithms is different from what has been used for UMTS in TS33.102 |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Reversion of definition so that same definition applies for both UMTS and LTE use of algorithm inputs |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Inconsistent specs and risk for misimplementations. Direct reuse of UMTS implementation of SNOW 3G algorithms for EPS is not possible. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | B.1.1 and B.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | N | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | N | Test specifications |  |  |  |  |  |  |
|  |  |  | N | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

# B.1	128-bit ciphering algorithm

### B.1.1	Inputs and outputs

The input parameters to the ciphering algorithm are a 128-bit cipher key named KEY, a 32-bit COUNT, a 5-bit bearer identity BEARER, the 1-bit direction of the transmission i.e. DIRECTION, and the length of the keystream required i.e. LENGTH. The DIRECTION bit shall be 0 for uplink and1 for downlink.

Figure B.1-1 illustrates the use of the ciphering algorithm EEA to encrypt plaintext by applying a keystream using a bit per bit binary addition of the plaintext and the keystream. The plaintext may be recovered by generating the same keystream using the same input parameters and applying a bit per bit binary addition with the ciphertext.

Figure B.1-1: Ciphering of data

Based on the input parameters the algorithm generates the output keystream block KEYSTREAM which is used to encrypt the input plaintext block PLAINTEXT to produce the output ciphertext block CIPHERTEXT.

The input parameter LENGTH shall affect only the length of the KEYSTREAM BLOCK, not the actual bits in it.

### B.1.2	128-EEA1

128-EEA1 is based on SNOW 3G and is identical to UEA2 as specified in [14]. The used IV is constructed the same way as in subclause 3.4 of that TS.

### B.1.3	128-EEA2

128-EEA2 is based on 128-bit AES [15] in CTR mode [16]

The sequence of 128-bit counter blocks needed for CTR mode T1, T2, …, Ti, … shall be constructed as follows:

The most significant 64 bits of T1 consist of COUNT[0] .. COUNT[31] │ BEARER[0] .. BEARER[4] │ DIRECTION │ 026 (i.e. 26 zero bits).  These are written from most significant on the left to least significant on the right, so for example COUNT[0] is the most significant bit of T1.

The least significant 64 bits of T1 are all 0.

Subsequent counter blocks are then obtained by applying the standard integer incrementing function (according to Appendix B1 in [16]) mod 264 to the least significant 64 bits of the previous counter block.

## B.2	128-Bit integrity algorithm

### B.2.1	Inputs and outputs

The input parameters to the integrity algorithm are a 128-bit integrity key named KEY, a 32-bit COUNT, a 5-bit bearer identity called BEARER, the 1-bit direction of the transmission i.e. DIRECTION, and the message itself i.e MESSAGE. The DIRECTION bit shall be 0 for uplink and 1 for downlink. The bit length of the MESSAGE is LENGTH.

Figure B.2-1 illustrates the use of the integrity algorithm EIA to authenticate the integrity of messages.

Figure B.2-1: Derivation of MAC-I (or XMAC-I)

Based on these input parameters the sender computes a 32-bit message authentication code MAC-I using the integrity algorithm EIA. The MAC-I is then appended to the message when sent. The receiver computes XMAC-I on the message received in the same way as the sender computed MAC-I on the message sent and verifies the data integrity of the message by comparing it to the received MAC-I.

End of modifications