3GPP TSG SA WG3 (Security) Meeting #80                                                         S3-151708

24-28 August 2015, Tallinn (Estonia)

Source:	Telecom Italia

Title:	Providing test cases related to the requirement 5.2.3.6.1 “Security event logging”

Document for:	Discussion/approval

Agenda Item:

Work Item / Release:	SCAS / Rel-13

Abstract of the contribution: This contribution proposes to add test cases to the requirement 5.2.3.6.1 of TS 33.117.

# Introduction

Based on the discussion held during the call of 5th of August,  this contribution proposes:

- To mandate the support of four of the event logs currently listed in the requirement 5.2.3.6.1 of  TS 33.117 0.1.1;

- To let the support of  “Change of group membership for accounts” event optional;

- To add an Event Type that can be used to classify the events;

- To add a Timestamp to each logged event;

- To add a test case for each mandatory event;

- Changes for the “Requirement Description” which are not intended to be editorial and need to be better discussed.

- Some minor editorial changes.

Note that the Requirements Reference in the test case template used in TR 33.806 is no longer needed here since in TS 33.117 the test cases immediately follows the requirement.

# Proposed pCR

***	BEGIN OF FIRST CHANGE	***

#### 5.2.3.6	Logging

##### 5.2.3.6.1	Security event logging

Requirement Name: Security event logging

Requirement Reference: TBA

Requirement Description: Security events shall be logged together with a unique system reference (e.g., host name, IP or MAC address) and the exact time the incident occurred. For each security event, the log entry shall include user name and/or  timestamp and/or performed action and/or result and/or length of session and/or values exceeded and/or value reached.

RFC 3871 [4] section 2.11.10 specifies the minimum set of security events. Each vendor shall document what security events that the product logs so it can be verified by testing.

In particular, it shall be possible to log the following events (which are intended to be supported by the network product and which can be enabled by default at manufacturing time or at a later time by the operator):

| Event Types | Description | Event data to be logged |
| --- | --- | --- |
| Incorrect Login Attempts | An event that records incorrect login attempts. | • Username, • Source IP address of remote access • Timestamp. |
| Administrative Access | An event that records system access with accounts with administrator rights | • Username, • Access timestamp, • Length of session, • Source IP address of remote access |
| Account administration | An event that records account administration | • Administrator username, • Administered account, • Activity performed (configure, delete, enable and disable), • Timestamp |
| Resource Usage | An event that records any critical rise in system values such as disk space, CPU load over a longer period. | • Value exceeded, • Value reached, (Here suitable threshold values shall be defined depending on the individual system.) • Timestamp. |

In addition, optionally it shall be possible to log also the following event (if supported):

| Event Types | Description | Event data to be logged |
| --- | --- | --- |
| Change of group membership for accounts | An event that records any change of group membership for accounts | • Administrator account, • Administered account, • Activity performed (group added or removed) • Timestamp. |

Threat References: TBA

Security Objective references: tba.

Test case:

Test Name: TC_INCORRECT_LOGIN_ATTEMPTS_LOGGING

## Purpose:

To verify that the network product correctly logs the Incorrect Login Attempts event type.

## Procedure and execution steps:

## Pre-Conditions:

- The following information shall be provided by the documentation accompanying the network product:

- The log where the event is recorded (e.g. in the Security Log rather than, for instance, System log) and its location in the network product (i.e. the complete path).

- If the event type is enabled by default or how to enable it.

- If the SSH service is enabled  by default or how to enable it.

- The tester has administrative privileges

- A tester machine is available.

- The network product has at least one interface (named if1) enabled.

## Execution Steps

- The tester create an account A with username/password in the network product.

- The tester enables the SSH service on if1 interface if disabled

  - The tester attempts from the tester machine to establish an SSH connection to if1 by login as user A and using for two consecutive times a wrong password. At the third tentative the tester inserts the correct password and the log in is successful.

  - The tester checks the log and verifies the presence of two different entries  recording the two failed login attempts for the SSH access.

  - The tester verifies that each entry contains the timestamp, the IP address of the tester machine from which the tester connect via SSH to the network product and the username.

- The tester attempts to log in the network product locally as user A with a wrong password.

  - The tester checks the log and verifies the presence of  a new entry recording this failed login attempt.

  - The tester verifies that this new entry contains the timestamp and  the username.

## Expected Results:

## Incorrect login attempts are correctly logged by the network product.

## Expected format of evidence:

A testing report provided by the testing agency which will consist of the following information:

- The account created for the test and the configured username

- Log (e.g. file)

- Test result (Passed or not)

Test Name: TC_ADMINISTRATIVE_ACCESS_LOGGING

## Purpose:

To verify that the network product correctly logs the Administrative Access event type.

## Procedure and execution steps:

## Pre-Conditions:

- The following information shall be provided by the documentation accompanying the network product:

- The log where the event is recorded (e.g. in the Security Log rather than, for instance, System log) and its location in the network product (i.e. the complete path).

- If the event type is enabled by default or how to enable it.

- If the SSH service is enabled  by default or how to enable it.

- The tester has administrative privileges

- A tester machine is available.

- The network product has at least one interface (named if1) enabled.

## Execution Steps

- The tester create an administrative account A with username and password.

- The tester enables the SSH service on if1 interface if disabled

  - The tester from the tester machine establishes an SSH connection by log in the network product as user A.

  - The tester checks the log and verifies the presence of a new entry recording this access with an administrative account.

  - The tester verifies that this new entry contains the timestamp, the IP address of the tester machine from which the tester connect via SSH to the network product, the logged in username and the length of the session.

- The tester logs in the network product locally as a user A.

  - The tester checks the log and verifies the presence of a new entry recording this local access with an administrative account.

  - The tester verifies that this new entry contains the timestamp, the length of the session and the logged in username.

## Expected Results:

## Administrative accesses are correctly logged by the network product.

## Expected format of evidence:

A testing report provided by the testing agency which will consist of the following information:

- The account created for the test and the configured username

- Log (e.g. file)

- Test result (Passed or not)

Test Name: TC_ACCOUNT_ADMINISTRATION_LOGGING

## Purpose:

To verify that the network product correctly logs the Account Administration event type.

## Procedure and execution steps:

## Pre-Conditions:

- The following information shall be provided by the documentation accompanying the network product:

- The log where the event is recorded (e.g. in the Security Log rather than, for instance, System log) and its location in the network product (i.e. the complete path).

- If the event type is enabled by default or how to enable it.

- The tester has administrative privileges.

## Execution Steps

- The tester logs in the network product with an administrative account (named A)

- The tester creates an account B with username and password.

  - The tester checks the log and verifies the presence of a new entry recording this account creation/configuration.

  - The tester verifies that this new entry contains the timestamp, the Administrator username (i.e. account A username), the Administered account (i.e. account B), the activity performed (i.e. creation/ configuration).

## Expected Results:

## Account administration activities are correctly logged by the network product.

## Expected format of evidence:

A testing report provided by the testing agency which will consist of the following information:

- The account B created for the test and the configured username

- The administrative account A used to log in the network product

- Log (e.g. file)

- Test result (Passed or not)

Test Name: TC_RESOURCE_USAGE_LOGGING

## Purpose:

To verify that the network product correctly logs the Resource Usage event type.

## Procedure and execution steps:

## Pre-Conditions:

- The following information shall be provided by the documentation accompanying the network product:

- The log where the event is recorded (e.g. in the Security Log rather than, for instance, System log) and its location in the network product (i.e. the complete path).

- If the event type is enabled by default or how to enable it.

- The defined threshold values and how to reproduce a critical rise.

- The tester has administrative privileges.

## Execution Steps

- Based on the information provided by the vendor, the tester creates a critical rise in the network product.

- The tester logs in the network product with an administrative account.

  - The tester checks the log and verifies the presence of a entry recording this situation.

  - The tester verifies that this new entry contains the timestamp, the value exceeded and the value reached.

## Expected Results:

## Critical rises are correctly logged by the network product.

## Expected format of evidence:

A testing report provided by the testing agency which will consist of the following information:

- The network product configuration to produce the critical rise

- Log (e.g. file)

- Test result (Passed or not)

Note: If the Event Type “Change of group membership for accounts” is supported, an analogous test case has to be performed.

***	END OF CHANGES	***