# 3GPP TSG SA WG3 (Security) Meeting #94	S3-190198

# Kochi (India), 28 January – 1 February 2019

# Source:	Huawei, HiSilicon

# Title:	URLLC solution for N3 tunnel redundancy

# Document for:	Approval

Agenda Item:	8.13

# 1	Decision/action requested

This contribution proposes a pCR for a solution to TR 33.825.

# 2	References

[1]	3GPP TS 23.725 Study on enhancement of Ultra-Reliable Low-Latency Communcation (URLLC) v0.4.0

# 3	Rationale

The KI#1 is proposing to address the security policy handling for redundant transmission. Even if redundant transmission is the means to guarantee the high-reliability communication, the added user plane paths for transmission redundancy shall provide equal level of security as is provided to the single path. According to SA2 TR23.725 [1], there is a conclusion for redundant transmission that recommends to support high-reliability via single RAN and single UPF. In this case, there are two redundant N3 tunnels established for transferring the same data.

The contribution proposes a solution addressing the security support for N3 tunnel redundancy.

# 4	Detailed proposal

It is proposed to approve the following changes for inclusion in TR 33.825.

***	BEGIN CHANGES	***

## 6.X	Solution <X>: Security support for N3 tunnel redundancy

### 6.x.1	Introduction

This solution addresses Key Issue#1. If the user data is redundant by means of two duplicated N3 tunnels to fulfill the high-reliability feature, as illustrated in figure 6.x.1-1 below, the NDS/IP framework which is used to secure the network domain interfaces should be reused with the following precautions.

Figure 6.x.1-1 redundant transmission architecture

This data redundancy architecture supports the redundant transmission based on two N3 tunnels between a single NG-RAN node and the UPF. It is required that the RAN node and UPF shall support NDS/IP to ensure the security of the N3 interface. The transport of user data over N3 shall be integrity, confidentiality and replay-protected.

### 6.x.2	Solution details

If the IPsec tunnel is used to protect all N3 tunnels between NG-RAN and UPF, the current mechanism defined in TS33.501[X] clause 9.3 and clause 9.1 shall be reused.

The N3 interface for 5GC and 5G-AN according to NDS/IP is specified in TS 33.210 [Y]. Traffic on interfaces carrying control plane signalling can be both integrity and confidentiality protected according to NDS/IP.

IPsec ESP implementation shall be done according to RFC 4303 [Z] as profiled by TS 33.210 [Y]. For IPsec implementation, tunnel mode is mandatory to support while transport mode is optional.

IKEv2 certificate-based authentication implementation shall be done according to TS 33.310. The certificates shall be supported according to the profile described by TS 33.310 [R]. IKEv2 shall be supported conforming to the IKEv2 profile described in TS 33.310 [R].

In order to protect the traffic on the N3 reference point, it is required to implement IPsec ESP and IKEv2 certificate-based authentication. IPsec is mandatory to implement on the NG-RAN. On the core network side, a SEG may be used to terminate the IPsec tunnel.

### 6.x.3	Evaluation

Editor’s note: The evaluation of the solution is FFS.

***	END OF CHANGES	***