# 3GPP TSG SA WG3 (Security) Meeting #96	S3-193145

# 26-30 August, Wroclaw (Poland)	revision of S3-192728

# Source:	China Mobile

# Title:	Resolving the Editor's note for Solution 5 in TR 33.853

# Document for:	Approval

Agenda Item:	8.14

1	Decision/action requested

The pCR attempts to resolve the Editor’s note for Soultion 5 in TR 33.853, and is kindly asked to be approved by SA3.

2	Reference

[1]		 TR 33.853, Technical report on key issues and potential solutions for Integrity protection of the User Plane

3	Rationale

There is an Eidtor’s note in the solution 5 of TR 33.853, i.e.“Editor’s Note: IP header may not be always in the beginning of data payload, the effectiveness of this solution in such case is ffs.”This Eidtor’s note can be resolved for the following reasons.

The data payload in the IP layer refers to the entire SDU of upper layer (i..e transport layer), which contains the transport layer header (TCP or UDP) and application data.  IP header is therefore located at the beginning of the data paylaod. Moreover, no matter whether the IP packet is fragemented or not, each PDCP packet has an IP header, as shown in the following figure. As a result, IP header in the beginning of data payload  is maintained in the  PDCP layer.

4	Detailed proposal

6.5	Solution #5: Integrity Protection of packet header in the User Plane

6.5.1	Introduction

This solution addresses key issue #5 (Optionality of integrity protection in UP DRB). The solution only protectes the integrity of the packet header when data rate is over 64kbit/s.

6.5.2	Network options affected

This solution is applicable to the following network options:

- Option 2 - NR standalone with 5G Core

6.5.3	Solution Description

The protocol stack of the user plane in 5G network differs from the protocol stack of the user plane in 4G network in that a Service Data Adaptation Protocol (SDAP) is added as shown in the Figure 6.5.3-1. The role of SDAP [2] is to map the QoS flow to the Data Radio Bearer (DRB) and mark the QoS flow identifier (ID) in the SDAP header. It provides a finer-grained management of QoS than 4G with respect to the user plane data.

Figure 6.5.3-1 Protocol stack

After the user plane packet is encapsulated, the PDCP layer packet format is as follows:

Figure 6.5.3-2 Packet format in the PDCP layer

Where HPDCP is the header of PDCP layer, HSDAP is the header of the SDAP layer, and Hdata is the header of the packet. The PDCP sequence number (PDCP SN) in the HPDCP is part of the packet count value COUNT.   To protect the header of the packet, the following parameters are input to the integrity function (e.g. NIA1) to produce MAC-I.

- COUNT，

- MESSAGE

- DIRECTION

- BEARER

- KEY

Where MESSAGE is to be integrity protected, which is the cascade of the HSDAP and Hdata.

6.5.4	Solution Evaluation

IP header may not be always in the beginning of data payload, the effectiveness of this solution needs to be carefully considered.

Editor's note: FFS