3GPP TSG-SA3 Meeting #97-LI	s3i250179

# Washington, United States, 29th Apr 2025 - 2nd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.127 | CR | 0278 | rev | 2 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Stage 2 on Personal IoT Network (PIN) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3LI (Ministère Economie et Finances) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI19 |  |  |  |  |  | Date: |  |  | 2025-04-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Absence of LI for 5G Personal IoT Network (PIN) management in Stage 2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add LI for 5G PIN management in Stage 2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The LI for 5G PIN management in Stage 2 would still be missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.3, 7.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | s3i250109, s3i250170 |  |  |  |  |  |  |  |  |

START OF FIRST CHANGE

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System".

[3]	3GPP TS 33.126: "Lawful interception requirements".

[4]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[5]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[6]	OMA-TS-MLP-V3_5-20181211-C: "Open Mobile Alliance; Mobile Location Protocol, Candidate Version 3.5", https://www.openmobilealliance.org/release/MLS/V1_4-20181211-C/OMA-TS-MLP-V3_5-20181211-C.pdf".

[7]	ETSI TS 103 120: "Lawful Interception (LI); Interface for warrant information".

[8]	ETSI TS 103 221-1: "Lawful Interception (LI); Internal Network Interfaces; Part 1: X1 ".

[9]	3GPP TS 33.501: "Security Architecture and Procedures for the 5G System".

[10]	ETSI GR NFV-SEC 011: "Network Functions Virtualisation (NFV); Security; Report on NFV LI Architecture".

[11]	3GPP TS 33.107: "3G Security; Lawful interception architecture and functions".

[12]	3GPP TS 23.214: "Architecture enhancements for control and user plane separation of EPC nodes; Stage 2".

[13]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[14]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[15]	3GPP TS 33.128: "Protocol and Procedures for Lawful Interception; Stage 3".

[16]	ETSI TS 103 221-2: " Lawful Interception (LI); Internal Network Interfaces; Part 2: X2/X3".

[17]	MMS Architecture OMA-AD-MMS-V1_3-20110913-A.

[18]	Multimedia Messaging Service Encapsulation Protocol OMA-TS-MMS_ENC-V1_3-20110913-A.

[19]	3GPP TS 22.140: "Multimedia Messaging Service (MMS); Stage 1".

[20]	ETSI GS NFV-IFA 026: "Network Functions Virtualisation (NFV) Release 3; Management and Orchestration; Architecture enhancement for Security Management Specification".

[21]	3GPP TS 33.108: "Handover Interface for Lawful Interception (LI)".

[22]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for 
Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[23]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[24]	3GPP TS 23.280: "Common functional architecture to support mission critical services; Stage 2".

[25]	OMA-AD-PoC-V2_1-20110802-A: "Push to talk over Cellular (PoC) Architecture".

[26]	GSMA IR.92: "IMS Profile for Voice and SMS".

[27]	GSMA NG.114: "IMS Profile for Voice, Video and Messaging over 5GS".

[28]	3GPP TS 24.147: "Conferencing using the IP Multimedia (IM) Core Network (CN) subsystem; Stage 3".

[29]	ETSI GS NFV-SEC 012: "Network Functions Virtualisation (NFV) Release 3; Security; System architecture specification for execution of sensitive NFV components".

[30]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[31]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage3".

[32]	3GPP TS 29.122: "T8 reference point for Northbound APIs".

[33]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[34]	OMA-AD-CPM-V2_2-20170926-C: "Open Mobile Alliance, OMA Converged IP Messaging System Description", http://www.openmobilealliance.org/release/CPM/V2_2-20200907-C/OMA-AD-CPM-V2_2-20170926-C.pdf.

[35]	GSMA RCC.07: "Rich Communication Suite – Advanced Communications Services and Client Specification".

[36]	IETF RFC 4975: "The Message Session Relay Protocol (MSRP)".

[37]	IETF RFC 6714: "Connection Establishment for Media Anchoring (CEMA) for the Message Session Relay Protocol (MSRP)".

[38]	IETF RFC 3862: "Common Presence and Instant Messaging (CPIM): Message Format".

[39]	3GPP TS 24.229: "IP Multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[40]	IETF RFC 8224: "Authenticated Identity Management in the Session Initiation Protocol (SIP)".

[41]	IETF RFC 8946: "Personal Assertion Token (PASSporT) Extension for Diverted Calls".

[42]	IETF draft-ietf-stir-passport-rcd-26, "PASSporT Extension for Rich Call Data".

NOTE:	The above document cannot be formally referenced until it is published as an RFC.

[43]	IETF RFC 7095: "jCard: The JSON Format for vCard".

[44]	3GPP TS 24.196: "Enhanced Calling Name (eCNAM)".

[45]	IETF RFC 8816: "Secure Telephone Identity Revisited (STIR) Out-of-Band Architecture and Use Cases".

[46]	IETF RFC 9475: "Messaging Use Cases and Extensions for Secure Telephone Identity Revisited (STIR)".

[47]	3GPP TS 33.535: "Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS)".

[48]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[49]	3GPP TS 33.222: "Generic Authentication Architecture (GAA); Access to network application functions using Hypertext Transfer Protocol over Transport Layer Security (HTTPS)".

[50]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[51]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[52]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[53]	3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture".

[54]	3GPP TS 29.272: "Evolved Packet System (EPS); Mobility Management Entity (MME) and Serving GPRS Support Node (SGSN) related interfaces based on Diameter protocol".

[55]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[56]	3GPP TS 38.455: "NR Positioning Protocol A (NRPPa)".

[57]	3GPP TS 36.455: "LTE Positioning Protocol A (LPPa)".

[58]	3GPP TS 23.140: "Multimedia Messaging Protocol. Functional Description. Stage 2".

[59]	3GPP TS 32.303: "Proximity-based Services (ProSe); Stage 2".

[60]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[61]	3GPP TS 32.277: "Proximity-based Services (ProSe) charging".

[X]	3GPP TS 23.542: "Application layer support for Personal IoT Network".

END OF FIRST CHANGE

START OFSECOND CHANGE

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

5GMS	5G Media Streaming

5GS	5G System

AAnF	AKMA Anchor Function

AC	Application Client

ACR	Application Context Relocation

ADMF	LI Administration Function

AF	Application Function

AF_ID	Application Function Identity

AKA	Authentication and Key Agreement

A-KID	AKMA Key IDentifier

AKMA	Authentication and Key Management for Applications

AMF	Access and Mobility Management Function

AS	Application Server

AUSF	Authentication Server Function

BBIFF	Bearer Binding Intercept and Forward Function

BSS	Business Support System

CAG	Closed Access Group

CC	Content of Communication

CP	Control Plane

CP NF	Control Plane Network Function

CPIM	Common Presence and Instant Messaging

CPS	Call Placement Service

CSI	Cell Supplemental Information

CSP	Communication Service Provider

CSR	Cell Site Report

CUPS	Control and User Plane Separation

DC-AS	Data Channel Application Server

DCSF	Data Channel Signalling Function

DDNMF	Direct Discovery Name Management Function

DN	Data Network

DNAI	Data Network Access Identifier

DoNAS	Data over NAS

EAP	Extensible Authentication Protocol

EAS	Edge Application Server

ECGI	E-UTRAN Cell Global Identifier

eCNAM	Enhanced Calling Name

ECSP	Edge Computing Service Provider

E-CSCF	Emergency – Call Session Control Function

EDN	Edge Data Network

EEC	Edge Enabler Client

EECID	Edge Enabler Client IDentifier

EES	Edge Enabler Server

eIMS-AGW	enhanced IMS-AGW

eP-CSCF	enhanced P-CSCF

GPSI	Generic Public Subscription Identifier

HMEE	Hardware Mediated Execution Enclave

HR	Home Routed

IBCF	Interconnection Border Control Functions

ICF	Identity Caching Function

IEF	Identity Event Function

IMS-AGW	IMS Access Gateway

IMS-HSS	HSS supporting IMS services for 5GC

IM-MGW	IM Media Gateway

IP	Interception Product

IPPR	Internet Protocol Packet Reporting

IQF	Identity Query Function

IRI	Intercept Related Information

KAF	AKMA Application Key

KAKMA	AKMA Anchor Key

KID	Key IDentifier

KLI	Decryption key(s) for services encrypted by CSP-provided keys

KSF	Key Server Function

LAF	Location Acquisition Function

LALS	Lawful Access Location Services

LARF	Location Acquisition Requesting Function

LBO	Local Break Out

LEA	Law Enforcement Agency

LEMF	Law Enforcement Monitoring Facility

LI	Lawful Interception

LI CA	Lawful Interception Certificate Authority

LICF	Lawful Interception Control Function

LI_HI1	Lawful Interception Handover Interface 1

LI_HI2	Lawful Interception Handover Interface 2

LI_HI3	Lawful Interception Handover Interface 3

LI_HI4	Lawful Interception Handover Interface 4

LI_HILA	Lawful Interception Handover Interface Location Acquisition

LI_HIQR	Lawful Interception Handover Interface Query Response

LIID	Lawful Interception Identifier

LIPF	Lawful Interception Provisioning Function

LIR	Location Immediate Request

LI_SI	Lawful Interception System Information Interface

LISSF	Lawful Interception State Storage Function

LI_ST	Lawful Interception State Transfer Interface

LI_T1	Lawful Interception Internal Triggering Interface 1

LI_T2	Lawful Interception Internal Triggering Interface 2

LI_T3	Lawful Interception Internal Triggering Interface 3

LI_X0	Lawful Interception Internal Interface 0

LI_X1	Lawful Interception Internal Interface 1

LI_X2	Lawful Interception Internal Interface 2

LI_X2_LA	Lawful Interception Internal Interface 2 Location Acquisition

LI_X3	Lawful Interception Internal Interface 3

LI_X3A	Lawful Interception Internal Interface 3 Aggregator

LI_XEM1	Lawful Interception Internal Interface Event Management Interface 1

LI_XER	Lawful Interception Internal Interface Event Record

LI_XLA	Lawful Interception Internal Interface Location Acquisition

LI_XQR	Lawful Interception Internal Interface Query Response

LMF	Location Management Function

LMISF	LI Mirror IMS State Function

LMISF-CC	LMISF for the handling of CC

LMISF-IRI	LMISF for the handling of IRI

LTF	Location Triggering Function

MA	Multi-Access

MANO	Management and Orchestration

MDF	Mediation and Delivery Function

MDF2	Mediation and Delivery Function 2

MDF3	Mediation and Delivery Function 3

MF	Media Function

MRFP	Multimedia Resource Function Processor

MSRP	Message Session Relay Protocol

N3A	Non-3GPP Access

N3IWF	Non 3GPP Inter Working Function

N9HR	N9 Home Routed

NAS	Non-Access Stratum

NCGI	NR Cell Global Identity

NEF	Network Exposure Function

NFV	Network Function Virtualisation

NFVI	Network Function Virtualisation Infrastructure

NFVO	Network Function Virtualisation Orchestrator

NIDD	Non-IP Data Delivery

NNI	Network to Network Interfaces

NPLI	Network Provided Location Information

NR	New Radio

NRF	Network Repository Function

NSSF	Network Slice Selection Function

NWDAF	Network Data Analytics Function

OSS	Operations Support System

PAG	POI Aggregator

PCF	Policy Control Function

P-CSCF	Proxy - Call Session Control Function

PEI	Permanent Equipment Identifier

PGW	PDN Gateway

PGW-C	PDN Gateway Control Plane

PGW-U	PDN Gateway User Plane

PIN	Personal IoT Network

POI	Point Of Interception

PLMN	Public Land Mobile Network

ProSe	Proximity based Services

PTC	Push to Talk over Cellular

RCD	Rich Call Data

RCS	Rich Communication Suite

S8HR	S8 Home Routed

SCEF	Service Capability Exposure Function

SCS	Service Capability Server

SGW	Serving Gateway

SGW-C	Serving Gateway Control Plane

SGW-U	Serving Gateway User Plane

SHAKEN	Signature-based Handling of Asserted information using toKENs

SIRF	System Information Retrieval Function

S-CSCF	Serving - Call Session Control Function

SIP	Session Initiation Protocol

SMF	Session Management Function

SMSF	SMS-Function

STF	Security Terminating Function

STIR 	Secure Telephony Identity Revisited

SUCI	Subscriber Concealed Identifier

SUPI	Subscriber Permanent Identifier

TAI	Tracking Area Identity

TF	Triggering Function

TLS	Transport Layer Security

TNGF	Trusted Non-3GPP Gateway Function

TrGW	Transit Gateway

TWIF	Trusted WLAN Interworking Function

UDM	Unified Data Management

UDR	Unified Data Repository

UDSF	Unstructured Data Storage Function

UPF	User Plane Function

VN Group	Virtual Network Group

VNF	Virtual Network Function

VNFC	Virtual Network Function Component

W-AFG	Wireline Access Gateway Function

WAF	WebRTC Authorisation Function

WIC	WebRTC IMS Client

WWSF	WebRTC Web Server Function

xCC	LI_X3 Content of Communication

xIRI	LI_X2 Intercept Related Information

END OF SECOND CHANGE

START OF THIRD CHANGE

## 7.X	LI at PIN Server

### 7.X.1	Background

5G Personal IoT Network (PIN) is described in 3GPP TS 23.542 [X] and is defined as a group of PIN Elements (PINE) that are able to communicate each other directly or via PIN Elements with Gateway Capability (PEGC), communicate with 5G network via at least one PEGC, and managed by at least one PIN Element with Management Capability (PEMC). A PINE may be a UE or non-3GPP device. PEGC and PEMC are UEs.

The PIN architecture consists of PIN client deployed in PIN Element and PIN Server deployed in Data network. The PIN Server can be deployed by mobile operator or an authorized third party. When the PIN Server is deployed by third party, the interworking with 5GS need to be transferred via NEF. PIN Server may support the following functionalities:

-	Registration management of PINE, PEGC, PEMC.

-	PIN management and PIN status management.

-	Configuration and parameters provisioning to PEGC, PEMC, PINE.

-	Support for discovery of PIN, PINE and PEGC.

-	AF requested QoS control for PEGC and specific PINE traffic flow.

The overall 5G PIN Architecture is shown in Figure 6.2.1-1 of clause 6.2.1 of TS 23.542 [X].

### 7.X.2	Architecture

The PIN Server forms part of the Application layer support for Personal IoT Networks defined in 3GPP TS 23.542 [X]. It is aimed to support the server-side functionalities required for managing the PIN and the PIN clients. The PIN Server shall provide the IRI-POI function. Figure 7.X.2-1 gives a reference point representation of the LI architecture with PIN Server as a CP NF providing the IRI-POI function for PIN management.

Figure 7.X.2-1: LI architecture for Personal IoT showing LI at PIN Server

### 7.17.3	Target identities

The LIPF present in the ADMF provisions the intercept information associated with the following target identities to the IRI-POI present in the PIN Server:

-	GPSI.

-	PIN client ID.

-	Identity token.

-	PIN ID.

### 7.17.4	IRI events

The IRI-POI in the PIN Server shall generate xIRI when it detects the following specific events or information:

-	PINAPP message (see clauses 8.3 to 8.10 of TS 23.542 [X]).

-	Start of interception with PIN client in PIN.

The PINAPP message xIRI is generated when the IRI-POI present in the PIN Server detects that interactions relating to PIN application layer procedures occur between the PIN Server and:

-	a target PIN client.

-	any PIN client of a target PIN.

The start of interception with PIN client in PIN xIRI is generated when the IRI-POI present in the 5G PIN Server detects that interception is activated:

-	on a target PIN client which is already member of one or several PINs.

-	on a target PIN with PIN clients already made as members.

END OF THIRD CHANGE