3GPP TSG SA WG-4 Meeting #133-e	S4-251201

Online, July 10 – 23  2025                                            	In revision of S4-250776

# Source:	Huawei, HiSilicon

# Title:	New SID on Dynamically Changing Traffic Characteristics in Media Applications and Services

# Document for:	Agreement

# Agenda Item:	17.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Study on Dynamically Changing Traffic Characteristics in Media Applications and Services

Acronym: FS_DCTC_MED

Unique identifier:	 xxxx

{A number to be provided by MCC at the plenary}

Potential target Release:	Rel-20

# 1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | x |  |  |  |
| No |  |  | x | x | x |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a study

| x | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

### 2.3	Other related Work Items and dependencies

FS_XRTraffic defined traffic models and quality evaluation methods for media and XR Services in the 5G Network (see TR 26.926). FS_TyTrac studied typical traffic characteristics of media services and defined the generic traffic characteristics such as bitrate, file format and provides a summary of the work in 3GPP on QoS (see TR 26.925).

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 870013 | FS_XRTraffic | Modelling and evaluation of end-to-end XR services in 5G network |
| 810005 | FS_TyTrac | Defines the basic (aggregate) traffic characteristics such as bit-rate media format etc. for XR services |
| 1030000 | FS_5G_RTP_PH2 | Studied some of the dynamically changing traffic characteristics for the case of RTP |
| 960046 | 5G_RTP | Defined some of the user plane signaling for dynamic traffic characteristics in RTP |

# 3	Justification

Neither document TR 26.926 nor TR 26.925 present a study on dynamically changing traffic characteristics in common media applications and services with a link to exploiting them using the features available in the 5GS.

Additional support for transmission of content with dynamically changing traffic characteristics was added to the 5G System, see Table 1 for an overview of these features from TS 23.501. In RAN benefits related to power saving and scheduling which can benefit the UE have been studied and adopted.  SA4 should study how such dynamically changing traffic characteristics can be characterized and identified in media applications and services, mainly to make it easier in applications to detect and identify characteristics. If additional definition and clarification is needed from the SA4 perspective this should be documented. Some related characteristics in the 5GS are shown in Table 1.

NOTE: These characteristics are of interest as these have adopted in 5GS, in case other dynamic aspects are identified they can be documented in this study, but will be considered with a lower priority as RAN and SA work for 5GS is considered frozen in this study.

Table 1  Some 5GS features based on traffic characteristics defined in TS 23.501

| Feature | Reference | Benefit |
| --- | --- | --- |
| a) Data Burst Size | TS 23.501 clause 5.37.10.1 | Data burst for improved scheduling of the transmission and power saving |
| b) Time to Next Burst | TS 23.501 clause 5.37.10.2 | Time to next burst for improved transmission and resource saving |
| c) Expedited data transfer with reflective QoS | TS 23.501 clause 5.37.10.3 | Improve data transmission by using alternative QoS enabling expedited data transfer |
| d) End of Data Burst indication | TS 23.501 clause 5.37.8.2 | Improved power saving and potentially improved transmission |

Both XR and non-XR Media applications and services can benefit from these features in ways defined and studied in other working groups, resulting in more efficient transmission and power management.  However support and understanding of such characteristics is limited. The detection and identification of such dynamically changing traffic characteristics (in Table 1), both on the uplink and downlink, is not well understood and documented for media applications and services.

This study aims at:

- Developing tests for identifying such characteristics in media applications and services

- Characterization of features in different media applications and services with additional parameters (start, end, volume).

- Potentially defining other different dynamic traffic characteristics.

Some of the characteristics to be studied in different applications and service considered are:

- Variable bit-rate support in encoders using expedited data transfer.

- Time to next burst, (b) from Table 1, in live and video on demand or live streaming scenarios.

- Data burst size indication in real-time communication services.

- Data bursts in a-periodic media streams with high burst frequency and low burst data volume.

- Aspects related to multi-modality and relationships between streams can be studied when considering dynamic traffic.

- Characteristics of bursty uplink transmission of captured media for rapid AI inference in the (edge-) cloud. The captured media can include video, audio and/or text.

Dynamically changing traffic characteristics occurring in relevant scenarios will be documented.

Further characterization and identification can be useful for subsequent normative work. Such normative work could result in:

- Specification of the dynamically changing traffic characteristics and their link to features from 23.501 in Table 1, and common features in media applications (e.g. frame rate, mpd update period and others) where possible, as to enable media application and developers to better understand how to make use of such features.

- Additional improvements to using QoE and QoS frameworks (and potential additional quality metrics) for media applications and services, in case some of these characteristics impact such framework.

These aspects are relevant for both low-bitrate and high bitrate applications. For example, in the study of haptic media, traffic characteristic evaluation has highlighted that the bit-stream includes larger bursts but overall, the stream has a very low bandwidth. In AR/VR services animation and/or pose streams sent uplink may have irregular traffic pattern with low bitrate and burst characteristics. In high bitrate conversations and XR streams, patterns of data bursts and aperiodicity have also been identified in 5G RTP Ph2 and FS_XR_Traffic.

An emerging use-case of transmitting media to be leveraged by an AI enhanced application can impact traffic characteristics in new ways. On the uplink, media is transmitted that may include, chunks of video, audio, video and text generating a periodic or aperiodic burst pattern.

Generally High bitrate/low latency applications may experience the most benefit as congestion is most likely in these cases, such as XR video or High quality video conferencing and streaming applications.

# 4	Objectives

The objectives are:

- Identify and describe the functionality of popular media applications and service scenarios that may exhibit dynamically changing traffic characteristics (such as defined in Table 1), such as:

  - Real-Time Communication for conversational, XR and/or gaming applications and services, both on the uplink and downlink.

  - Video on demand streaming and progressive download

  - Live Streaming

  - Short form video download

  - Media transmission for upstream AI inference.

NOTE: 	The aim is to strengthen the generic support for these applications in the 5GS. The aim is not to include them in an SA4 architecture or make them part of SA4 specification (if they are not already).

- Develop tests to evaluate these scenarios for both uplink and downlink traffic. Tests will collect network traffic traces in real and emulated network conditions and include quality related aspects. The tests will be documented to enable reproducing them.

- Characterize and document dynamically changing traffic characteristics observed in these scenarios for single stream or dependent multi-modal streams and:

  - Identify how they relate to the 3GPP system (i.e.., see Table 1), if applicable.

  - Identify specific characterization mechanism to enable identification and detection of these characteristics in media applications and services, i.e. work on definition of SA4 defined dyn. traffic characteristics corresponding to Table 1 with additional constraints if needed to enable detection and identification in media services and applications.

NOTE: 	examples of characterization in media applications could be the following:

    - Conceptual or mathematical Models to describe dynamic traffic characteristic including their timing (start, end) and size (volume)

    - Media application specific aspects that can enable linking to dynamic traffic characteristics (e.g. frame rate to time to next burst etc., minimum update period of media presentation description to time to next burst)

  - Dynamic traffic characteristics not related to those currently defined in the 3GPP system may be documented, but will be studied with a lower priority as RAN and SA specifications are considered frozen in this study. However some of these related aspects such as idle time or variable burst sizes may be of interest to explore.

Based on the findings of the previous objectives, work on the following objective will be targeted:

- Identify normative work based on the potential solutions documented by this study.

  - Normative definition of media application related dynamically changing traffic characteristics in different media applications and services, relating to those supported in the 5G system.

  - Explore if any traffic characteristics from TR 26.925 and/or TR 26.926 would benefit from definition in a normative specification

  - Potential definition of any other relevant traffic characteristic

# 5	Expected Output and Time scale

| Impacted existing TS/TR |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TR 26.926 | Additional information on dynamically changing traffic characteristics | SA#113 (Sep ’26) | Add additional updates and finding of this study to report 26.926 |
| TR 26.925 | Additional information about dynamically changing traffic characteristics | SA#113 (Sep ’26) | Add additional updates and finding of this study to report 26.925 |
| TR 26.812 | Additional QoE metric handling | SA#113 (Sep ’26) | Optional in case there are updates related to XR use cases |

# 6	Work item Rapporteur(s)

Rufael Mekuria Rufael.mekuria@huawei.com

# 7	Work item leadership

SA4

# 8	Aspects that involve other WGs

Inform SA2 or RAN-2 about this study at the beginning and at the end.

Coordination with different 3GPP working groups if requested

# 9	Supporting Individual Members

Supporting IM name

Huawei

HiSilicon

[Interdigital Communications]

[Peng Cheng Laboratory]

[Meta]

[CMCC]

[CATT]

Sony??

Vivo??