Source:	Beijing Xiaomi Mobile Software

Title:	Update templates for DaCAS recording scenarios and databases

Document for:	Discussion & Agreement

Agenda Item:	1.6

# Introduction

Based on related contributions from the SA4 #131 Geneva meeting and the post-131 telcon meeting held on 10th March 2025, as well as the agreements in the telcon meeting, this document updates the initial templates proposed in[1] by incorporating agreed improvements from[2], the updated revisions aim to:

- Specify three recording scenario types.

- Clarify relationships between target device, recording scenario, and recording databases.

- Remove redundancies and enhance template fields for reproducibility.

- General recommendations on defining recording scenarios.

# Recommendations

## 2.1 Recording scenario types

There are three recording scenario types proposed as follows:

Device characterization

- Recordings targeted to identify electro-acoustical characteristics of proposed target devices. For example, to identify device frequency response, etc.

Development recordings

- Recordings targeted to facilitate spatial capture algorithm development and potential tuning.

Evaluation recordings

- Recordings targeted to evaluate example solutions, including subjective and objective evaluation. For objective evaluation, alignment with TS 26.260 is expected.

Above categories should not be understood as strictly orthogonal scenarios, as it is expected that certain scenarios/recordings may be useful for multiple purposes. Furthermore, while other recording scenario types may be anticipated than what is presented here, it is seen useful to consider whether and how the proposed recording scenarios would fit into the above recording scenario types.

## 2.2 Recommendations on defining recording scenarios

Along the above recording scenario types, different recording scenarios for different purposes are anticipated. To ensure definition of meaningful and useful recording scenarios, at least following aspects should be considered when defining new recording scenarios:

- Varying recording scenarios should be represented, from simple single source scenarios to complex scenarios, and from free-field conditions to reverberant conditions.

- For the sake of reproducibility, high quality loudspeakers with well-defined source signals should be favored when possible.

- For device characterization, algorithm development, and objective evaluation purposes, controlled rooms in terms of reverberation and room frequency response should be favored. Referencing existing specifications is recommended, if applicable.

- For subjective evaluation purposes, vast number of varying environments should be represented. Number of proposed recording scenarios in indoor and outdoor environments should be in balance.

- Additional care should be taken when describing and defining the positioning of sound sources and UE.

- Proposed updates on recording scenarios and database

## 3.1 proposal of the relationships of the three templates

The relationships of the three templates are illustrated in Figure 1 below. Furthermore, cross-referencing between the databases should be favored in general, in order to reduce redundancy.

Target device database

Target device No

Type

Size

Shape

…

Recordings database

Recording item No

Target device No

Rec scenario No

UE orientation

…

Shape

…

Recording scenarios database

Recording scenario No

Sound source

Signal characteristics

Playback level calibration

…

Figure 1 – Templates/databases relationship

## 3.2 Update template for recordings database.

New identifier fields for target devices and recording scenarios are added into the template, the new template is in “Template for recordings database. xlsx”

## 3.3 Update template for recording setups and scenarios

The calibration level is changed from “0 dBFS” to “-3 dBFS” in item of “Sound source calibration”, the new description is “Loudspeaker calibrated with  -3 dBFS pink noise to 80 dBSPL(A) at the UE position”.

The new template is in “Template for recording setups and scenarios. xlsx”

- Conclusion

It is recommended that the templates in clause 3 are used for the basis for further updates and collecting databases, and the contents in clause 2 should be considered when designing the databases.

# References

[1] S4-250289: Templates for target devices and common database

[2] S4aA250008: On DaCAS recordings and recording scenarios