Source:	Bytedance

Title:	Recordings of smartphone-like target device

Document for:	Discussion

Agenda Item:	1.7 – DaCAS

# 1. Introduction

In the DaCAS work plan [1], definitions of test methods for example solutions are included. Initial discussion has been triggered in [2], [3]. In this document, we propose agreement for additional objective performance test methods for Scene-Based Audio example solutions.

# 2. Objective performance evaluation

Four different test methods for assessing the sound quality of IVAS-based UEs are defined in section 5 of 3GPP TS 26.260[4], and have been discussed as evaluation methodologies for DaCAS solutions in [3]. However, these methods focus on simple sound scene evaluation where only one sound source is considered. DaCAS example solutions, on the other hand, are expected to tolerate more complex scenes with multiple sound sources.

In section 5.4 of [5], objective test methods for scene-based audio format in sending direction have been agreed. Thus, we propose those tests to objectively assess the performance of available DaCAS example solutions in SBA format.

# 3. Recording scenarios for evaluations

To obtain suitable recordings for the evaluation in Section 2, we also propose the following 3 recording scenarios with multiple sound sources to be included in the database. These recording scenarios should be made by all available target devices, or, at a minimum, target devices with potential solutions in the SBA format. The proposed recording scenario is also attached as Recording_scenario_performance_evaluation.xlsx.

Table x-1: Recording scenario for multiple sound source scenario 1

| No | X-1 (objective testing) | X-2 (objective testing) | X-3 (objective testing) |
| --- | --- | --- | --- |
| Sound source | Loudspeakers or HATSs according to clause 4.0.2 in TS 26.260 | Loudspeakers or HATSs according to clause 4.0.2 in TS 26.260 | Loudspeakers or HATSs according to clause 4.0.2 in TS 26.260 |
| Source signal | British English single talk sequence according to the ITU-T P.501 | British English single talk sequence according to the ITU-T P.501 | British English single talk sequence according to the ITU-T P.501 |
| Signal characteristics | Language: English Gender: Male, Female Range: 20Hz-20kHz Length:35.4 s Level: -27 dBRMS Sample Rate: 48 kHz Bit depth: 16 bit | Language: English Gender: Male, Female Range: 20Hz-20kHz Length:35.4 s Level: -27 dBRMS Sample Rate: 48 kHz Bit depth: 16 bit | Language: English Gender: Male, Female Range: 20Hz-20kHz Length:35.4 s Level: -27 dBRMS Sample Rate: 48 kHz Bit depth: 16 bit |
| Sound source calibration | Loudspeaker is calibrated with -3 dBFS pink noise to produce 70 dBSPL (A) at the UE position. Same playback settings are then used with the defined test signal | Loudspeaker is calibrated with -3 dBFS pink noise to produce 70 dBSPL (A) at the UE position. Same playback settings are then used with the defined test signal | Loudspeaker is calibrated with -3 dBFS pink noise to produce 70 dBSPL (A) at the UE position. Same playback settings are then used with the defined test signal |
| Acoustic environment | According to clause 4.0.3 of 3GPP TS 26.260 | According to clause 4.0.3 of 3GPP TS 26.260 | According to clause 4.0.3 of 3GPP TS 26.260 |
| Detailed Positioning | Source distance: >0.5 m Source direction: Loudspeaker diaphragm towards the UE Source angles: -60°, 30°, | Source distance: >0.5 m Source direction: Loudspeaker diaphragm towards the UE Source angles: -30°, 90°, | Source distance: >0.5 m Source direction: Loudspeaker diaphragm towards the UE Source angles: -90°, 0°, 60° |
| Reference recording | Same setup, recorded Eigenmike em32 or em64 (should be documented) | Same setup, recorded Eigenmike em32 or em64 (should be documented) | Same setup, recorded Eigenmike em32 or em64 (should be documented) |
| Description/additional info | Silence before and after the measured direction shall be at least 1 s | Silence before and after the measured direction shall be at least 1 s | Silence before and after the measured direction shall be at least 1 s |

# 4. Proposal

- We propose additional objective test methods to evaluate the performance of DaCAS solutions for SBA format.

- Corresponding recording scenarios are also proposed to be included in the recording scenarios database to conduct the proposed test methods.

The source proposes to trigger the discussion on those topics in order to make test of DaCAS solutions more robust.

# References

[1] 	S4-251169_Work Plan for DaCAS-v0.4

[2] 	S4-250957 On performance evaluation

[3] 	S4aA250021 On objective performance evaluation

[4] 	3GPP TS 26.260: “Objective test methodologies for the evaluation of immersive audio systems” version 18.1.0 Release 18

[5] 	S4-251020 - PDoc ATIAS-2 v0.4