Source:	Nokia

Title:	On loudspeaker setups

Document for:	DISCUSSION

Agenda item:	1.6 – ATIAS_Ph2

# 1. Introduction

During the recent SA4 meetings, applied loudspeaker setups for different tests has been widely discussed in documents [1], [2], [3] and [4]. Varying views on applying, e.g., loudspeaker setup from ETSI TS 103 224 for certain test scenarios and on utilizing periphonic array with high number of loudspeakers for other test scenarios are discussed.

One identified issue with varying views on the applied loudspeaker setups is the test setup compatibility across different test scenarios. In this document, we present further views on how the existing and future test setups could be defined to yield sufficient modularity and compatibility across different test scenarios.

# 2. Discussion

# In the current test specification for IVAS-based UEs, 3GPP TS 26.260 [5], only single source tests are specified in send direction. Furthermore, new test methods for assessing the ambience transmission are currently under development within this work item. In addition, it is envisioned that in the future there might be need to define tests for assessing capture of multiple static sound sources, tests to assess forefront speech characteristics in presence of ambient sound field, and tests to assess capture in complex sound scenes, e.g., comprising dynamic movement.

# Each test scenarios has their unique needs regarding the applied test setup in terms of number of loudspeakers and their placement. For example, when assessing the fundamental characteristics of ambient transmission, it is considered that extensively accurate spatial reproduction is not necessarily required. On the other hand, if transmission of complex sound scenes would be assessed, the spatial accuracy of the reproduction system may need to be higher than the assessed spatial capture accuracy. In addition, when considering all the possible test scenarios, the required distances between UE and all applied loudspeakers may not be equal.

# Moreover, wide variety of different test scenarios may easily lead to scattered definitions of applied test setups, as the playback reproduction requirements may vary depending on the tested scenario. At the same time it is considered important that the applied loudspeaker configuration is appropriate as well as the most feasible option for the assessed scenario, e.g., overly specified loudspeaker system with highest spatial accuracy (number of loudspeakers), should not be required for a test without technical justification.

# Thus, to avoid specifying multiple specific loudspeaker configurations for individual tests, test setups should be defined to yield sufficient modularity and compatibility across all specified test setups. For example, when defining new test utilizing test setup with high number of loudspeakers, it would be seen beneficial to inherently include loudspeakers from other specified setups comprising lower number of loudspeakers, if applicable.

# In the Figure 1 below, the discussed approach for defining test setups is illustrated including existing, proposed and possible future test setups. In the first box at the left, a setup for static single source tests utilizing one loudspeaker and turntable is illustrated. In the middle box, the same setup for static single source testing is extended with sufficient number of loudspeakers to achieve compatible setup for, e.g., testing multisource scenarios, ambient transmission scenarios, and speech in presence of ambient sound field scenarios. In the rightmost box, the previous test setups are further extended with elevated sound sources to produce more complex sound fields. The actual defined test setups may have higher granularity than what is illustrated, i.e., number of loudspeakers for extended setups may vary depending on, e.g., the targeted spatial reproduction accuracy or the assessed UE-type.

Figure 1 Illustration of envisioned test setup modularity for existing and possible future test methods.

# With presented approach future revisions of the test specification would be more manageable by, e.g., test laboratories in terms of backwards compatibility as well as predictability. Naturally, presented approach should not be prioritized over the technical requirements of the defined tests. For example, defining a test requiring highly specific test setup should not be prevented even if the presented approach could not be followed, if the test in overall is considered to be important and acceptable. In addition, as some of the applied test setups may follow technical specifications controlled by other standardization bodies, it may not be always possible to obtain coherent set of utilized test setups in terms of modularity and compatibility across all defined tests.

# 3. Conclusion

It is considered that the current and possible future tests for assessing IVAS-based UEs may require multiple different loudspeaker setups to address certain test specific needs and to reconstruct sound fields with intended accuracy/characteristics. To avoid specifying multiple different loudspeaker setups for different test scenarios, we consider that inherent compatibility and modularity should be considered across defined test setups, if applicable. On the other hand, such compatibility and modularity should not be prioritized over the technical needs of the defined tests.

# 4. References

[1]	S4-251004: “Assessing Diffuseness Reproduction of Loudspeaker Layouts for ATIAS”, Qualcomm Incorporated

[2]	S4-250612: “Using a Periphonic Speaker Array for Improved Soundfield Reconstruction”, Qualcomm Incorporated

[3]	S4-250088: “Toward Test Setup and Room Conditions for Send Tests with Complex Sound Scenes”, HEAD Acoustics GmbH

[4]	S4-250205: “On testing UEs in ambience”, Nokia

[5]	3GPP TS 26.260: "Objective test methodologies for the evaluation of immersive audio systems", V18.1