3GPP TSG-SA WG4 Audio SWG post 132	S4aA250075

Online, July 11,  2025                                                                   revision of S4aA250073

Source:	Dolby Laboratories, Inc., Nokia, Novamint, EDF

Title:	[FS_ULBC] On ULBC bit rates

Agenda item:	1.9

Document for:	Agreement

- Introduction

For IMS Voice Call over GEO, in TR 22.887, a “Transmission data rate” of [1-3] kbit/s is assumed. While “Transmission data rate” is not defined in TR 22.887, discussions are ongoing as to what available source (or codec) bit rates this would correspond to depending on, e.g., the chosen transport formats, the bundling time, the potential packet loss resilience of a future ULBC codec, and RTP/UDP/IP packetization overhead.

This contribution addresses the question of whether the ULBC should operate on a specific required bit rate and how this bit rate should be determined. Also discussed is the concept of ‘feasible TBS’ that is currently assumed by certain parties to be used in the determination of the ULBC bit rate.

- Past practice

The question of potentially setting required bit rates was discussed and decided in several 3GPP speech codec standardization exercises.

In case of EVS, design constraint bit rates were based on TBSs (Transport Block Sizes) originally designed for the UTRAN system to allow optimized transmission of AMR and AMR-WB speech codec frames. IVAS codec design constraint bit rates followed the EVS codec bit rates.

It is thus worth understanding how source coding bit rates for AMR and AMR-WB were decided. As can be seen from AMR-WB design constraints [1] and performance requirements [2], no specific bit rates were required in either of these Pdocs. Absent deeper insights than what codec proponents had with their specific domain knowledge, finding the best possible bit rates was considered part of the design freedom that each proponent had when developing its candidate codec. The task was to provide a codec that offered the best possible overall quality in the GSM system across a set of relevant carrier-to-interference (C/I) ratios. The set of relevant C/I conditions was determined based on GSM system measurements and ranged from 4 dB, which was a challenging channel condition for the EFR (Enhanced Full Rate) reference codec, to 19 dB and ideal channel.

The outcome of the AMR/AMR-WB standardization efforts is well-known. It resulted in multi-mode/multi-rate speech codecs and a system allowing for fast bit rate adaptation in case of changing channel conditions.

- Teachings for ULBC case

Due to substantial differences in the target systems, the example of how the AMR/AMR-WB bit rates were found is not directly applicable. However, there are some teachings that suggest how ULBC bit rates can be determined. In the following differences and commonalities are discussed:

- AMR/AMR-WB standardization included the specification of channel coding. It involved the use of a radio simulator that included all relevant components of the GSM transmission channel including multipath/Rayleigh fading channel modelling. In the ULBC case, physical uplink and downlink channel coding (and modulation) procedures are fully specified. A specific Transport Format - implying the selection of a channel coding and modulation scheme - is chosen by correspondingly setting relevant parameters such as Bundling time, TBS, SCS, MCS and NRep. The presently envisioned Transport Formats allow a large range of PHY bit rates and corresponding source bit rates. This leaves design freedom for codec developers to find most suitable codec bit rate(s) that maximize(s) the quality for the relevant receive SNR conditions.

- The target system for AMR/AMR-WB was GSM (circuit-switched), while ULBC will be operated over IMS over NTN NB IoT. The main difference is that the transmission of ULBC frames will be packet-oriented, where some packetization overhead for the header information reduces the available source bit rate. The current assumption is that the overhead may be 7 bytes per packet.

- Another difference is that the AMR/AMR-WB decoder was designed to handle residual bit errors. In the ULBC case, it is assumed that the decoder only receives an indication if a frame is received without residual error or, if it is lost.

- AMR/AMR-WB was standardized together with a link adaptation scheme. The result was a multi-mode/multi-rate codec. In the ULBC case, the need and possibility of link adaptation has not yet been identified, but it should however be considered. This would imply that a solution could support multiple or scalable bit rates and that the selection of a specific operation bit rate could be based on a suitable adaptation technique either per fixed configuration, per calls or during a call.

- In the AMR/AMR-WB case, the codec was developed with the objective of offering the best possible performance over a range of relevant (average) C/I conditions after multipath reception. The corresponding objective in the ULBC case is to provide a codec that offers the best possible performance over a set of relevant receiver SNR conditions after fading based on NTN-TDL-C [3] channel modelling.

- In the AMR/AMR-WB case, the most suitable source bit rate(s) or any requirements to handle certain frame were upfront unknown. What was known were the relevant receive conditions in terms of a set of C/I ratios for which the overall voice call quality should be optimized. For ULBC, the situation is similar. The most suitable source bit rate is unknown and cannot be known since it depends on the specific properties such as intrinsic quality (without frame losses) and frame loss resiliency of each potential ULBC candidate. What is known is a set of relevant receive SNRs, for which the voice call quality should be maximized.

- In the standardization of AMR/AMR-WB, no specific target frame loss rates were defined for codec optimization. The reason is straightforward: for a given carrier-to-interference (C/I) ratio, the frame loss rate depends on the balance between error protection and the source bit rate. A solution with a higher source bit rate inherently has less room for error protection, resulting in a higher frame loss rate at the same C/I ratio compared to a solution with a lower source bit rate and stronger error protection.

Since the source bit rates were unspecified or unknown during standardization, it would not have been feasible to fairly compare candidate solutions using a fixed frame loss rate. Applying the same frame loss rate across different candidate solutions would have unfairly penalized those with lower source bit rates and better error protection.

The same logic applies to the ULBC case. Therefore, using a fixed target BLER (e.g., 2%) based on a notion of ‘TBS feasibility’ for all codec candidates is problematic. This approach would effectively force all candidate solutions to operate at the same source bit rate, which contradicts the fact that the optimal source bit rates depends on the specific codec designs that, to date, are unknown. As a result, such a method would likely lead to the choice of a suboptimal source bit rate and in consequence to suboptimal performance of a standardized solution.

- Proposal

To give ULBC proponents the design freedom to deliver the most optimized candidate codec proposals, the following steps are needed:

- Agree on a set of relevant operation points in terms of maximum achievable receive SNRs for which ULBC candidates will be evaluated. This set should cover the range from marginal operation to good and essentially error-free operation and include fading based on NTN-TDL-C [3] channel modelling. Care should be taken that this covers the full range of receive SNRs for which IMS Voice Call over GEO is expected to operate.

- Define performance requirements for each of the receive SNR operation points.

- For each of the potential bundling times out of the set of 80, 160 and 320 ms, agree on a set of potential source bit rates that candidate solutions could use. This set should be based on presently discussed transport formats in terms of the following transport format parameters TBS, SCS, MCS, NRep. 
Note: With the presently discussed transport format parameters, a range of potential source bit rates from 825 – 4650 bits/s is achieved. However, the granularity of possible bit rates in that range appears insufficient and unequal. The source addresses this point in a separate contribution [4].

- For each potential source bit rate, determine the optimum transport format parameter combination SCS, MCS, NRep that lead to best possible transmission performance at that bit rate. This could be done based on BLER vs SNR curves like those presented in [5].

- Produce packet loss patterns for each bundling time and each potential source bit rate for the set of relevant maximum achievable receive SNRs (after channel modelling). For a codec selection, it should be made sure that the specific loss patterns used in a comparative evaluation are unknown to any proponent.

- Compare ULBC candidates with each other on the basis of performance requirements for the relevant receive SNRs. Based on the choice of source bit rate and bundling time, a given candidate codec will be subjected to the corresponding packet loss patterns.

Moreover, although an ULBC codec selection should be primarily done for GEO NTN Nb IoT (as prioritized in the SID), it should be made sure that the candidate codecs are additionally tested for other conditions/use cases than NTN NB IoT where the BLER can assumed to be independent of the source bit rate, e.g., Terrestrial IMS (1% BLER target usually), OTT (ideal channel (0% BLER)) and, potentially, extreme conditions (e.g. 10% BLER or typical long losses due to blockage).

- Conclusion

It is proposed to agree the preceding paragraph and to document it in the TR.

References

[1] Tdoc S4/SMG11 (00)0087: Permanent project document WB-4: Design Constraints, v.1.0

[2] Tdoc S4/SMG11 (00)00321: AMR wideband performance requirements (WB-3) version 2.2

[3] 3GPP TR 38.811: Study on New Radio (NR) to support non-terrestrial networks

[4] Tdoc S4aA250058: [FS_ULBC] On source bit rate granularity

[5] Tdoc S4aA250058: Simulation results for FS_ULBC, Beijing Xiaomi Mobile Software