3GPPSA4-(AH) Audio SWG post 132	S4aA250081

Online, 11 July 2025															     Revision of S4aA250059

Source:	China Mobile Com. Corporation

Title:	On Assumptions and Open Issues for NB-IoT GEO Simulation

Spec:	3GPP TS 26.940

Agenda item:	1.9

Document for:	Agreement

1. Introduction

During recent discussions on the ULBC simulation work, several parameters were identified as lacking consensus. This contribution aims to document the assumptions and open issues raised during the Audio Ad-hoc meetings.

2. Proposed Updates

5.2.2	Simulation Model to derive error traces

5.2.2.x 	Assumptions and Open Issues for NB-IoT GEO Simulation

This clause documents the assumptions and open issues that require further investigation.

| Issues | Description | Status |
| --- | --- | --- |
| 1. UE Power Class | Whether to use the specified 23 dBm power class for NTN NB-IoT or the broader range of power (e.g., 26, 29, 31, and 33 dBm) supported by commercial UEs remains undecided, as no consensus has been reached. | Agreed: 26 dBm and 31 dBm Pending: 33 dBm (Requires RAN’s input and confirmation) |
| 2. Latitude-Dependent Loss | Scintillation loss should be set to 2.2 dB or 0 dB, depending on latitude. As specified in TR 38.821, the impact becomes negligible and shall be consider to zero for beyond the ±20° latitude. | Solved: The simulation accounts for latitude-dependent loss using the X term. |
| 3. Elevation Angles | A proposal has been made to keeping both 2.3° and 12.5° elevation angles in the simulation to consider the worst-case scenario for maintaining acceptable quality. | Solved: The simulation accounts for elevation angles using the X term. |
| 4. UL/DL Guard Time | An assumption of a 1 millisecond (ms) guard time for UL/DL switching is used in link budget analyses. The feasibility of this duration may need to be confirmed by RAN. | Pending: Needs to be confirmed by RAN. |
| 5. Determine feasible TBS values | Whether to adopt the other potential TBS values need further verification. - Xiaomi's proposal: S4aA250035 - Fraunhofer's: S4aA250031 - Skylo’s proposal: S4aA250045 - Dolby's proposal: S4aA250076 | Unsolved |
| 6. Overall Simulation Methodology Description | There is a recognized need to have a high-level description of how the simulation would be run, including parameters to optimize and result parameters. | Unsolved: To be addressed after all simulation work is completed. |
| 7. Simulation Channel Model | NTN-TDL-C or NTN-TDL-C5 | Unsolved |
| 8. Protocol Overhead | The proposal to include new solutions (e.g., NIDD) requires further discussion and confirmation by RAN2 and SA2. | Pending: Needs to be confirmed by RAN. |
| 9. Repetition numbers | It was proposed to specify and report the number of repetitions in the simulation. | Unsolved: To be addressed |

3. Proposal

It is proposed to include the content in Clause 2 of this document into the P-doc and to continue tracking the status of these issues.