3GPP Audio Telco Post TSG SA WG4 #133-e	S4aA250209

Erlangen, Germany, Sept 23-25, 2025

# Title:	On the RAN simulation methodologies

Source:	Qualcomm Innovation Center Inc.

Agenda item:	4.3

# Document for:	Agreement

# 1. Background

Several RAN simulation methodologies were proposed [2], [3], and [4] (which is included under clause 5.2.2.4.1 ‘open issues’ of [1]). This paper brings up the comments on [4].

[4] tries to reuse the methodology used in the standardization of AMR. However, that methodology was not used in TS26.132 [5].

In contrast, the proposals in [2] and [3] follow the methodology in TS26.132 [5]. The methodology in TS26.132 has been agreed in clause 5.2.1.3 of the PD:

Based on the functional description in Table E.1 of TS 26.132, the following input parameters are required to implement the simulation model:

BLER_tx / BLER_rx:

These parameters are required to simulate block error rates in both uplink and downlink.

NOTE: the resulted error trace based on Clause 5.2.2 will be used to serve as the BLER_tx/BLER_rx.

The proposal in [4] is not suitable for NB-IoT because the NB-IoT RAN is much more complicated than the RAN at the time of the AMR standardization. For example, for GSM, the amount of frequency domain bandwidth allocated to all UEs in the same cell are the same, while that could be as different as many as 48 times for NB-IoT, making the SNR not a suitable operation point. Additionally, the proposal in [4] mixes RAN simulation process and codec performance evaluation process, completely ignores system capacity, and misses important details.

# 2. Comments

The detailed comments on [4] are provided below in red.

##### 5.2.2.4.1	Methodology for Determining ULBC Bit Rate

Editor’s Note: The methodology described in this clause remains an open issue.

###### 5.2.2.4.1.1	Introduction

To give ULBC proponents the design freedom to deliver the most optimized candidate codec proposals, the following steps are needed:

-	Agree on a set of relevant operation points in terms of maximum achievable receive SNRs for which ULBC candidates will be evaluated. This set should cover the range from marginal operation to good and essentially error-free operation and include fading based on NTN-TDL-C [2] channel modelling. Care should be taken that this covers the full range of receive SNRs for which IMS Voice Call over GEO is expected to operate.

The performance is not just a function of receive SNRs. It also depends on the number of receive antennas (in the case of DL), the Doppler spread and the number of tones (for the UL). For example, fixing the UE TX power, the SNR for 3.75kHz SCS and a single tone is 6dB higher than that for 15kHz SCS and a single tone, and yet the two SNRs are equivalent in terms of UE’s performance if the transmission durations are the same and the SNRs are low.

-	Define performance requirements for each of the receive SNR operation points.

This is too vague. What are the performance requirements?

-	For each of the potential bundling times out of the set of 80, 160 and 320 ms, agree on a set of potential source bit rates that candidate solutions could use. This set should be based on presently discussed transport formats in terms of the following transport format parameters TBS, SCS, MCS, NRep.

This misses other important parameters, including number of receive antennas (in the case of DL), the number of tones, and parameters on the time-domain resource allocation including ISF, IRU, ITBS.
Note: With the presently discussed transport format parameters, a range of potential source bit rates from 825 – 4650 bits/s is achieved. However, the granularity of possible bit rates in that range appears insufficient and unequal. The source addresses this point in a separate contribution [3].

-	For each potential source bit rate, determine the optimum transport format parameter combination SCS, MCS, NRep that lead to best possible transmission performance at that bit rate. This could be done based on BLER vs SNR curves like those presented in [4].

What is the criterion for ‘optimum’ here? Without being spelled out, this is meaningless for the RAN simulation.

Also, we should refrain from talking about potential source bit rates in the RAN simulation, because as agreed in the WID, they are to be derived later from GEO channel characteristics, as stated in objective #2 from the WID:

2.	Study GEO channel characteristics and derive service-related dependencies, e.g. bitrates, mouth-to-ear delay or loss/delay/jitter profiles.

-	Produce packet loss patterns for each bundling time and each potential source bit rate for the set of relevant maximum achievable receive SNRs (after channel modelling). For a codec selection, it should be made sure that the specific loss patterns used in a comparative evaluation are unknown to any proponent.

We should separate RAN simulation and codec selection. For codec selection, a separate paper needs to be submitted for discussion.

-	Compare ULBC candidates with each other on the basis of performance requirements for the relevant receive SNRs. Based on the choice of source bit rate and bundling time, a given candidate codec will be subjected to the corresponding packet loss patterns.

Codec selection should be deferred after the RAN simulation.

NOTE:	Although the ULBC codec selection should primarily be done for GEO NTN Nb IoT (as prioritized in the SID), it may be beneficial to ensure that the candidate codecs are additionally tested for other conditions/use cases than NTN NB IoT where the BLER can assumed to be independent of the source bit rate, e.g., Terrestrial IMS (1% BLER target usually), OTT (ideal channel (0% BLER)) and, potentially, extreme conditions (e.g. 10% BLER or typical long losses due to blockage).

###### 5.2.2.4.1.2	Example

The following example illustrates how a ULBC proponent could find the best possible bitrate for its solution such that the voice quality is optimized.

Assume that the proponent has a design that allows operation at two net (codec) bitrates, at 0.95 kbps and 3.4 kbps. The proponent seeks to determine whether the low or the high bitrate option would lead to better voice quality.

Additionally, assume that the study is done for UL and that the bundling time is 160 ms. In that case, and further assuming that the IP overhead (after header compression) is 7 bytes, a TBS of 208 bits would be chosen for the low-rate option, while a TBS of 600 bits would be chosen for the high-rate option.

Furthermore, the UE tx power assumptions and link budget assumptions according to Tdoc S4aA250058 [4] are taken, namely, leading to the following receive CNR/SNR values based on the link budget analysis of this document:

Table 5-1 [5]: CNR results under different parameter assumptions

|  | SCS/BW | UE power | UL CNR |
| --- | --- | --- | --- |
| Config. 1 | 3.75kHz/3.75kHz | 23dBm | 8.66 dB |
| Config. 2 | 15kHz/15kHz | 23dBm | 2.64 dB |
| Config. 3 | 3.75kHz/3.75kHz | 26dBm | 11.66 dB |
| Config. 4 | 15kHz/15kHz | 26dBm | 5.64 dB |
| Config. 5 | 3.75kHz/3.75kHz | 29dBm | 14.66dB |
| Config. 6 | 15kHz/15kHz | 29dBm | 8.64dB |
| Config. 7 | 3.75kHz/3.75kHz | 31dBm | 16.66dB |
| Config. 8 | 15kHz/15kHz | 31dBm | 10.64dB |

What about DL? What about system capacity?

To facilitate a comparison of the achievable voice quality with the two ULBC bitrate options, BLER patterns are generated using NTN-TDL-C [2] channel modelling in the following steps.

Initially, among the available transport format parameter configurations, the one that offers best transmission performance is chosen.

For TBS = 208 bits, the choice is made among the configurations shown in the following table (with tx-time =128 ms) :

Table 5-2: Possible transport format configurations for TBS=208 bits and 160 ms bundling

| Configuration | TBS | SCS | MCS | Number of RU | Repetition number | PHY bitrate(kbps) |
| --- | --- | --- | --- | --- | --- | --- |
| #1 | 208 | 15 | 0 | 8 | 2 | 1.3 |
| #2 | 208 | 15 | 3 | 4 | 4 | 1.3 |
| #3 | 208 | 15 | 12 | 1 | 16 | 1.3 |
| #4 | 208 | 3.75 | 3 | 4 | 1 | 1.3 |
| #5 | 208 | 3.75 | 12 | 1 | 4 | 1.3 |

For TBS = 600 bits, the choice is made among the configurations shown in the following table (with tx-time =128 ms):

Table 5-3: Possible transport format configurations for TBS=600 bits and 160 ms bundling

| Configuration | TBS | SCS | MCS | Number of RU | Repetition number | PHY bitrate(kbps) |
| --- | --- | --- | --- | --- | --- | --- |
| #1 | 600 | 15 | 15 | 2 | 8 | 3.75 |
| #2 | 600 | 3.75 | 15 | 2 | 2 | 3.75 |

Subsequently, for the selected configurations that are found most suitable, generate the following BLER patterns using an NTN-TDL-C [3] channel simulator:

Table 5-4: BLER patterns to be generated for low and high rate for 4 relevant UE tx power settings

| UE power | Low rate (TBS=208 bits) | High rate (600 bits) |
| --- | --- | --- |
| 23 dBm | ep-ul-208-23dBm | ep-ul-600-23dBm |
| 26 dBm | ep-ul-208-26dBm | ep-ul-600-26dBm |
| 29 dBm | ep-ul-208-29dBm | ep-ul-600-29dBm |
| 31 dBm | ep-ul-208-31dBm | ep-ul-600-31dBm |

This would have an issue: the range of BLERs will be very limited for each bit rate. Specifically, for the low bitrate, the BLER for different UE power levels will be concentrated at the lower end, e.g., 0.1% - 2%, while for higher bitrate the BLER will be concentrated at the higher end, e.g., 5% - 20%. However, for robustness testing, a wider range of BLER is preferred for both lower bit rates and higher bit rates.

Next, prepare proponent codec simulation such that it can be operated with inserted packet loss using the BLER patterns generated in the previous step.

Proceed to run codec simulation using these patterns, encoding suitable test speech material. The input speech data is assumed to be called ‘inp_speech.wav’. The simulation generates output speech data files for the different applied BLER patterns. The following output files are generated:

Table 5-5: Output speech files for low and high rate options for the different UE tx power settings

| Low rate (TBS=208 bits) | High rate (600 bits) |
| --- | --- |
| out_speech-ul-208-23dBm.wav | out_speech-ul-600-23dBm.wav |
| out_speech-ul-208-26dBm.wav | out_speech-ul-600-26dBm.wav |
| out_speech-ul-208-29dBm.wav | out_speech-ul-600-29dBm.wav |
| out_speech-ul-208-31dBm.wav | out_speech-ul-600-31dBm.wav |

Lastly, evaluate the quality of the output speech, e.g. in a listening test. The quality evaluation will have to consider that the four tx power settings may result in different respective quality levels. To get to an overall voice quality judgement, it will be needed to make some (weighted) averaging of the obtained opinion scores.

NOTE:	It is important to note that the quality is expected to increase with higher UE tx power since fewer packet losses are expected to occur. Moreover, it may be expected that the high rate codec option may show relatively degraded quality especially for low UE tx power. On the other hand, the low rate option of the codec may be limited due to low bit rate and that bitrate option may not benefit from higher UE tx power conditions since the transmission for low UE tx power may already be essentially error-free. The evaluation would provide deeper insight and let the proponent choose the best option overall.

Furthermore, note that speech files of a reference system to be used as performance requirements could be generated in a very similar fashion. Assuming that such a reference codec would operate at e.g. 1.5 kbps, the task would be to identify the TBS and transport format configuration that would be most suitable to transport payloads of this reference codec over the GEO NTN NB-IoT channel, generate the corresponding BLER patterns for the relevant UE tx power settings and then generate the respective output speech files to be used as performance requirement.

# 3. Proposals

Proposal 1: New RAN simulation methodologies shall be consistent with the methodology documented in clause 5.2.1.3 of the PD.

Because Clause 5.2.2.4.1 of the PD contradicts with the currently agreed RAN simulation methodology in clause 5.2.1.3 of the PD, is not suitable for NB-IoT, and is not aligned with objective #2 in the WID, we propose:

Proposal 2: Remove Clause 5.2.2.4.1 from the PD.

# References

[1] S4-251585, Permanent Document FS_ULBC, 3GPP TSG SA WG4 #133-e, July 18-25, 2025.

[2] S4aA250072, "Potential approaches to determining feasible TBS values and generating loss traces," July 11, 2025.

[3] S4-251332, Updates on end to end simulation for error trace generation, vivo, 3GPP TSG SA WG4 #133-e, July 18-25, 2025.

[4] S4-251388, On ULBC bit rates, Dolby, et al, 3GPP TSG SA WG4 #133-e, July 18-25, 2025.

[5] TS 26.132, “Speech and video telephony terminal acoustic test”, v18.2.0, 2024.