SA4-Audio SWG AH	S4aA250238

Erlangen, 23 – 25 September 2025

Source:	Fraunhofer IIS

Title:	On testing methodologies for ULBC

Spec:	3GPP TR 26.940

Agenda item:	4.7

Document for:	Discussion

1. Introduction

In Tdoc S4-251395, it was proposed to allow a noise suppression functionality as part of the ULBC coding algorithm. Several questions were raised about the impact on the testing methodologies. This document aims to address these comments and derive proposals accordingly. Especially, the comparison of codec with and without noise suppression capability is discussed.

2. Test categories

Mainly two test categories need to be distinguished considering the impacts of a noise suppression module for codec testing.

- Clean speech

- Noisy speech

Those have been used in the past, e.g. for the EVS standardization where clean speech was tested with either P.800 ACR or DCR and noisy speech was tested with P.800 DCR.

2.A. Clean speech tests

For clean speech tests, the impact of a noise suppression module is rather limited. When comparing codecs with and without noise suppression, only the effect of noise gating needs to be addressed. Noise gating means the recoding noise level may be removed which leads to more pleasant soundings and give higher scores in a P.800 test. This can easily be addressed by requiring a minimum recording noise level of, e.g. -80 dB (SNR) or preprocess the P.800 input material by an open-source noise suppression modules, e.g. DeepFilterNet2. With this prerequisite, codecs with and without noise suppression modules can be compared using P.800 ACR and DCR methodology.

2.B. Noisy speech tests

In the past, e.g. in the Test Plans Selection of EVS, the P.800 DCR methodology was used where the reference condition also contains the noisy signal. Typical noise types such as car noise, office and street have been used with SNR levels between 15 dB and 25 dB.

Including a noise suppressor in the tests leads to the effect that subjects prefer the noise suppressed version and consequently the coded version should get a better score than the reference. However, according to the degradation scale of DCR, noise suppression is seen as an artefact by the subject. The DCR method in this version is therefore not suitable. In the following several approaches to overcome this issue are outlined.

In all proposals, it is up to the ULBC candidates to support noise suppression and any disadvantage of reference codecs without noise suppression may be compensated by a higher bitrate.

2.B.1 Noisy speech using P.835

In P.835, the subjects rate in the three categories speech signal distortion (DCR scale), background noise attenuation (background noise rating scale) and overall quality (ACR scale).

Due to the separated focus of the test, the distortion of the speech signal itself can be evaluated independently which is of most importance for the ULBC codec. A combination of the speech signal distortion and the overall quality may be considered, while the background noise rating can be ignored. The testing procedure implies a significant higher effort as three grades per condition need to be assessed.

2.B.2 Noisy speech using P.800 CCR

The P.800 Comparison Category Rating (CCR) allows a rating better than a reference. This means the effect of noise suppression and coded signal quality can be combined into one grade. A direct comparison to the original signal is given and the testing effort is comparable to a P.800 DCR test.

2.B.3 Noisy speech using P.800 ACR

As the P.800 ACR method provides an absolute category rating without comparing to the reference. It should be noted that the direct condition may not get the highest score as conditions with noise suppression enabled might be preferred quality wise.

It should be noted that P.800 ACR was widely used in past codec standardization exercises and provides the most efficient method for testing allowing the highest number of conditions in one test.

3. Testing requirements

TR 26.940 V0.2 (S4-251152) mentions that “hallucination, for example, is a category that plays only a role in ML-based coding systems but not for signal-processing based codecs, which AMR, AMR-WB and EVS are”.

To test that the content of the coded signal is preserved at least one category should be tested with a direct comparison to the reference signal. To provide a good balance between testing effort and quality assurance, it is therefore proposed to use the P.800 DCR for clean speech tests and P.800 ACR for noisy speech tests.

Alternatively, if noisy speech is considered as more challenging the check on the correct content with a comparison to the original signal can be done within the noisy speech tests. For maintaining the testing effort, P.800 ACR can be used for clean speech and P.800 CCR for noisy speech.

To ensure subjects verify the content of the speech signal, a clarifying statement should be added to the instructions.

4. Conclusion

The present document proposes solutions for testing methodologies for codecs with and without noise suppression. The source propose to agree on the proposed solutions as working assumption and invites cross checks of the methods to finally conclude at the next SA4 meeting in Dallas.