3GPP Audio Telco Post TSG SA WG4 #133-e	S4aA250259

Erlangen, Germany, Sept 23-25, 2025                                           Revision of S4aA250211

# Title:	Contribution process for ULBC RAN simulation

Source:	Qualcomm Innovation Center Inc., vivo, Xiaomi, Huawei Technologies Co. Ltd.

Agenda item:	4.3

# Document for:	Agreement

# 1. Background

It has been agreed in the PD [1] that the simulation results need to maintain channel consistency among different combinations of simulation parameters.

The objective is to generate multiple loss traces for a combination of frame loss rate (target BLER), raw bitrate (TBS), voice bundling period and Doppler spread, while maintaining channel consistency among different combinations.

As of meeting SA4 #133-e, only Qualcomm verified channel consistency [2] in the simulation.

It has been agreed that companies need to validate BLER vs SNR performance before generating the loss traces. Four companies submitted simulation results as of meeting SA4 #133-e. The calibration outcome is done in [3] for scenarios in [4], and is summarized below:

- For NPUSCH

  - Scenario 1.1, Qualcomm and Hauwei’s results are close (within a 0.6dB difference at 1% BLER) and can be considered aligned, while the other two companies’ results diverge from the aligned result significantly.

  - Scenario 1.2, only Qualcomm and vivo provided results, and they differ by 2.8dB at 1% BLER.

- For NPDSCH

  - Scenario 2.1.1, Qualcomm, vivo, Huawei’s results are close (within a 0.3dB different at 1% BLER) can be considered aligned.

    - Qualcomm [2] further showed that with optimized use of DMRS in channel estimation, 1 dB gain can be achieved.

  - Scenario 2.1.2, only Qualcomm provided results.

  - Scenario 2.2.1, only Qualcomm and vivo provided results, and the difference is about 1dB at 1% BLER.

  - Scenario 2.2.2, only Qualcomm provided results.

# 2. Proposed contribution process

To ensure the technical correctness of the contributed loss traces while keeping the simulation workload manageable for individual companies, the following contribution process is proposed:

- Verification

For the simulation results to be considered (and the loss traces are admitted for further cross-check), the following results shall be provided and be comparable to other existing results:

- BLER vs SNR, i.e., required SNR to meet BLER 1%, 2%, 6% and 10%

- Channel consistency (as defined in [4])

- Sharing simulation workload

The scenarios are divided among the qualified companies. Each scenario (a combination of simulation parameters) is simulated by at least two companies which will then cross-check the loss traces. As an example, if four companies are qualified, then each scenario is simulated by two companies which will cross-check the loss traces between them, and the simulation workload is halved for each company.

# 3. Proposal

Proposal: Agree the contribution process in section 2.

# References

[1] S4-251585, Permanent Document FS_ULBC, 3GPP TSG SA WG4 #133-e, July 18-25, 2025.

[2] S4-251397, “Results for RAN simulation calibration for ULBC over NTN,” Qualcomm, 3GPP TSG SA WG4 #133-e, July 18-25, 2025.

[3] S4-251458 rev 2, “Calibration of RAN simulation results for ULBC over NTN”, 3GPP SA WG4 #133-e, July 18-25, 2025.

[4] S4-251303, “Scenarios for RAN simulation calibration for ULBC over NTN,” 3GPP SA WG4 #133-e, July 18-25, 2025.