SA4 (AH) Audio SWG post 133-e				                             S4aA250268

Revision of S4aA250248

F2F Erlangen, 23 – 25 Sept 2025

Source:	Dolby Laboratories Inc., Ericsson LM, Nokia, Novamint

Title:	[FS_ULBC] Packet Loss Concealment with existing AI based codec DAC

Agenda item:	4

Document for:	Discussion and Agreement

# Background

One of the objectives of FS_ULBC SID [1] is to do feasibility study of existing technologies as done in [2]. Furthermore, the study on bitrate and BLER is ongoing and various contributions have provided initial link level simulations [3], [4] and [5]. These results are still preliminary with respect to the assumptions made for the simulations, but as shown in these contributions, at a given SNR, the BLER percentage typically increases with the increase in TBS values. The ULBC Pdoc currently mentions BLER of 1, 2, 6 and 10% and the ULBC TBS study has been done so far with these BLER values.

The document [2] highlights that an AI based codec solution is better suited for the envisioned ULBC bitrate constraints. In this document, to further progress the study on codec bitrates and existing technologies, the source provides data on loss resiliency with an existing AI based codec DAC [6] at different bitrates and associated BLERs.

# Discussion

  - PLC experiment with DAC

DAC codec is trained as a scalable codec in terms of bitrate and the models are available for 16 khz, 24 khz and 44.1 khz input sample rates [6]. The sources took the default model available at 16 khz sampling rate and performed an experiment with 4 different bitrates by introducing 4 different loss percentages at the input to decoder. The bitrates and corresponding loss percentages are mentioned in Table 2.1-1.

| Bitrate (kbps) | Loss percentage (%) | Frame size considered (ms) |
| --- | --- | --- |
| 1 | 1 | 80 |
| 2.5 | 6 | 80 |
| 4.5 | 10 | 80 |
| 6 | 20 | 80 |

The bitrates and loss percentages (upto 4.5 kbps) were chosen based on the data shared in [3]. For this experiment, we took an operating point at around 3 dB CNR for SCS=15kHz and 9dB for SCS=3.75 kHz (UE Power = 26 dBm). The selected configuration for TBS=144 is case 3, 4 or 5 in Table 4 of [3] which would achieve some BLER of 1% or even less. The selected configuration for TBS=256 is case 11 in Table 6 of [3] yielding a BLER of 6%. The selected configuration for TBS=424 is case 45 in Table 10 of [3] with worst case BLER of 10% (With X=0 in (3GPP SET-1 UL SNR) – 10*log10(B/3.75) + (P - 23dBm) + G + [X] dB, the TBS=424 case does not strictly fit into the CNR values and X > 0 is assumed). The values considered here can also be achieved with UL CNR = 5.06 dB (UE Power = 31 dBm) in case 3, case 13/14 and case 45 of [3].

Note that the TBS to be transmitted and the achievable data rates will depend on the assumptions made for the link budget analysis and will be impacted e.g. by the scheduling schemes [8].

The DAC codec follows an autoencoder architecture wherein the input is down sampled into a latent representation which is then quantized using a residual vector quantizer (RVQ). At 16 khz sampling rate, with the default configurations of DAC, the input is padded and down sampled by a factor 320 (corresponds to 20ms). So, a padded input of X samples is converted into X/320 blocks of 1024 embeddings. Each block is then quantized using N number of codebooks (wherein each codebook results in 10 bits or 0.5 kbps rate) of the RVQ. The DAC model is trained as a scalable codec in a way that number of codebooks can be changed during inferencing to achieve different bitrates (quantizer dropouts technique during training as used in DAC helps with achieving a scalable solution here).

  - Introducing loss in quantized blocks

Packet loss was introduced in two different ways as shown below:

### Consecutive 4 blocks drop and repeat

As mentioned above, each block after the DAC 16 khz encoder represents 20 ms of data. Assuming a packet size of 80ms, each packet loss would result in a loss of 4 blocks. In this step, 4 consecutive blocks were dropped and replaced by N-1 block. For example, if the input is X samples then that gets converted into X/320 blocks after encoder and to simulate 2 % packet loss, Y = (X*0.02)/320 blocks are dropped by randomly choosing Y/4 indices and dropping 4 consecutive blocks. If index 5 is chosen then blocks 5, 6, 7, 8 are dropped and replaced by block 4.

### Interleaved drop and repeat

With this method, two 80 ms packets were concatenated and each loss is spread over 2 packet frames in an interleaved fashion. As shown below, y is quantized 20 ms block and x is lost block that is replaced by previous y block in order to simulate packet loss.

|y y y y|x x x x|.           ------- >.   |y x y x|y x y x|

This method avoids consecutive drops and may potentially help improve the performance of the codec. However, it comes at a cost of extra added latency.

- Results

A MUSHRA test was conducted with 8 listeners and 10 conditions described as follows:

- Uncoded reference

- 3.5 khz low pass filtered anchor

- 1 kbps, 1 % loss as per clause 2.2.1

- 1 kbps, 1 % loss as per clause 2.2.2

- 2.5 kbps, 6 % loss as per clause 2.2.1

- 2.5 kbps, 6 % loss as per clause 2.2.2

- 4.5 kbps, 10 % loss as per clause 2.2.1

- 4.5 kbps, 10 % loss as per clause 2.2.2

- 6 kbps, 20 % loss as per clause 2.2.1

- 6 kbps, 20 % loss as per clause 2.2.2

The test items were 80% clean speech items and 20 % noisy speech items

Fig 3.1: MUSHRA results for all items

Fig 3.2: MUSHRA results (mean result of all items)

  - Observations

- Despite of the higher loss percentage, both 4.5 kbps and 6 kbps variants show significant improvement over 1 kbps and 2.5 kbps

- 6 kbps with 20 % error rate was rated very close to 4.5 kbps with 10% error rate.

- As the error rate increases, the benefit with interleaving approach as described in clause 2.2.2 becomes more evident.

- Given that the experiment was done without modifying DAC model, there is a potential to further improve the resiliency of AI based solutions if the model is trained with random loss patterns. Although this needs to be verified.

  - Conclusion

- Above observations raises the question whether bitrate should be limited to 4.6 kbps as mentioned in [3] OR more design freedom should be allowed in terms of bitrate and BLER to achieve best possible quality at a given SNR.

- Limiting the BLER for “TBS feasibility” does not seem justified and may limit the overall performance of the codec solution and may potentially exclude solutions that have higher loss resiliency.

- The above experiment demonstrates a way to compare different solutions that are operating at different bitrates and corresponding BLERs.

- Proposal

In this contribution the source performs an experiment by simulating packet loss at various bitrates using an existing codec DAC. Furthermore, the source conducted subjective evaluation with different rates and associated BLERs and shared the observations.

It is proposed to document clause 2 and 3 to ULBC Pdoc. It is further proposed to agree to the following changes to 3GPP TR 26.940 [7].

* * * First Change * * *

### 7.1.5	PLC with existing technology

7.1.5.1	PLC experiment with DAC

DAC codec is trained as a scalable codec in terms of bitrate and the models are available for 16 khz, 24 khz and 44.1 khz input sample rates [6]. The sources took the default model available at 16 khz sampling rate and performed an experiment with 4 different bitrates by introducing 4 different loss percentages at the input to decoder. The bitrates and corresponding loss percentages are mentioned in Table 2.1-1.

| Bitrate (kbps) | Loss percentage (%) | Frame size considered (ms) |
| --- | --- | --- |
| 1 | 1 | 80 |
| 2.5 | 6 | 80 |
| 4.5 | 10 | 80 |
| 6 | 20 | 80 |

The bitrates and loss percentages (upto 4.5 kbps) were chosen based on the data shared in [3]. For this experiment, we took an operating point at around 3 dB CNR for SCS=15kHz and 9dB for SCS=3.75 kHz (UE Power = 26 dBm). The selected configuration for TBS=144 is case 3, 4 or 5 in Table 4 of [3] which would achieve some BLER of 1% or even less. The selected configuration for TBS=256 is case 11 in Table 6 of [3] yielding a BLER of 6%. The selected configuration for TBS=424 is case 45 in Table 10 of [3] with worst case BLER of 10% (With X=0 in (3GPP SET-1 UL SNR) – 10*log10(B/3.75) + (P - 23dBm) + G + [X] dB, the TBS=424 case does not strictly fit into the CNR values and X > 0 is assumed). The values considered here can also be achieved with UL CNR = 5.06 dB (UE Power = 31 dBm) in case 3, case 13/14 and case 45 of [3].

Note that the TBS to be transmitted and the achievable data rates will depend on the assumptions made for the link budget analysis and will be impacted e.g. by the scheduling schemes [8].

The DAC codec follows an autoencoder architecture wherein the input is down sampled into a latent representation which is then quantized using a residual vector quantizer (RVQ). At 16 khz sampling rate, with the default configurations of DAC, the input is padded and down sampled by a factor 320 (corresponds to 20ms). So, a padded input of X samples is converted into X/320 blocks of 1024 embeddings. Each block is then quantized using N number of codebooks (wherein each codebook results in 10 bits or 0.5 kbps rate) of the RVQ. The DAC model is trained as a scalable codec in a way that number of codebooks can be changed during inferencing to achieve different bitrates (quantizer dropouts technique during training as used in DAC helps with achieving a scalable solution here).

        - Introducing loss in quantized blocks

Packet loss was introduced in two different ways as shown below:

### Consecutive 4 blocks drop and repeat

As mentioned above, each block after the DAC 16 khz encoder represents 20 ms of data. Assuming a packet size of 80ms, each packet loss would result in a loss of 4 blocks. In this step, 4 consecutive blocks were dropped and replaced by N-1 block. For example, if the input is X samples then that gets converted into X/320 blocks after encoder and to simulate 2 % packet loss, Y = (X*0.02)/320 blocks are dropped by randomly choosing Y/4 indices and dropping 4 consecutive blocks. If index 5 is chosen then blocks 5, 6, 7, 8 are dropped and replaced by block 4.

### Interleaved drop and repeat

With this method, two 80 ms packets were concatenated and each loss is spread over 2 packet frames in an interleaved fashion. As shown below, y is quantized 20 ms block and x is lost block that is replaced by previous y block in order to simulate packet loss.

|y y y y|x x x x|.           ------- >.   |y x y x|y x y x|

This method avoids consecutive drops and may potentially help improve the performance of the codec. However, it comes at a cost of extra added latency.

        - Results

A MUSHRA test was conducted with 8 listeners and 10 conditions described as follows:

- Uncoded reference

- 3.5 khz low pass filtered anchor

- 1 kbps, 1 % loss as per clause 2.2.1

- 1 kbps, 1 % loss as per clause 2.2.2

- 2.5 kbps, 6 % loss as per clause 2.2.1

- 2.5 kbps, 6 % loss as per clause 2.2.2

- 4.5 kbps, 10 % loss as per clause 2.2.1

- 4.5 kbps, 10 % loss as per clause 2.2.2

- 6 kbps, 20 % loss as per clause 2.2.1

- 6 kbps, 20 % loss as per clause 2.2.2

The test items were 80% clean speech items and 20 % noisy speech items

Fig 3.1: MUSHRA results for all items

Fig 3.2: MUSHRA results (mean result of all items)

        - Observations

- Despite of the higher loss percentage, both 4.5 kbps and 6 kbps variants show significant improvement over 1 kbps and 2.5 kbps

- 6 kbps with 20 % error rate was rated very close to 4.5 kbps with 10% error rate.

- As the error rate increases, the benefit with interleaving approach as described in clause 2.2.2 becomes more evident.

- Given that the experiment was done without modifying DAC model, there is a potential to further improve the resiliency of AI based solutions if the model is trained with random loss patterns. Although this needs to be verified.

### 7.1.6	Conclusion on existing technology

The following conclusions on existing technologies can be drawn based on this sub-clause 7.1:

-	AI based solution may have the potential to provide substantial better audio quality compared to conventional solutions

-	Whether AI based codecs fit into complexity constraints of current chipset is FFS.

Increasing the frame duration to larger blocks than the standard 20 ms VoIP size seems to be beneficial for ultra-low bitrate coding.

-	None of the presented solutions can be considered as a complete candidate for ULBC.

-	Besides an optimal match on the basic parameters such as frame length or bitrate, the ULBC candidate needs to be implementable on mobile devices. Furthermore, the candidate needs to support required system aspects such as channel resilience capability or discontinuous transmission, which is currently not supported by any AI-based solution.

-        The observations made in clause 7.1.5.1.3. suggests that more design freedom should be allowed in terms of bitrate and BLER to achieve best possible quality at a given SNR.

* * * End of Changes * * *

References

[1] SP-250378 “New SID on Ultra Low Bitrate Speech Codec”

[2] Tdoc S4-250923, “Analysis of existing technologies for ULBC”

[3] Tdoc S4-251332, “[FS_ULBC] Updates on end-to-end simulation for error trace generation”

[4] S4-251251, “Uplink simulation results for RAN simulation calibration for ULBC over NTN,” Xiaomi, 3GPP TSG SA WG4 #133-e, July 18-25, 2025.

[5] S4-251397, “Results for RAN simulation calibration for ULBC over NTN,” Qualcomm, 3GPP TSG SA WG4 #133-e, July 18-25, 2025.

[6] https://github.com/descriptinc/descript-audio-codec

[7] TR 26.940, "Study on Ultra Low Bitrate Speech Codecs".

[8] Tdoc S4aA250232, “On assumptions for link budget analysis”