Source:	Ericsson LM, Dolby Laboratories Inc.

Title:	On assumptions for link budget analysis

Document for:	Discussion and Agreement

Agenda Item:	4.3

# 1	Introduction

As part of FS_ULBC, there is an objective to study channel characteristics and service-related dependencies, determining e.g. bitrates and loss/delay/jitter profiles. Based on [2] and discussions during Audio SWG AH on FS_ULBC, it is proposed to update [1] as follows.

# 2	Proposal

It is proposed to add the following text into the Pdoc [1], below Figure 5.2.2.3-2.

Figure 5.2.2.3-3 shows a scheme based on “Cell_specific_Koffset” approach, which does not depend on the “TA report UE capability”.

Figure 5.2.2.3-3 SPS scheme based on “Cell_specific_Koffset” approach, which does not depend on the “TA report UE capability”.

Notes: The gap between DL and UL can consist of:

- A “Processing time + DL-to-UL switching”: It can be discussed whether the time for decoding the DL transport block needs to be considered or not, but at least the time that a “half-duplex device” requires for switching from DL-to-UL shall be considered which is 1 ms.

- The “Max differential delay” shall be considered for the network to handle the different delays of different UEs in the NTN cell. The value of “Max differential delay” will vary and will typically range between [close to 0 and 10.3 ms].

Editor’s note: The range of the “Max differential delay” is TBC.

Note: RAN1 reply LS stated:

- “Although the example Figure 5.2.2.3-1 is supportable by RAN1 specifications in most scenarios, it may not be supportable in the case where the cell is very large (e.g. >3000km), when the UE does not support TA report and the network does not support UE-specific K-offset. The example Figure 5.2.2.3-1 itself also requires the UE to be configured with two HARQ processes and with HARQ feedback disabled.”

- RAN1/2 have not yet started the work on designing SPS. Therefore, RAN1 currently cannot confirm whether the example frame structure for SPS (related to Figure 5.2.2.3-2 and associated text) will be supported.

# 3	References

Tdoc S4-251585: Pdoc FS_ULBC, v.0.2.0

Tdoc S4aA250232: On assumptions for link budget analysis