3GPP TSG-SA4-e (AH) Audio SWG post 134-2	S4aA260005

# 15 January 2026, Online

Source:	Dolby Laboratories Inc., Nokia, Novamint

Title:	[FS_ULBC] On ULBC complexity and RTF analysis

Agenda item:	1.7

Document for:	DISCUSSION and AGREEMENT

# Background

As per the time plan of FS_ULBC [1], one of the objectives for SA4 #134 meeting is to make progress with complexity and memory design constraints. During SA4 #133-e meeting and ULBC ADHOC meeting in Erlangen, Germany, multiple proposals on complexity were discussed [2], [3], [4] and [5]. In [2], the sources provided details on potential theoretical metrics that can be used to set complexity design constraints while in [3] and [4], the sources provided real time factor (RTF) performance of DAC [6] and Lyra v2 [7]. In [5], the source provided information on challenges that may arise when running big models on smartphones (for e.g. challenges with respect to DRAM access).

With the various existing AI based codecs studied so far, there is some data available on real time analysis, but it is not enough to draw conclusions on complexity constraints for ULBC. For example, Lyra v2 RTF analysis is done in [7], HIL codec (9.58 M params) RTF analysis is done in [8] and DAC (with approximately 70 M params) RTF analysis is done in [3].

In order to define complexity metric for an AI based solution for ULBC, more data is required on the real time analysis of various models (architectures that are relevant to AI based audio coding) with different sizes so that an informed decision can be made on the complexity requirements  for a solution that works in real time on today’s smartphones.

In this contribution, the source provides RTF values with various AI based codec models (sizes between 15 M to 70 M), running on CPU on a sample of recent smartphones, to get better idea on how such models would perform.

# Discussion

## Experiment 1

The original ~70M DAC model size was modified by reducing the number of params in each encoder and decoder block. The models were re-trained specifically for around 1 kbps bitrate at 32 kHz sampling rate. With this, the source generated models of size 43 M params and 20 M params with float32 precision without changing the general DAC architecture. The latent dimension for 43 and 20 M model was configured to 640 (encoder rates = 4, 4, 8, 10 and decoder rates = 10, 8, 4, 4). These modified pytorch models were then converted to ONNX [9] and referenced using ONNX runtime library along with XNNPACK execution provider on a set of smartphones.

Each ONNX model (encoder+decoder) was inferenced 5 times using a 5 secs audio (entire 5 sec audio processed in one shot) on a given smartphone CPU using either 1 or 4 threads. Then the RTF value was computed for each run by dividing the duration of the audio signal by the time it took to encode and decode the given audio signal. Table 2.1 provides the min and max RTF values with smartphone device 1 [12], device 2 [13] and device 3 [14]. Table 2.1 also includes RTF analysis of unmodified DAC models at 44.1 kHz and 24 kHz sampling rate.

|  | Specification | Model size | GMACS |  | 1 thread | 4 threads |
| --- | --- | --- | --- | --- | --- | --- |
| Device 1 | Release year: 2023 CPU: Hexa-core (2x3.46 GHz + 4x2.02 GHz) | ~ 70 M (44.1 kHz sample rate) | 70 | Min RTF | 0.375 | 0.62 |
|  |  |  |  | Max RTF | 0.39 | 0.63 |
|  |  | ~ 70 M (24 kHz sample rate) | 42.6 | Min RTF | 0.64 | 1.05 |
|  |  |  |  | Max RTF | 0.67 | 1.05 |
|  |  | ~ 43 M (32 kHz sample rate) | 11.8 | Min RTF | 2.27 | 2.80 |
|  |  |  |  | Max RTF | 2.27 | 2.85 |
|  |  | ~ 20 M (32 khz sample rate) | 5.1 | Min RTF | 4.16 | 4.88 |
|  |  |  |  | Max RTF | 4.16 | 5 |
| Device 2 | Release year: 2023 CPU: Nona-core (1x3.0 GHz Cortex-X3 & 4x2.45 GHz Cortex-A715 & 4x2.15 GHz Cortex-A510) | ~ 43 M (32 khz sample rate) | 11.8 | Min RTF | 1.17 | 1.98 |
|  |  |  |  | Max RTF | 1.24 | 2.16 |
|  |  | ~ 20 M (32 khz sample rate) | 5.1 | Min RTF | 2.32 | 3.7 |
|  |  |  |  | Max RTF | 2.6 | 3.9 |
| Device 3 | Release year: 2022 CPU: Octa-core (1x3.00 GHz Cortex-X2 & 3x2.50 GHz Cortex-A710 & 4x1.80 GHz Cortex-A510) | ~ 43 M (32 khz sample rate) | 11.8 | Min RTF | 1.40 | 2.26 |
|  |  |  |  | Max RTF | 1.41 | 2.32 |
|  |  | ~ 20 M (32 khz sample rate) | 5.1 | Min RTF | 2.6 | 3.99 |
|  |  |  |  | Max RTF | 2.6 | 4.0 |

Table 2.1. RTF results with experiment 1

## Experiment 2

In this experiment, the model inferencing is done on frame-by-frame basis to simulate a use case that is closer to a voice communication use case. In a single experiment, each ONNX model (encoder+decoder) was inferenced with 80 ms frame size over a period of 2 minutes (total 1500 inferences in the same session). This experiment was repeated 5 times and min RTF, and max RTF were computed. The ONNX model used here are the 43 M and 20 M models mentioned in clause 2.1. The 70 M models are not included as the performance was worse than as recorded in clause 2.1.

It is observed that the RTF is minimum during the first frame inference and it improves significantly in some cases while inferencing the subsequent frames. This may be explained by the cache performance as for the subsequent frame, data may already be present in the cache. For this reason, table 2.2. captures RTF values for frame 1 and subsequent frames separately.

|  | Specification | Model size |  | 1 thread | 4 threads |
| --- | --- | --- | --- | --- | --- |
| Device 1 | Release year: 2023 CPU: Hexa-core (2x3.46 GHz + 4x2.02 GHz) | ~ 43 M (32 kHz sample rate) | Max RTF | 1.7 | 2.16 |
|  |  |  | Min RTF | 0.9 | 1.2 |
|  |  |  | Min RTF steady state | 1.14 | 1.33 |
|  |  | ~ 20 M (32 kHz sample rate) | Max RTF | 3.33 | 4.0 |
|  |  |  | Min RTF | 1.25 | 1.77 |
|  |  |  | Min RTF steady state | 2.58 | 2.35 |
| Device 2 | Release year: 2023 CPU: Nona-core (1x3.0 GHz Cortex-X3 & 4x2.45 GHz Cortex-A715 & 4x2.15 GHz Cortex-A510) | ~ 43 M (32 khz sample rate) | Max RTF | 1.29 | 2.16 |
|  |  |  | Min RTF | 0.65 | 0.78 |
|  |  |  | Min RTF steady state | 0.65 | 0.78 |
|  |  | ~ 20 M (32 khz sample rate) | Max RTF | 2.5 | 4.0 |
|  |  |  | Min RTF | 1.23 | 2.0 |
|  |  |  | Min RTF steady state | 1.23 | 2.0 |
| Device 3 | Release year: 2022 CPU: Octa-core (1x3.00 GHz Cortex-X2 & 3x2.50 GHz Cortex-A710 & 4x1.80 GHz Cortex-A510) | ~ 43 M (32 khz sample rate) | Max RTF | 1.23 | 2.0 |
|  |  |  | Min RTF | 0.96 | 1.3 |
|  |  |  | Min RTF steady state | 0.96 | 1.48 |
|  |  | ~ 20 M (32 khz sample rate) | Max RTF | 2.0 | 4.0 |
|  |  |  | Min RTF | 1.23 | 2.0 |
|  |  |  | Min RTF steady state | 1.23 | 2.0 |
| Device 4 | Release year: 2019 CPU: Hexa-core (2x2.65 GHz Lightning + 4x1.8 GHz Thunder) | ~ 43 M (32 khz sample rate) | Max RTF | 1.14 | 1.66 |
|  |  |  | Min RTF | 0.6 | 0.43 |
|  |  |  | Min RTF steady state | 0.6 | 0.43 |
|  |  | ~ 20 M (32 khz sample rate) | Max RTF | 2.35 | 3.07 |
|  |  |  | Min RTF | 1.08 | 1.03 |
|  |  |  | Min RTF steady state | 1.77 | 2.0 |

Table 2.2. RTF results with experiment 2

Figure 2.1. RTF vs Model size plot with experiment 2 (single thread)

Figure 2.2. RTF vs Model size plot with experiment 2 (4 threads)

# Observations

The following observations can be drawn when running various models on smartphone CPUs (devices released in 2022-2023) with 5 sec chunk-based processing as defined in clause 2.1:

- Unmodified DAC model (~70 M) at 44.1 kHz does not meet real time requirements even when running on 4 threads.

- Unmodified DAC model (~70 M) at 24 kHz barely fits in real time when running on 4 threads. The model does not meet real-time requirements when running on 1 thread.

- Modified DAC model (~40 M) at 32 kHz shows an RTF = 2.27 on device1, RTF = 1.2 on device2 and RTF = 1.4 on device3 when running on a single thread.

- Modified DAC model (~20 M) at 32 kHz shows an RTF = 4.16 on device1, RTF = 2.3 on device2 and RTF = 2.6 on device3 when running on a single thread.

- The modified DAC model (~ 20 M) RTF of 4.16 is slightly worse than EnCodec (~15 M) RTF of 4.22-4.25 reported on a 3 Ghz clock CPU in [8].

The following observations can be drawn when running various models on smartphone CPUs (devices released in 2019-2023) with 80ms frame-based processing as defined in clause 2.2:

- On certain devices, processing time is higher for first frame, and the processing time reduces during subsequent frames.

- Modified DAC model (~40 M) at 32 kHz does not meet real time requirement on device 2, 3 and 4 even when running on 4 threads.

- In the first frame, modified DAC model (~20 M) at 32 kHz shows an RTF = 1.25 on device1, RTF = 1.23 on device2, RTF = 1.23 on device3 and RTF = 1.08 on device4 when running on a single thread.

- In the steady state, modified DAC model (~20 M) at 32 kHz shows an RTF = 2.58 on device1, RTF = 1.23 on device2, RTF = 1.23 on device3 and RTF = 1.77 on device4 when running on a single thread.

# Proposal

The source proposes to document clause 2 and clause 3 in clause 6.4 of 3GPP TR 26.940.

References

[1] Tdoc S4-250751, “Time plan for FS_ULBC study item”

[2] Tdoc S4aA250269, “On ULBC complexity design constraints”

[3] Tdoc S4aA250264, “Considerations on measuring ULBC complexity”

[4] TdocS4aA250263, “Discussion on CPU-centric Complexity for ULBC”

[5] TdocS4aA250253, “Complexity and memory considerations”

[6] https://github.com/descriptinc/descript-audio-codec

[7] https://opensource.googleblog.com/2022/09/lyra-v2-a-better-faster-and-more-versatile-speech-codec.html

[8] HIL Codec https://arxiv.org/pdf/2405.04752

[9] https://docs.pytorch.org/tutorials/beginner/onnx/export_simple_model_to_onnx_tutorial.html

[10] https://onnxruntime.ai/docs/

[11] https://onnxruntime.ai/docs/execution-providers/Xnnpack-ExecutionProvider.html

[12] https://support.apple.com/en-au/111831

[13] https://www.gsmarena.com/google_pixel_8-12546.php

[14] https://www.gsmarena.com/oppo_find_x5_pro-11236.php

[15] https://www.gsmarena.com/apple_iphone_11-9848.php