Source:	Samsung Electronics Co. Ltd

Title:	[AMD_PRO-MED] AF requested modification of set of Network Slice(s) for a UE

Agenda Item:	2.5

Document for:	Discussion and Agreement

# Introduction

The study on Network Slicing Extensions for 5G media services in TR 26941 (FS_MS_NS_Ph2) looked into few key issues and candidate solutions related to impact of 5G network slicing on 5GMS architecture. One of the key issues that was looked into during this study, and documented in clause 6.3 of TR 26941, is the issue of moving media flows to other network slices. This key issue studied the aspect of network slice replacement specified by 3GPP SA2 in TS 23.501 (clause 5.15.19) and the below conclusion was documented in clause 6.3.3 of TR 26941 to further monitor related work in SA2:

This Key Issue studied the aspect of network slice replacement specified by 3GPP SA2 in TS 23.501 [7] and its impact on 5G Media Streaming procedures. The study found that: - The network slice replacement procedure is transparent to the Application Function and the Application Service Provider. - There is no guarantee that the Alternative S-NSSAI provides similar performance as that of the replaced S-NSSAI when a network slice replacement procedure is undertaken by the 5G System. We conclude that: - Per 3GPP SA2 guidance on the current support of network slice replacement in the 5G System, no normative work to enhance existing procedures in TS 26.501 [20] and data model definitions in TS 26.512 [21] is required at this time. - Future work in 3GPP SA2 related to network slice replacement is to be monitored, and the data model definitions in TS 26.510 [42] enhanced, if necessary.

After the completion of FS_MS_NS_Ph2 study, 3GPP SA2 introduced and specified an alternative procedure for network slice replacement based on request from AF. This work was carried out as part of TEI19_SliceSel work item. 3GPP SA2 specified stage-2 procedures for AF requested modification of set of Network Slice(s) for a UE in TS 23501, 23502, and 23503. Based on these stage-2 procedures, 3GPP CT3 and CT4 specified stage-3 details to support AF requested modification of set of Network Slice(s) for the UE in TS 29507, 29502, and 29504. While the original network slice replacement procedure described in clause 5.15.19 of TS 23501 had no impact on the AF (hence transparent to AF as per conclusion for key issue#3 in TR 26941), the new procedure on AF requested slice change in clause 5.15.5.2.2a of TS 23501 is directly related to the AF functionality.

Clause 2 of this contribution describes the details of stage-2 and stage-3 procedures of AF requested slice change in 3GPP SA2, CT3, and CT4 specifications. Clause 3 of this contribution provides a brief description of impact of the above new procedure on 5GMS. Clause 4 presents a brief proposal for way forward on documenting the impact of above procedure in our specifications.

# Background specification on AF requested modification of Network Slice(s) for a UE

Clause 5.15.5.2.2a of TS 23.501 specifies procedure for AF requested modification of the Set of Network Slice(s) for a UE. As specified in this procedure

-	Step a: An authorized AF, or an AF subject to authorization via the NEF, may request to replace a certain S-NSSAI (Replaced S-NSSAI) for a UE with another S-NSSAI (Alternative S-NSSAI) which is part of the UE subscription. The above AF may then, later on, request to terminate the replacement and then request to use the original Replaced S-NSSAI again instead of the Alternative S-NSSAI. All these requests are sent to the PCF for the UE handling the impacted subscriber

-	Step b: The PCF notifies the AF may subscribe to notification on the outcome of the AF requested Network Slice Replacement

-	Step c: The PCF sends Slice replacement management in the access and mobility management policies to the AMF per implicitly subscription, as described in TS 23.503. The Slice replacement management policy includes the Replaced S-NSSAI and the corresponding Alternative S-NSSAI as provided in the AF request, PCRT to request Notification on outcome of Network Slice Replacement request, together with a Network Slice Replacement Type. The Network Slice Replacement Type indicates that the Network Slice Replacement is requested by an AF.

-	Step d: The AMF, upon receiving the above Slice replacement management policy, performs necessary slice management operations (e.g., remove Replaced S-NSSAI from Allowed S-NSSAI, and add the Replaced S-NSSAI into the Rejected S-NSSAI by UE Configuration Update procedure). The UE and AMF release PDU sessions that were using the Replaced S-NSSAI. After updating necessary UE context information locally, the AMF reports to the PCF the outcome of network slice replacement as specified in clause 6.1.2.5 of TS 23.503. The PCF further notifies the AF of this outcome

-	Step e: Operator policies in the PCF ensure that UE is configured with a URSP rule that contains the Alternative S-NSSAI to establish a new PDU Session.

Steps b-e above are repeated again when the AF intends to go back to the original Replaced S-NSSAI from Alternative S-NSSAI.

Clause 4.15.6.9.2 of TS 23.502 specifies the updated call flow for handling AF request targeting an individual UE using NEF to incorporate changes because of the AF requested modification of Network Slice(s).

Clauses 6.1.2.1 and 6.1.2.6 of TS 23.503 specifies the details of the above slice replacement procedure as part of AF influence on Access and Mobility related policy control.

Corresponding stage-3 updates based on the above procedure are specified by 3GPP CT4 (in clauses 5.2.2.4 and 6.1.6.3.8 of TS 29502, and clause 6.18 of TS 29504), and by 3GPP CT3 (in clauses 4.2 and 5.6 of TS 29507).

# Potential impact on 5GMS architecture

The original network slice replacement procedure described in clause 5.15.19 of TS 23.501, based on which the FS_MS_NS_Ph2 study documented the findings in clause 6.3 in TR 26941, it is specified that the information about Alternate S-NSSAI is informed to the AMF by any of the PCF, NSSF, or OAM.

However, in clause 5.15.5.2.2a in which 3GPP SA2 specified the alternate procedure of AF requested modification of the Set of Network Slice(s) for a UE, there exists a note:

NOTE 1: It is assumed that in the area where the AF requested Network Slice Replacement feature described in this clause is expected to operate, the AMFs supports AF requested Network Slice Replacement notifications handling from the PCF. How the AF obtains the S-NSSAI to be replaced and the Alternative S-NSSAI is out of scope of this specification.

From the above note, it is clear that how AF obtains the Replaced S-NSSAI and the Alternative S-NSSAI is unknown.

We believe that such information may be provided to the AF by the 5GMS Application Provider using M1 API specified in TS 26501 and TS 26510. The 5GMS Application Provider may be aware of this information (e.g., via OAM or network operator) using the mechanisms described in clauses 4.4 and 5 of TR 26941.

We believe the procedure for AF requested modification of the set of Network Slice(s) for a UE may be accomplished using the following steps:

-	Step 1: Using existing M1 API, during policy template provisioning procedure, multiple Network Slice and/or Data Network tuples may be provisioned if the Policy Template allows media to be delivered in multiple Data Networks and/or network slices at reference point M4. The data model for PolicyTemplate resource can enhanced to indicate which tuple among the above multiple Network slice and/or Data Network tuples correspond to the primary(original) network slice (S-NSSAI) and which one corresponds to the Alternate network slice (S-NSSAI).

-	Step 2: The dynamic policy invocation configuration information is fetched by the Media Session Handler from the 5GMS AF using the M5 Service Access Information API. The response includes a list of 5GMS AF endpoint addresses that provide the dynamic policy API and set of valid policyTemplateIds.

-	Step 3: 5G Media Streaming media flows are carried over PDU Sessions in the primary (original) network slice S-NSSAI to begin with. Media is delivered to the UE in primary S-NSSAI at reference point M4

Some time later:

-	Step 4: When the Media Session Handler intends to activate a QoS-related Dynamic Policy Template, it includes a ClientQoSSpecification property in the dynamic policy request to the AF. For transient operational reasons, it is possible that the requested ClientQoSSpecification cannot be satisfied by the network slice for which policy template applies at the time of the dynamic policy invocation request.

-	Step 5: To satisfy the network QoS requirements of the target policy template, a potential solution is to extend the scope of the 5GMS architecture to allow the affected media flow to be migrated to a different PDU Session in a different network slice. The AF identifies the Alternate network slice S-NSSAI based on configuration in Step-1 above, and performs the AF requested modification of the Network Slice for the UE procedure specified in clause 5.15.5.2.2a of TS 23501 and introduced in clause 2 of this contribution. The primary(original) S-NSSAI configured in Step-1 is the Replaced S-NSSAI in context of the AF requested modification of Network slice procedure. As a result of this slice change procedure executed by the AF, the 5G Media Streaming media flows are delivered to the UE in Alternate S-NSSAI at reference point M4.

# Proposal

The following is proposed

-	A detailed discussion paper with a call flow and text description of the procedure described in clause 3 above is submitted to SA4#132.

-	S4-250634, an endorsed stage-3 CR of TS 26510 (CR0008r3) during SA4-131-bis-e meeting, is to be extended with data model enhancements and the design principles based on the procedure presented in clause 3 of this contribution, and submitted to SA4#132

.