# Advancing 5G Media Delivery: Lessons from Trials and Commercial Applications in Unicast, Broadcast, and Hybrid Systems

Thomas Stockhammer4, Frédéric Gabin1, Jordi Gimenez2, Daniel Silhavy3, Richard Bradbury5 and Johann Mika6

1Dolby Labs, Paris, France
25G-MAG, Geneva, Switzerland
3Fraunhofer FOKUS, Berlin, Germany
4Qualcomm Germany GmbH, Munich, Germany
5BBC, London, United Kingdom
5ORS, Vienna, Austria

ABSTRACT

The rapid evolution of 5G technology continues to revolutionize media delivery, with significant advancements in both Advanced Media Delivery (AMD) and 5G Broadcast systems. The core motivation behind AMD, a Rel-19 work item, is to enhance the efficiency and quality of media streaming over 5G networks. This involves addressing segmented media objects, ensuring interoperability, and introducing new functionalities to meet the growing demands of modern media consumption. Key technologies driving AMD include common client metadata, server-assisted streaming, multi-CDN and multi-access media delivery, DRM integration, enhanced support for QoS as well as MBS enhancements. These innovations aim to provide seamless, high-quality media experiences by leveraging the capabilities of 5G networks.

In addition, the ETSI TS 103 720 5G Broadcast profile is extended, the next version is set to be completed for IBC 2025. It incorporates updates from 3GPP Release 18 and 19 and focuses on new functionalities and improvements based on industry feedback. Recently completed and ongoing extensions include the specification of broadcast bandwidth of 6/7/8 MHz for 5G Broadcast, all transmitter and receiver requirements for UHF Band operation (the famous band 108), and finally also time-frequency interleaving to increase coverage and robustness in mobility cases. Still under discussion there are minor extensions to support time-multiplexing legacy broadcast with 5G Broadcast. It also includes a receiver launch profile, as well as recommendations on how to combine 5G broadcast with DVB-I. The integration into tests, trials and commercial services is ongoing and the latest status will be presented.

These developments highlight the dynamic nature of 5G distribution, showcasing the continuous efforts to enhance media delivery systems and ensure they meet the evolving needs of consumers and industry alike. The paper will also present initial thoughts for the transition to 6G.

Introduction

5G-based media delivery addresses several needs of content providers, service providers, mobile network operators as well as verticals including broadcast network operators. In this paper, an overview of the recent advances in standards and industry is provided.

AdVANCED mEDIA dELIVERY - OVERVIEW

TS 26.501 [1] defines the 5GMS architecture, call flows, and procedures. TS 26.502 [2] defines the MBS User Service architecture with related call flows and procedures. Mobile media delivery is as important as never before with everlasting growth of video streaming traffic and new functionalities and services provided by mobile network operators and third-party service providers. It is also worth noting that 5G-MAG has defined created reference implementations of both 5G Media Streaming and MBS.

3GPP conducted an extensive study in Rel-19 to identify different key areas to extend 5G Media Streaming and MBS. Figure 1 provides and overview of the study areas and for several of those key issues, normative follow-up work was conducted. The study outcomes are documented in TR 26.802 for MBS and in TR 26.804 for 5G Media Streaming. Based on this extensive feasibility study, several new features are defined in Rel-19.

Figure 1 Study Areas for 3GPP Advanced Media Delivery (yellow indicates follow-up normative work)

In particular, TS 26.502 on MBS User Services has been updated to address the following features

- In-session unicast repair for MBS Object Distribution: This functionality addresses all stage-2 procedures for unicast repair, and in particular adds the feature to use unicast for repairing potentially lost data in objects, for example for DASH/HLS over MBS during an ongoing live session.

- Time Synchronization for MBS: The MBS client and the network functions require time sync. Based on exchange with SA2 and RAN2, two options are provided: (1) usage of SIB9, and (2) a user domain time service.

- Deployment Guidelines for MBS User Service and Delivery Protocols for eMBMS: A new informative Annex documents deployment scenarios to operate MBS User Services over eMBMS, in particular to harmonize northbound reference point and client APIs, as well as to deploy MBS user service over eMBMS/5G Broadcast. Before making this normative, further validation is required to determine whether modifications in the MBS service architecture in TS 23.247 are needed to address all interoperability aspects of these deployment options.

In particular, TS 26.501 on 5G Media Streaming has been updated to address the following features

- In-band reporting for Common Client Metadata: This feature permits to collect client data (buffer status, bitrates, codecs, and so on) collected by modern media players including DASH and HLS and aggregate and expose through NWDAF. Doing so offers opportunities for MNOs for more data information. More details are provided below.

- Multi-access media delivery: This feature adds an informative Annex with description of multi-access media delivery, and mapping of Access Traffic Steering-Switching-Splitting (ATSSS) architecture into the 5GMS architecture.

- Media delivery from multiple service endpoints/locations: This feature enables the provisioning and distribution of content from different service locations, for example CDNs. It may for example permit that a media player accesses content from a 5GMS AS and from an external application server concurrently or switches according to client or network decisions. It may also allow to provision multiple service locations in a 5GMS AS. A new reference point M13 is added. The feature enables stage-3 functionalities such as Content Steering or Coded Multisource Media Format (CMMF). For some details refer to below.

- Distributing encrypted and high-value content: This feature permits to support packaging and transcoding in network for DRM-protected content. While provided for 5GMS, it is generic to also be combined with MBS and MBMS and hence provides a workflow for distributing DRM-protected content in 3GPP services.

- Improved QoS support for Media Streaming services: This feature in permits enabling ECN marking for L4S in the 5G System. If set, this flag directs the 5GMS Client to select and activate ECN marking for L4S when it instantiates the Policy Template. It also permits to enable QoS monitoring in the 5G System (as described in clause 5.45 of TS 23.501) for measurement and reporting of QoS parameters when this Policy Template is instantiated. Feature applies to both, uplink and downlink streaming.

- Media Streaming aspects of Network Slicing: This feature provides Dynamic Policies procedures for uplink and downlink to support policy provisioning for a plurality of Network Slices and/or Data Networks and provides bootstrapping of application invocation on a Network Slice.

Several topics have not been concluded, but are recommended to be subject for further study, including:

- For Uplink Streaming for the application of uplink 5G Media Streaming for media production and contribution be studied in more detail.

- For Common server-and network-assisted streaming to continue studying Common Media Server Data (CMSD) and its potential benefits in the context of 5G Media Streaming

- For Optimising modem usage for media streaming to identify the current limitations and assess the potential of candidate solutions of device-level media resource management with network-assisted scheduling for media delivery.

- For DASH/HLS Interoperability to continue studying this Key Issue in future versions of the present document take into account the expected revised version of CTA-5005A [147] in such a future study.

- For QUIC-based Media Delivery to further study:

  - The standardized definitions of QUIC client and server APIs and their availability on the market.

  - The standardized definitions of WebTransport client and server APIs and their availability on the market.

  - The fragmentation of QUIC implementations (both on the client and server side) and their performance for segmented media delivery.

  - Evaluation of the identified candidate technologies with regards to QoE metrics (start-up delay, stalling events, etc.).

  - The impact of reporting qlog metrics via the downlink interface (M4) to the UE (bidirectional streams).

- For Secure Communication of Network Properties (SCONE-PRO) and 5G Media Streaming to study the potential impact of Secure Communication of Network Properties (SCONE-PRO) as defined in IETF on 5G Media Streaming.

- For Dynamic content generation from multiple sources to further study support for Redundant encoding and packaging for segmented live media (ReAP) and configuration of REaP in 5GMS

In-band reporting for Common Client Metadata

One of the key additions in Rel-19 AMD is the ability to support In-band reporting for Common Client Metadata. Media players and the Delivery Networks that serve them live in two separate worlds. At the media player is the notion of a playback session which, by example, may last for an hour as the end-user watches a sporting event. Over that time there may be some rebuffering issues and the player may switch bitrates. Due to the nature of segmented media playback, the delivery networks view of this same event is thousands of independent requests for video files, audio files, caption files and playlists. Hence, it is difficult for content distributors as well as the distribution network operators to infer anything about the quality of experience (QoE) of an individual session, or of the aggregate health of a player cohort. The industry needed a bridge between these two worlds –a mechanism by which a player can communicate mutually beneficial media-related information to a CDN. This method of exchange was standardized as Common Media Client Data (CMCD) CTA-5004 by the Consumer Technology Association in September 2022. CMCD defines a simple set of rules by which media players attach structured information to each media object request as either a query argument or a custom header. This CMCD data is added to the CDN logs and is processed by both the CDN and the content distributor. Player support has grown rapidly, with most player ecosystems, including AVPlayer and Exoplayer, now supporting CMCD. CMCD is simple to understand, simple to implement and common across all players and all CDNs. The use-cases which CMCD enables are broad – including robust prefetching, analytics solutions, forensic debugging, CDN delivery optimization, alerting and monitoring systems, low latency optimizations, server-side switching, research analytics and content steering decisioning.

As the technology gets broadly supported in players and distributions systems, enabling the collection of CMCD data was one of the key issues for Advanced Media Delivery. The use cases which CMCD enables are broad – including robust pre-fetching of content, analytics solutions, forensic debugging, CDN delivery optimisation, alerting and monitoring systems, low latency optimisations, server-side switching, research analytics and content steering decision-making. In the context of 5G Media Streaming, the CMCD information may be used for several purposes that are elaborated further in the remainder of this clause:

- Operational optimisation of the 5GMSd AS: The Application Server uses CMCD information to optimise its operation. For example, the 5GMSd AS may choose to pre-fetch content from the 5GMSd Application Provider based on the value of the CMCD Next object request key signalled by the Media Player.

- Operational optimisation of the 5GMSd AF and 5G Media Streaming: The Application Function uses CMCD information in order to configure the 5G System for optimised media delivery across the User Plane. For example, the 5GMSd AF may invoke service operations on the PCF in order to effect a delivery boost if the CMCD Buffer starvation flag is set by the Media Player.

- UE data collection on media playback and 5GMS reception, reporting and event exposure by 5GMS System: The Application Function uses the data collection and reporting framework to provide CMCD information to the 5G System or to external 5GMS Application Providers. This could, for example, take the form of a time series event log for audit purposes. Such data could, for example, be useful to determine the quality of reception and to identify anomalies or errors that can occur on players.

While several options were studied on how collection of CMCD data is most suitably integrated into 5G Media Streaming, it was concluded that the preferred solution is In-band reporting of CMCD information via reference points M4d and M3d, for the following reasons:

- In-band reporting reference point M4d is broadly implemented in common media clients nowadays.

- In-band reporting permits operational optimizations by the 5GMSd AS, which would not the case with Out-of-band reporting of CMCD information via reference points M11d and M5d.

- Passing the CMCD information to the 5GMS AF at reference point M3d (Option 1) permits operational optimizations by the 5GMSd AF, which is not the case with In-band reporting of CMCD information via reference points M5d and R4.

- All envisaged use cases can be supported by these options.

Figure 2 CMCD integration into 5G Media Streaming

Figure 3 illustrates a scenario where client data collection and in-band reporting by the 5GMSd AS is configured by the 5GMSd AF via reference point M3d. In this example, it is assumed that the client data collection information provided by the 5GMSd AF comprises instructions/rules regarding client data reporting. It is further assumed that the 5GMSd AF is required to expose client data events to separate destination entities, upon optionally having performed post-processing on the collected report information. The 5GMSd AF and 5GMSd AS may be either trusted or untrusted.

Figure 3 Client data collection in 5GMSd reported in band via M4d and M3d

Stage-3 work is currently in progress, to define all protocol features. This includes, relevant provisioning and configuration extending the service-based protocols, reporting from clients using header and query mechanisms, as well as a JSON-based QoE metrics reporting envelope to be used in the first instance to convey CMCD information at reference point M3d.

Media delivery from multiple service endpoints/locations

Content distributors often use multiple Content Delivery Networks (CDNs) to distribute their content to end-users. As an example, they may upload a copy of their catalogue to each CDN, or more commonly have all CDNs pull the content from a common origin. A multi-CDN or multiple service location/endpoint content offering generally provides content in a redundant manner at locations that can be differentiated by the client, and provide different technical or commercial options. Different multi-source/service location approaches considered include:

- DNS-based switching as defined by the IETF in RFC1034 and RFC1035

- MPEG-DASH client-side switching, as defined in ISO/IEC 23009-1

- Content steering driven switching as defined for DASH in ETSI TS 103 998

- SAND4M-based switching as defined in 3GPP TS 26.247

- CMMF-based multi-source delivery as defined in ETSI TS 103 973.

Two different architecture options are considered, once for which the multiple service locations are provided as part of the 5GMS AS, and a second case for which the 5GMS AS serves as one of the service locations, but content may also be delivered from the Application Service Provider.

A prominent example for multi-service location is Coded Multi-source Media Format (CMMF). CMMF is an extensible container format designed to facilitate the management and interchange of audio-visual media and metadata in one or more coded representations (e.g., encoded with application-layer, forward error correction (FEC), linear, network, or channel codes). The coded media representations supported by CMMF enable the efficient use of multi-source, multi-path, and multi-access connectivity for network-delivered media applications. The use of CMMF does not replace the basic media streaming architectures and procedures already defined. Rather, it is intended to supplement them to provide additional capabilities.

A typical Video-on-Demand (VoD) MPEG-DASH HTTP adaptive streaming system is set up similarly to the non-shaded blocks shown in Figure 4. Source media (e.g. audio/video elementary streams) are segmented and encoded into multiple representations, each with a different quality and bit rate. These segments are packaged together using MPEG-DASH (or HLS) and stored on an origin server located within the network. One or more CDNs are set up to distribute and deliver this content to an OTT service provider’s customer base. These CDNs obtain every requested MPEG-DASH (or HLS) segment from an origin server, caches these segments at their respective network edges, and deliver these segments to clients.

Enabling multisource delivery using CMMF within this existing delivery architecture can be accomplished through the addition of a CMMF Bitstream Generator/Source before segments are delivered to the CDNs, and a CMMF Receiver on each client, as illustrated by the shaded boxes in Figure 4.

Figure 4 CMMF media delivery

Annex D of ETSI TS 103 973 defines a content delivery protocol instantiation. In this case, CMMF is considered as a Content Delivery Protocol (CDP) as defined in clause 8 of RFC 5052 [131]. Annex D provides a mapping of CMMF to the RFC 5052 principles and vice versa. This instantiation also permits re-use of existing FEC Codes including Raptor (the 3GPP MBMS code) as defined in RFC 5053 [132] and RaptorQ as defined in RFC 6330 [127]. The content delivery protocol also makes use of 3GPP concepts such as the FLUTE [133] File Delivery Table (FDT) as used in MBMS User Services [54] and MBS User Services [135].

Figure 5 Candidate topics for the the third version of the ETSI TS 103 970

Latest on 5G Broadcast

On the 5G Broadcast front, 5G-MAG is currently developing the third version of the ETSI TS 103 970 (LTE-based 5G Broadcast) specification. The work is guided by the discussion on github and addresses among others the following aspects (1) bugfixes and corrections based on feedback from implementors, and (2) extensions to address new developments in 3GPP and the industry.

In particular, extensions defined in the completed 3GPP Rel-18 and the ongoing Rel-19 (until September 2025) are strong candidates for the extension. An overview of the expected features is provided in the following, based on the discussion seen in Figure 5:

- New bands: Rel-18 added the entire UHF band for LTE-based 5G Broadcast as Band 108, and it is expected that this band is added to the third version of the ETSI specification. In addition, in Rel-19 3GPP added additional sub-bands of the UHF Band 108, namely Band 112 (primarily for Europe), and Band 113 (primarily for North America deployments).

- Rel-19 adds a compelling time interleaver of up to 512 ms to the 3GPP specs alongside a frequency interleaver with a careful design including shifts an permutations to maximize time and frequency diversity. Both make a substantial difference in performance as extensively demonstrated in the simulation results evaluated. And all designed taking into account the hardware constraints of the products we all have in our pockets, i.e. smartphones with 3GPP chipsets, following the smart design to reuse the functional building blocks of the 3GPP UEs. Preliminary results were presented at IBC 2023.

- Rel-19 also adds CAS muting which allows that deployed broadcast systems such as ATSC 3.0 and DVB-T2 and 5G Broadcast can be operated in the same RF channel. For instance, ATSC 3.0 introduces the concept of the "bootstrap signal" that allows introducing "gaps" in the time domain transmission to be used by future extensions of the standard. DVB-T2 introduces a similar mechanism for the introduction of "future extension frames". This extension addresses potentially regulatory requirements as well as soft refarming of RF carriers for dual use. A comprehensive overview was provided at IBC 2024.

- DVB-I is a potential service layer on top of 5G, including 5G Broadcast. 3GPP and DVB have done relevant extensions to address the integration of 5G Broadcast to the DVB-I service layer, including 3GPP service URLs, identifier-based DVB-I services and defining the relevant mechanisms to combing with unicast delivery. A detailed explanation was provided in earlier papers in IBC2023 and IBC2024.

- A key topic of the upcoming version is the definition of a set of features for a potential commercial service introduction in the near future. This includes receiver profiles for radio, service and application constraints, for example frequency bands, bandwidth, support for specific protocols and codecs, etc.

- Additional features include concurrent unicast and broadcast operation, enhancement to public warning system, a broadcast positioning service, receiver implementation options using different Android APIs, as well as enhancements to the service layer based on updated 3GPP specifications.

Figure 6 5G Broadcast features defined in Rel-19

The new version of the ETSI specification for 5G Broadcast is expected to be stable by September 2025 and will be published by the end of the year.

CONCLUSIONS

5G Media Delivery is an active development area and this paper summarized the key updates for Advanced Media Delivery and 5G Broadcast. The development in these areas also paves the way for what is expected in 6G: More collaboration, more harmonization with general media delivery architectures, support for verticals, and cost savings.

The 5G-MAG Reference Tools (see developer.5g-mag.com) are making available reference implementations with a focus on the specifications related to Advanced Media Delivery and 5G Broadcast.

ACKNOWLEDGEMENTS

The authors would like to thank all the members involved in the Advanced Media Delivery and 5G Broadcast standardization activity. We would also like to thank the International Broadcasting Convention for permission to publish this paper.

REFERENCES

- 3GPP TS 26.501 – 5G Media Streaming (5GMS); General description and architecture

- 3GPP TS 26.502 – 5G multicast-broadcast services; User service architecture

- 3GPP TS 26.531 – Data Collection and Reporting; General Description and Architecture

- 3GPP TR 26.804 – Study on 5G media streaming extensions

- CTA-5004 – Web Application Video Ecosystem - Common Media Client Data

- ETSI TS 103 998 – DASH-IF: Content Steering for DASH

- CTA-5006 – Web Application Video Ecosystem - Common Media Server Data

- ISO/IEC 23009-1 – Information technology — Dynamic adaptive streaming over HTTP (DASH) Part 1: Media presentation description and segment formats

- 3GPP TS 23.501 – System architecture for the 5G System (5GS)

https://github.com/orgs/5G-MAG/projects/32/views/1?query=sort%3Aupdated-desc+is%3Aopen