3GPP TSG-SA4 Meeting #-(AH) MBS SWG post 132	S4aI250098

# Online, , 5th Jun 2025 - 10th Jul 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.501 | CR | 0111 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [AMD_PRO-MED] WT2: Alignment with TS 26.512 CR 0091 - CMMF architecture and procedures mapping |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Dolby Laboratories Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AMD_PRO-MED |  |  |  |  |  | Date: |  |  | 2025-06-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Alignment with TS 26.512 CR 0091 |  |  |  |  |  |  |  |  |
| Summary of change: |  | New annex that maps the use of CMMF onto the 5GMS architecture and procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Objectives of the Work Item not completely satisfied. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 26.512 CR 0091 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4al250098 |  |  |  |  |  |  |  |  |

=====  CHANGE =====

# 2	References

…

[48]	ETSI TS 103 973: "Coded Multisource Media Format (CMMF) for Content Distribution and Delivery", October 2024.

=====  CHANGE =====

## 3.3	Abbreviations

…

CMMF	Coded Multisource Media Format

…

=====  CHANGE =====

Annex X (normative):
CMMF-enabled media streaming

#### X.1	Introduction

Coded Multi-source Media Format (CMMF) as specified in ETSI TS 103 973 [48] is an extensible container format designed to facilitate the management and interchange of audio-visual media and metadata in one or more coded representations (e.g., encoded with Application Layer Forward Error Correction, linear, network, or channel codes). The coded media representations supported by CMMF enable the efficient use of multi-source, multi-path, and multi-access connectivity for network-delivered applications.

Content delivery within the 5GMS System may be augmented using CMMF to enable delivery of media resources through the simultaneous use of multiple service locations exposed by the 5GMS AS at reference point M4 and/or the 5GMSd Application Provider at reference point M13d. The use of CMMF for content delivery is initiated by the 5GMS Application Provider where the establishment of a Provisioning Session in which a Content Hosting Configuration or Content Publishing Configuration is defined to distribute CMMF objects at reference point M4 and/or M13d, in addition to the definition of Media Entry Points containing CMMF configuration information. Media resources may be encoded within CMMF objects by either the 5GMS Application Provider, 5GMS AS, or 5GMS Client depending on the use case. Likewise, the 5GMS Application Provider, 5GMS AS, or 5GMS Client (depending on the use case) may decode received CMMF objects to recover media resources.

This annex maps the use of multi-source object coding using CMMF onto the 5GMS System.

#### X.2	CMMF-enabled 5G unicast downlink Media Streaming architecture

A mapping of CMMF onto the 5G unicast downlink media streaming architecture described in clause 4.2 is shown in figure X.2-1. Compared with figure 4.2.1-2 and 4.2.2-1, the 5GMSd AS is augmented with a CMMF Object Server subfunction and the Media Access Client in the Media Player is enhanced with the ability to download and process CMMF objects.

Figure X.2-1: CMMF-enabled 5G unicast downlink media streaming architecture

The descriptions for the following functions, as defined in clause 4.2, are expanded upon to describe the use of CMMF for downlink media streaming:

-	Media Player: A function as defined in clause 4.2.1. A Media Player that supports CMMF-enabled downlink media streaming sets up and configures a CMMF-enabled Access Client upon reception of a CMMF Media Player Entry containing necessary CMMF configuration information and a description of a media presentation (e.g., MPD for DASH content, URL to a video clip file, etc.).

-	Media Access Client (CMMF-Enabled): A function as defined in clause 4.2.2. A CMMF-enabled Media Access Client:

-	May extend the functionalities of other defined Media Access Clients (e.g., a DASH Access Client) to enable the streaming of media using CMMF.

-	Uses the Media Player Entry to locate and request CMMF objects containing encoded representations of the media resources (e.g., CMAF segments) required by the Media Player from one or more service locations exposed by the 5GMSd AS at reference point M4d and/or by the 5GMS Application Provider at reference point M13d. If indicated by the Media Player Entry, the CMMF-enabled Media Access Client may also translate media resource URLs contained within a media presentation (e.g., MPD for DASH content, URL to a video clip file, etc.) into URLs pointing to the CMMF objects containing encoded representations of those media resources.

-	For each media resource (e.g., CMAF segment), downloads one or more CMMF objects (either partially or in full) from one or more service locations exposed by the 5GMSd AS at reference point M4d and/or 5GMSd Application Provider at reference point M13d.

-	Recovers the media resource by decoding the received CMMF object(s).

-	Makes decoded media resources (e.g., CMAF segments) available to the Media Playback and Content Decryption Platform (specified in TS 26.511 [26]) for immediate or delayed consumption.

-	If retrieval of a media resource using CMMF fails, the CMMF-enabled Media Access Client should attempt to acquire the media resource using an alternate method (for example, by retrieving the non-CMMF-encoded media resource directly from a single service location exposed by the 5GMSd AS at reference point M4d).

-	5GMSd AS: A function as defined in clause 4.2.1. The 5GMSd AS may:

-	Create or modify a Media Entry Point (or a document pointed to by a Media Entry Point) that contains necessary CMMF configuration information and media presentation description(s).

-	Ingest CMMF objects, where each object is a different CMMF-representation of a media resource, from the 5GMSd Application Provider at reference point M2d.

-	Ingest a media resource (e.g., CMAF segment) from the 5GMSd Application Provider at reference point M2d and encode it within one or more CMMF objects (where each is a different CMMF-encoded representation of the ingested media resource) using a provisioned Content Preparation Template.

-	Cache ingested and/or prepared CMMF objects.

-	Distribute CMMF objects to CMMF-enabled Media Access Clients from one or more service locations exposed at reference point M4d.

-	5GMSd Application Provider: A function as defined in clause 4.2.1. The 5GMSd Application Provider may:

-	Create a Media Entry Point (or a document pointed to by a Media Entry Point) for ingest at reference point M2d or distribution at reference point M8d. The Media Entry Point (or a document pointed to by a Media Entry Point) contains CMMF configuration information and/or a description of a media presentation (e.g., MPD for DASH content, URL to a video clip file, etc.), including any relevant service location information for service locations exposed at reference point M13d.

-	Encode a media resource (e.g., CMAF segment) within one or more CMMF objects (where each is a different CMMF-encoded representation of the media resource) prior to ingest at reference point M2d and/or distribution at reference point M13d.

-	Host media resources (e.g., CMAF segments) and/or CMMF objects for ingest at reference point M2d and/or distribution at reference point M13d.

#### X.3	CMMF-based procedures for downlink media streaming

## X.3.1	General

The CMMF-based procedures for downlink media streaming follow those described in clause 5. This clause provides additional details for provisioning the 5GMS System for CMMF-based multisource downlink media streaming and multisource delivery of CMMF-encoded media.

## X.3.2	CMMF-based multi-source delivery procedure

Figure X.3.2-1 illustrates a variant of the high-level procedure provided in clause 5.2.6 in which downlink streaming is from multiple service locations. Differences from the baseline procedure are highlighted in boldface.

The procedure makes the following assumptions:

-	CMMF-encoded content is hosted at two or more service locations. These service locations may be located inside the 5GMS System (i.e., hosted by the 5GMSd AS and exposed via reference point M4d) or outside it (i.e., hosted by the 5GMSd Application Provider and exposed via reference point M13d).

NOTE:	It is the responsibility of the 5GMSd Application Provider to configure and provision service locations hosted outside of the 5GMS System. In such cases, the 5GMSd Application Provider is also responsible for updating the Media Player Entry to include the necessary multiple service location configuration information to access content from these service locations.

-	The CMMF-enabled Access Client in the Media Player has the necessary functionality to acquire CMMF-encoded media from multiple service locations. This includes the functionality needed to acquire CMMF-encoded objects from multiple service locations concurrently.

-	CMMF configuration information required to access content across multiple service locations is available within the Media Player Entry (or available alongside the Media Player Entry, such as within a document referenced by the Media Player Entry). This configuration information may be:

-	Embedded in a Media Player Entry document (e.g., MPD),

-	Provided alongside the Media Player Entry document, such as in a separate document referenced by a Media Player Entry document (e.g., MPD), or

-	Provided as the Media Player Entry document with a reference to a document containing the media streaming presentation information (e.g., MPD).

Figure X.3.2-1: High-level procedure for downlink streaming from multiple service locations
using CMMF

Steps:

1.	The 5GMSd Application Provider provisions the 5G Media Streaming System, including content hosting and ingest, such that CMMF-encoded content is available from two or more service locations (labelled Service Location 1, Service Location 2, etc.) where each service location hosts a unique CMMF representation of the content to be streamed. Upon successful provisioning and content ingest (see clause 5.4.4), either the 5GMSd Application Provider or the 5GMSd AS may create or update Media Player Entry documents (or documents pointed to by each Media Player Entry document) to include any necessary CMMF configuration information required by the 5GMSd Client to access media content from multiple service locations.

2.	The 5GMSd-Aware Application triggers the Service Announcement and Service and Content Discovery procedure. The Service and Content Discovery procedure only involves the 5GMSd-Aware Application and the 5GMSd Application Provider. The Service Announcement includes either the whole Service Access Information (i.e. details for Media Session Handling at reference point M5d and for Media Streaming access at reference point M4d) or a reference to the whole Service Access Information.

3.	A media content item is selected.

4.	The 5GMSd-Aware Application triggers the 5GMSd Client to initiate the 5G Media Streaming Service.

When the 5GMS-Aware Application has received only a reference to the Service Access Information (see step 1):

5.	The Media Session Handler interacts with the 5GMSd AF to acquire the whole Service Access Information. The Service Access Information may include Media Player Entry URLs.

6.	The Media Session Handler provides the Media Player Entries to the 5GMS-Aware Application. The information may indicate a precedence order for these Media Player Entries.

Then:

7.	The Media Player is invoked with the selected Media Player Entry to start media access and playback.

8.	The CMMF-enabled Access Client establishes the transport session for acquiring the Media Player Entry.

9.	The CMMF-enabled Access Client requests the Media Player Entry.

10.	The CMMF-enabled Access Client receives the Media Player Entry.

11.	The Media Player processes the Media Player Entry. From the Media Player Entry, the Media Player determines the CMMF configuration, including the set of available service locations where CMMF-encoded content can be accessed and the CMMF object URL(s) for the media resources to be acquired. It further determines, for example, the number of transport sessions to each service location needed for media acquisition. The Media Player uses the Media Player Entry information to initialize the media pipelines for each media stream. The Media Player Entry also contains information to initialize the DRM client, when DRM is used.

12.	The Media Player notifies the Media Session Handler about the Media Player Entry.

13.	Optional: the Media Player acquires the necessary DRM information, for example a DRM License.

14.	The Media Player configures the media playback pipeline.

15.	The Media Player establishes the necessary transport sessions for the content according to CMMF configuration information indicated by the Media Player Entry. These transport sessions may be established between the CMMF-enabled Access Client and any one or more of the available service locations. For example, the Media Player may establish one transport session for each media component (audio, video, etc) and possibly additional transport sessions for other media representations to each service location.

16.	The Media Player notifies the Media Session Handler that it is ready to commence playback and optionally provides transport session parameters for those transport sessions terminating at the 5GMSd AS.

17.	The Media Player requests and obtains the initialization information. The Media Player repeats this step for each required initialization segment.

17a.	The CMMF-enabled Access Client requests and obtains, in parallel, unique CMMF-encoded representations of the required initialization information from two or more available service locations.

17b.	Upon reception of enough information from the combination of partially or fully obtained CMMF-encoded objects, these objects are decoded by the CMMF-enabled Access Client, allowing it to recover the required initialization information.

17c.	The CMMF-enabled Access Client provides the decoded initialization information to the Media Playback and Content Decryption Platform as specified in TS 26.511 [26] where it is inserted into the appropriate media rendering pipeline.

18.	The Media Player requests and obtains the Media Segments. The Media Player repeats this step for each required Media Segment according to the Media Player Entry information.

18a.	The CMMF-enabled Access Client requests and obtains, in parallel, unique CMMF-encoded representations of the Media Segment from two or more available service locations.

18b.	Upon reception of enough information from the combination of partially or fully obtained CMMF-encoded objects, these objects are decoded by the CMMF-enabled Access Client, allowing it to recover the required Media Segment.

18c.	The CMMF-enabled Access Client provides the decoded Media Segment to the Media Playback and Content Decryption Platform as specified in TS 26.511 [26] where it is inserted into the appropriate media rendering pipeline.

## X.3.3	Provisioning Session for CMMF-based media streaming

A simplified domain model is depicted in figure X.3.3-1. It consists of a Provisioning Session containing, at a minimum, a Content Hosting Configuration where one or more Distribution Configurations are defined. Each Distribution Configuration is associated with a single service location exposed by the 5GMSd AS at reference point M4d.

The 5GMSd Application Provider may provision one or more Content Preparation Templates that are referenced by the configured Distribution Configurations for the purposes of encoding media resources (e.g., CMAF segments) ingested at reference point M2d into one or more CMMF objects where each CMMF object is a different encoded representation of the ingested media resource.

Different encoded representations of the ingested media resource (each a different CMMF object) are distributed to a 5GMSd Client at reference point M4d via a different service location exposed by the 5GMSd AS. There should be a one-to-one mapping between a CMMF object containing one coded representation of the media resource and the service location exposed by the 5GMSd AS at reference point M4d.

The procedure to provision the features of the 5GMS System for CMMF-based media streaming are described in clause 5.

Figure X.3.3-1: CMMF provisioning domain model