3GPP TSG SA4-MBS SWG AH 	S4aI250140

Paris, France, 03rd – 05th Sep 2025

Title:	[FS_Energy_Ph2_MED]: New service requirements for energy study phase 2

Agenda Item:	8

Source:	Samsung Electronics Co Ltd.

- Introduction

Document SP-250636 was agreed during the SA#108 meeting. This document proposed a phase-2 study on media energy consumption exposure and evaluation framework (FS_Energy_Ph2_MED). The phase 1 study on this topic was completed in SA4 MBS group, and the agreements were documented into TR 26942. Post the completion of phase 1 study, other groups in 3GPP such as SA1 extended their study on this topic and documented more use cases and requirements. This contribution introduces some of those requirements that apply to the phase 2 study in 3GPP SA4, and proposes to include them into existing requirements documented in TR 26942.

- Work in 3GPP SA1 on energy efficiency as service criteria

TR 22883 describes use cases and potential requirements on aspects regarding enahancements on energy as service criteria. This clause describes some of the use cases and requirements in TR 22883 that apply to FS_Energy_Ph2_MED phase 2 study.

  - Energy saving service for UE:

Clause 5.1 of TR 22883 describes a use case on energy saving service for UE. As per the use case description in clause 5.1.1, to achieve enegy efficiency, the operator will need to introduce the concept of energy as a service to provide different energy saving services for subscribers. And for users, when subscribing specific network services (e.g., for AR, XR applications), they will have the opportunities to choose the energy saving services based on their requirement.

Clause 5.1.6 of TR 22883 describes the following new requirements to support the above use case:

[PR.5.1.6-1] The 5G network shall be able to support a mechanism for the subscriber to provide their preference for the energy saving actions to be provided by the network, e.g., by providing energy saving service options to the subscriber. NOTE: The mapping between energy saving service options and network actions is defined by the operator.

  - Dynamic service adjustment support in the network based on energy information:

Clause 5.2 of TR 22883 describes a use case on dynamic service adjustment support in the network based on energy infomation. As per the use case description in clause 5.2.1, the network can utilize energy exposure information (such as type of energy resources, information such as renewable or non-renewable etc.) for energy-aware network decisions towards energy goals (e.g., energy-aware load balancing, choosing functions based on their energy source type).

Clause 5.2.6 of TR 22883 describes the following requirements to support the above use case:

[PR 5.2.6-1] Subject to user consent, operator policy, and regulatory requirements, the 5G network shall be able to provide means to reduce service performance at the flow level in response to energy rationing. NOTE: This assumes that the reduction in service performance will reduce energy consumption in the 5G network. [PR 5.2.6-2] Subject to user consent and regulatory requirements, the 5G network shall allow the network operator to configure policies to reduce service performance at the flow level considering energy rationing.

  - Tolerance to QoS degradation:

Clause 5.7 of TR 22883 describes a use case on tolerance to QoS degradation due to network energy saving. As per the use case description in clause 5.7.1, network energy savings techniques try to optimize energy consumption without QoS degradation, but sometime, the mobile networks may identify further energy savings opportunities at the cost of QoS degradation e.g., reducing the QoS performance of a UE sevice. The use case description further describes that the 5G network can perform actions to reduce service performance in order to save energy, taking into account:

-	The service performance characteristics (e.g. reduced/augmented bitrate, latency, 5QI) that would result from the energy-saving action;

-	The QoS policy associated with the impacted service;

-	Incentives: Charging impacts (e.g. lower energy credit consumption rate, rewarded credits), or future dynamic adjustments of the provided communication service (e.g. temporary improved QoS or service performance);

-	Applicability conditions, which can be (semi-) permanent, or limited in terms of e.g., applicable network slice, time or area, application/service. Such conditions can also refer to explicit actions from UEs and/or users (e.g. move to another location, stay at a given location for a minimal duration).

Clause 5.7.6 of TR 22883 describes the following requirements to support the above use case:

[PR.5.7.6-1] Subject to operator’s policy, regulatory requirements and user consent, the 5G system shall support subscription policies that include tolerance to service performance degradation aimed at network energy saving for services with QoS criteria. NOTE 1: Tolerance corresponds to allowed service performance degradation and applicability conditions (e.g. time, geographical area, slice, application). [PR.5.7.6-2] Subject to operator’s policy, regulatory requirements and user consent, the 5G network shall be able to perform energy saving actions resulting in service performance degradation (e.g. QoS parameters, maximum bitrate) for services with QoS criteria, according to the tolerance to service performance degradation. NOTE 2: This requirement implies that the tolerance could disallow some network energy saving actions to be performed for services with QoS criteria. [PR.5.7.6-3] Subject to operator’s policy and regulatory requirements, the 5G network shall be able to trigger charging events corresponding to an impacted UE when degrading performance of services with QoS criteria due to energy saving. NOTE 3: Such charging events can result in incentives to the subscriber.

  - Postponing content download to times with favourable energy mix and radio signal conditions:

Clause 5.8 of TR 22883 describes a use case on green social media and email content download. As per the description in clause 5.8.1, the use case aims to reduce the carbon impact of instant messaging and email services by postponing the download of attachments (i.e., the bulk of the data) to a moment in time when both the energy mix is and/or the radio signal conditions are favourable. Similarly, other non-time critical data transfers, such as firmware downloads, uploads of social media or email attachments (e.g., recorded videos) and downloads from OTT video providers for offline viewing could be scheduled using the same constraints.

Clause 5.8.6 of TR 22883 describes the following requirements to support the above use case:

[PR.5.8.6-1] The 5G system shall be able to expose information to an Application Function in the network on the (expected) energy related characteristics applicable to the serving site/network. NOTE: This information can be used by applications in the Application Function to schedule non time-critical data transfer to or from the UE.

  - Notifying UEs about network energy-related characteristics:

Clause 5.9 of TR 22883 describes a use case on notifying UEs about network energy-related characteristics. As per the description in clause 5.9.1, these “energy-related characteristics” can be made available to users, UEs or 3rd parties (applications, enterprise customers etc), in order to create awareness also to avoid misinterpretations on potentially limited service availability & performance. The use case further describes that depending on the changes of such characteristics (e.g. energy, load, traffic), dynamic service and performance adjustments of the delivered communication service may happen (including QoS-based services), both from 5G network and UE perspective. For example, some service may be barred or limited and the UE should be properly notified.

Clause 5.9.6 of TR 22883 describes the following requirements to support the above use case:

[PR.5.9.6-1] Subject to operator’s policy, regulatory requirements and user consent, the 5G system shall be able to provide UEs with information related to the current and planned service availability, service performance characteristics and energy-related characteristics of their serving network. NOTE 1: A subset of target UEs can be selected, for example, based on their location, their energy consumption in the network, their service experience, etc. NOTE 2: Planned information can refer to the next hour(s) or day(s), to the remaining serving time before shutdown/shortage etc. [PR.5.9.6-2] Subject to operator’s policy, regulatory requirements and user consent, the 5G system shall be able to support a means to configure and provision policies enabling a UE to take action (e.g. defer background traffic, change access type, disable video service etc) based on the received service availability and/or service performance and energy-related characteristics of the serving network. NOTE 3: The resulting UE actions, if any, can depend on the UE type or class (e.g. MPS/MCS, IoT, smartphone etc). In some cases, the device operating system can be designed to centrally manage all non-time critical traffic to optimize both network and UE battery usage, also by leveraging this information. [PR.5.9.6-3] Subject to user consent, operator policy and regulatory requirements, the 5G network shall be able to assist an authorized 3rd party to identify a set of target UEs for whom to adjust the provided application service, considering criteria such as the current and planned energy-related characteristics of their serving network.

- Work in 3GPP SA1 on Sustainability and Energy Efficiency

TR 22870 describes use cases and potential requirements for 6G. This clause describes some of the use cases and requirements in TR 22870 that apply to FS_Energy_Ph2_MED phase 2 study.

  - End-to-end energy efficiency improvement:

Clause 5.8.1.6 of TR 22870 describes the following requirements:

[PR 5.8.1.6-3] The 6G system shall provide means for a user to provide a preference for UE energy efficiency. NOTE: The 6G network takes into account this preference when improving UE energy efficiency. Examples of preference are user preference on prioritizing service performance, prioritizing UE energy efficiency, or prioritizing essential service (e.g. voice call) while improving UE energy efficiency.

  - Energy efficiency of 6G system with multiple access networks:

Clause 5.8.2 of TR 22870 describes a use case on energy efficiency of 6G system with multiple access networks(TN and NTN). As per the description in clause 5.8.2.1, 6G system is assumed to support both TN and NTN access networks, and the energy consumption associated to each access network may differ for a one-to-many service (e.g. PWS, broadcast, multicast) and the number of targeted users and/or targeted service area.

Clause 5.8.2.6 of TR 22870 describes the following requirements:

[PR 5.8.2.6-1] The 6G network with multiple access networks (e.g. terrestrial, satellite) shall provide means to determine (e.g. through measurement and/or calculation) the energy consumed by the access networks for the provision of a one-to-many service (e.g. PWS, broadcast, multicast) characterised by at least the QoS, targeted service area and/or targeted set of users. [PR 5.8.2.6-2] The 6G network shall be able to support means to provide to the network operator an indication of the energy consumed by the available access networks for the provision of a one-to-many service. NOTE: The network operator may take into account this energy consumption information when deciding how to deliver the service.

  - Energy control at slice level:

Clause 5.8.3 of TR 22870 describes a use case on supporting energy control at slice level. As per the description in clause 5.8.3.1, this use case demonstrates that by providing the 3rd party the ability to define an upper bound on the aggregate quantity of energy consumption for its private slice(s) and a means to limit the energy consumption of its private slice(s), the 6G system supports sustainability.

Clause 5.8.3.5 describes existing features partly or fully covering the use case functionality. Below is an extract from this clause:

Subject to operator’s policy, regulatory requirements and user consent, the 5G network shall support subscription policies that include alternative (i.e. degraded) service performance (e.g. QoS parameters, maximum bitrate) of services with QoS criteria for energy saving reasons.

Clause 5.8.3.6 of TR 22870 describes the following requirements to support above use case:

[PR 5.8.3.6-1] Subject to operator’s policy, the 6G system shall support network slice-specific energy management policies that define a maximum slice energy credit limit for services without QoS criteria. [PR 5.8.3.6-2] Subject to operator’s policy, the 6G system shall support a mechanism to perform slice energy credit limit control for services without QoS criteria.

  - Energy saving for network in industry park:

Clause 5.8.6 of TR 22870 describes a use case on energy saving for network in industry park. As per the description in clause 5.8.6.1, in the 5G phase, UE could select specific energy saving actions, and the associated network activities with the energy saving actions are defined by operator, e.g. modify the network policies, for example, QoS modification, or enable/disable related network function etc. In 6G, considering there will be not only communication service, but also new sensing capability, AI and computing related capabilities, the mechanisms for energy saving could be enhanced and offer better energy efficiency for operator, trusted 3rd party and users.

Clause 5.8.6.6 of TR 22870 describes the following requirements to support above use case:

[PR 5.8.6.6-1] Subject to operator’s policy, regulation and user’s consent, the 6G network shall provide suitable energy saving methods targeting different scenarios (e.g. different working time). [PR 5.8.6.6-2] Subject to regulation and operator’s policy, the 6G network shall be able to expose to a trusted third-party the network energy consumption information including the energy consumption related with sensing, AI, and computing services, over a specific time period (e.g. month etc.)

- Proposal

Following changes are proposed for TR 26942.

# First Change

# 5	Use cases

## 5.1	Baseline use cases defined by SA1

Use cases regarding enhancements to Energy Efficiency of 5G network and application service enabler aspects are listed in TR 22.882 [56]. Five of them have been identified as media-related and therefore fall within the scope of this study:

-	Use case 5.5 on service energy monitoring by an Application Server: The Application Service Provider cares about energy consumption in the Data Network as a result of the service provided by an Application Server to UEs. This could be for one or more of the following three reasons:

-	The Application Service Provider needs to demonstrate that it is reducing energy consumption;

-	The service has an associated energy cost, and the Application Service Provider wants to reduce it;

-	The Application Service Provider recognises that there are policies that limit energy use and controls the overall use of the service to operate within those constraints.

-	Use case 5.6 on supporting service-level energy efficiency analysis for verticals: An Application Service Provider is running three different enterprise applications over two network slices. It proposes exposure of data volume and energy consumption of different Network Functions participating in the delivery of the service for different time periods at the request of the Application Service Provider. The Application Service Provider may use existing 3GPP procedures to infer Network Slice energy consumption and the number of PDU sessions per network slice.

- 	Use case 5.8 on Application service Energy Efficiency (AEE) monitoring: The energy consumed by an application service at the device side as well as at the network side is monitored and predicted by the 5G System and is exposed as a monitoring event to the Application Service Provider to allow an application layer action. In the context of media delivery, this action could be for example triggering multicast/broadcast delivery for a given service area and time of the day.

-	Use case 5.9 on renewable energy consumption information exposure: Mobile Network Operators need to understand and track the proportion of energy consumed in their networks that is sourced from renewable sources, which can be made available to customers and authorized third parties.

-	Use case 5.10 on supporting carbon-aware communication service: The Mobile Network Operator provides to end users an estimate of the carbon emissions for the services consumed, for example the equivalent carbon dioxide emissions corresponding to the data consumed by a user during a particular billing cycle.

-	Use case 5.14 on reducing GHG footprint of Application Services: By considering the temporal and spatial information of sustainable energy source and availability, the possibility of reduction of the greenhouse gas footprint for application services is explored. Rather than optimising compute tasks for highest throughput or lowest latency, those tasks having flexibility in both when and where they are executed (e.g., some AI/ML training or video processing) are routed to a computing node using the (most) sustainable energy sources at that moment.

Use cases and requirements regarding energy efficiency as service criteria are listed in TR 22.883 [22883]. Following have been identified as use cases that fall within the scope of this study:

-	Use case 5.1 on energy saving service for UE: To achieve energy efficiency, the operator will need to introduce concept of energy as service to provide different energy saving services for the subscribers. And the subscribers will have the opportunities to choose one or more of the above energy-saving services based on service requirements

-	The network operator needs to provide energy saving service options to the subscriber

-	The network operator needs to support a mechanism for the subscriber to choose one or more of the energy-saving services, and provide this information to the network

-	Use case 5.2 on dynamic service adjustment support based on energy information: The network can utilize energy exposure information to adjust service experience for energy saving purposes.

-	Network receives energy exposure information such as type of energy sources and types used for delivering the service, for example renewable/non-renewable sources

-	Network takes decisions to reduce service performance at the flow level in response to energy rationing

-	Network may be configured with policies to influence the service performance at the flow level considering energy rationing

-	Network offers users the option to select services with their selected energy sources.

-	Use case 5.7 on tolerance to QoS degradation due to network energy savings: The network can perform energy saving actions that can lead to QoS degradation of services with QoS criteria based on certain tolerance to service performance degradation.

-	Network can be configured with policies that include tolerance to service performance degradation aimed at network energy savings

-	Network can perform energy saving actions that may reduce service performance of services with QoS criteria within the configured tolerance limits

- 	Users may prioritize their service quality preferences based on a trade-off between energy savings and performance.

-	Use case 5.8 on green social media and email content download: The network can postpone delivery of instant messaging content, email content, and other non-critical data to a moment in time when both the energy mix and/or the radion conditions become favorable.

-	Network can expose information to an Application Function in the network on the energy related characteristics applicable to service site/network

-	The Application Function can use energy exposure information to postpone delivery of content to later times when it is more conducive for it to be sent, for example when the energy mix and/or radio conditions are favorable

-	Use case 5.9 on notifying UEs about network energy-related characteristics: The network can notify energy related characteristics to subscribers, UEs and applications for awareness purposes, and also to avoid misinterpretations for potentially reduced service performance (e.g., QoS degradation) at both the network and the UE due to energy saving actions performed by the network

-	Network can provide UE with current and planned service availability, service performance characteristics, and energy related characteristics

-	5G System provides the capabilities to configure and provision policies enabling a UE to act (e.g., defer background traffic, change access type, disable video service etc) in response to receiving energy-related information from the network

Use cases and requirements regarding sustainability and energy efficiency are listed in TR 22.870 [22870]. Following have been identified as use cases that fall within the scope of this study:

-	Use case 5.8.1 on end-to-end energy efficiency improvement: 6G system can enable the user to provide a preference for UE energy efficiency. And the 6G system can use the user preference for energy efficiency information to prioritize service performance, UE energy efficiency, and priorizing essential services.

-	Use case 5.8.2 on energy efficiency of 6G system with multiple access networks (TN and NTN): The 6G system can determine the energy consumed by different access networks for the provisioning of one-to-many services characterised by at least the QoS, targeted service area and/or targeted set of users. The 6G system may consider such energy consumption information across different access networks when deciding how to deliver the service

-	Use case 5.8.3 on energy control at slice level: The 6G system can support sustainability by allowing the 3rd party or an application service provider the ability to define an upper bound on the aggregate quantity of energy consumption for its prive slice(s) and a means to limit the energy consumption of its private slice(s).

-	Network may be configured with policies that include alternative (i.e. degraded) service performance (e.g., QoS parameters, maximum bitrate) of services with QoS criteria for energy saving reasons

-	Network may be condfigured with slice-specific energy management policies that specify a maximum slice energy credit limit for services without QoS criteria

-	Network may perform slice energy credit limit control based on configured energy management policies

-	Use case 5.8.6 on energy saving for network in industry park: The 6G system can provide the UE with energy saving actions and associated network activities e.g. modify the network policies, for example, QoS modification, or enable/disable related network function etc, and the UE can select one or more of those actions. The 6G system can also expose to a trusted third party network energy consumption information related to sensing, AI, and computing services.

Media-related requirements associated with these use cases are addressed in the following Key Issues, complemented by requirements associated with the findings identified in clause 4.

# End of Changes