3GPP TSG-S4 Ad-Hoc Meeting on MTSI	S4aM210667

# Online, 27th October 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.962 | CR | - | rev |  | Current version: |  |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Guidelines on Overlays using MPEG-I Scene Description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | ITT4RT |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | <Cat> |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

## Scene Description-based Overlay

### Scene Description

#### Overview

A scene graph is a directed acyclic graph, usually just a plain tree-structure, that represents an object-based hierarchy of the geometry of a scene. The leaf nodes of the graph represent geometric primitives such as polygons. Each node in the graph holds pointers to its children. The child nodes can among others be a group of other nodes, a geometry element, a transformation matrix, etc.

Spatial transformations are attached to nodes of the graph and represented by a transformation matrix.

This structure of scene graphs has the advantage of reduced processing complexity, e.g. while traversing the graph for rendering. An example operation that is simplified by the graph representation is the culling operation, where branches of the graph are dropped from processing, if deemed that the parent node’s space is not visible or relevant (level of detail culling) to the rendering of the current view frustum.

#### glTF 2.0

glTF 2.0 is a new standard that was developed by Khronos to enable Physically Based Rendering. glTF 2.0 offers a compact and low-level representation of a scene graph. glTF 2.0 offers a flat hierarchy of the scene graph representation to simplify the processing. glTF 2.0 scene graphs are represented in JSON to ease the integration in web environments. The glTF 2.0 specification is designed to elimate redundancy in the representation and to offer efficient indexing of the different objects in the scene graph.

The structure of a glTF 2.0 scene graph document is arranged as follows:

### MPEG-I Scene Description

MPEG has developed an architecture for immersive media, where a Presentation Engine plays a center role. The Presentation Engine is responsible for receiving the scene description and its updates, parsing it, and rendering the scene. In order to access the media required to render the scene, the Presentation Engine invokes functionality provided by the Media Access Function API.

The MPEG-I scene description architecture is depicted by the following figure:

The MPEG solution also specifies a key set of extensions to glTF 2.0 to support timed media such as dynamic objects, video textures, and audio. The MPEG_media, MPEG_accessor_timed, and MPEG_buffer_circular extensions make up the core of these extensions and enable integrating all timed media. The MPEG_texture_video allows the integration of video textures that could for example be the sources of an overlay in an ITT4RT conference. The MPEG_audio_spatial allows for adding spatial audio sources to a scene and binding the listener to the scene’s camera.

The node structure of an MPEG-I scene description is depicted by the following diagram:

The MPEG-I scene description design allows for decoupling media access, which would be provided by the Media Access Function, from the scene rendering, which is provided by the Presentation Engine.

### Scene Description for ITT4RT Sessions

Scene Graphs make it very simple to compose scenes for an immersive conference. The composition may be performed at an MRF or MCU. Alternatively, a designated party in the conference may be selected to be responsible for creating the initial scene description and sharing it with all other participants in the call. This participant may be the one that contributes the main 360 content, for instance, the participant that is in the conference room with VR capture capabilities.

The scene description describes the whole scene, including all audible/visible participants and the main conference room. In an ITT4RT session, the participants may contribute the main 360 content node, one or more overlay nodes, and at most one audio source.

The 360 degree content is described through a sphere or cube-map geometry with associated video texture coming from the 360 video. The type of geometry depends on the selected projection for the ITT4RT session. The overlay nodes are typically rectangular plane regions with an associated video texture coming from the associated overlay video stream.

Unless the initial layout is static, each participant is required to share their actual position and orienation with the scene owner, e.g. MRF/MCU, which in turn will update the scene accordingly and share the updates with all participants that support scene description.

Participants are required to indicate if they support scene description by accepting an SDP offer that contains the data channel, which indicates “mpeg-sd” as the sub-protocol. A participant that does not support scene description will receive an overlay description in the SDP and may declare its overlay streams using the SDP 3gpp-overlay attribute.

In a scene, node names are expected to be unique to ensure there are no naming conflicts in nodes provided by different parties in a call. Nodes in the scene description may reference external media streams, such as other media streams that are declared in the SDP. A participant may mask nodes from certain parties in the rendering process, e.g. based on user input.

### Referencing Media Streams

In order to reference the source video streams for the 360 content and the overlays in a scene description document, the URL format as specified in 23090-14 Annex B is to be used.

The ABNF syntax for the URI format shall be as follows:

url="rtp://" fqdn_or_ip [":" port] "/" session_id "/" ssrc "/" mid

The fqdn_or_ip field contains the address of the source of the stream. The session_id corresponds to the SIP call id, and the ssrc corresponds to the SSRC identifier of the source. The mid corresponds to the mid attribute of the referenced media stream in the SDP.

### Example Scene Description with Overlays

The following example depicts a scene description with a sphere projection 360 video and two overlay streams.

{ "asset": { "version": "2.0" }, "scenes": [ { "name": "Scene", "nodes": [ 0, 1, 2 ] } ], "scene": 0, "nodes": [ { "name": "Sphere", "mesh": 0 }, { "matrix": [ 1, 0,0,0, 0, 1, 0, 0, 0, 0, 1, 0, 0, 1, -3, 1 ], "name": "Overlay1", "mesh": 1 }, { "matrix": [ 1, 0, 0, 0, 0, 1, 0, 0, 0, 0, 1, 0, 0, 1, 3, 1 ], "name": "Overlay2", "mesh": 2 } ], "bufferViews": [ { "buffer": 0, "byteOffset": 0, "byteLength": 6732, "target": 34962, "byteStride": 12 }, { "buffer": 0, "byteOffset": 6732, "byteLength": 6732, "target": 34962, "byteStride": 12 }, { "buffer": 0, "byteOffset": 13464, "byteLength": 4488, "target": 34962, "byteStride": 8 }, { "buffer": 0, "byteOffset": 17952, "byteLength": 5760, "target": 34963 }, { "buffer": 0, "byteOffset": 23712, "byteLength": 48, "target": 34962, "byteStride": 12 }, { "buffer": 0, "byteOffset": 23760, "byteLength": 48, "target": 34962, "byteStride": 12 }, { "buffer": 0, "byteOffset": 23808, "byteLength": 32, "target": 34962, "byteStride": 8 }, { "buffer": 0, "byteOffset": 23840, "byteLength": 12, "target": 34963 }, { "buffer": 0, "byteOffset": 23852, "byteLength": 48, "target": 34962, "byteStride": 12 }, { "buffer": 0, "byteOffset": 23900, "byteLength": 48, "target": 34962, "byteStride": 12 }, { "buffer": 0, "byteOffset": 23948, "byteLength": 32, "target": 34962, "byteStride": 8 }, { "buffer": 0, "byteOffset": 23980, "byteLength": 12, "target": 34963 } ], "buffers": [ { "byteLength": 23992, "uri": "decoded-file.bin" } ], "accessors": [ { "bufferView": 0, "componentType": 5126, "count": 561, "max": [ 5, 5, 5 ], "min": [ -5, -5, -5 ], "type": "VEC3" }, { "bufferView": 1, "componentType": 5126, "count": 561, "max": [ 1, 1, 1 ], "min": [ -1, -1, -1 ], "type": "VEC3" }, { "bufferView": 2, "componentType": 5126, "count": 561, "max": [ 1.015625, 1 ], "min": [ -0.015625, 0 ], "type": "VEC2" }, { "bufferView": 3, "componentType": 5123, "count": 2880, "max": [ 560 ], "min": [ 0 ], "type": "SCALAR" }, { "bufferView": 4, "componentType": 5126, "count": 4, "max": [ 1.5, 0.5, 0 ], "min": [ -1.5, -0.5, 0 ], "type": "VEC3" }, { "bufferView": 5, "componentType": 5126, "count": 4, "max": [ 0, 0, 1 ], "min": [ 0, 0, 1 ], "type": "VEC3" }, { "bufferView": 6, "componentType": 5126, "count": 4, "max": [ 1, 1 ], "min": [ 0, 0 ], "type": "VEC2" }, { "bufferView": 7, "componentType": 5123, "count": 6, "max": [ 3 ], "min": [ 0 ], "type": "SCALAR" }, { "bufferView": 8, "componentType": 5126, "count": 4, "max": [ 1, 0.5, 0 ], "min": [ -1, -0.5, 0 ], "type": "VEC3" }, { "bufferView": 9, "componentType": 5126, "count": 4, "max": [ 0, 0, 1 ], "min": [ 0, 0, 1 ], "type": "VEC3" }, { "bufferView": 10, "componentType": 5126, "count": 4, "max": [ 1, 1 ], "min": [ 0, 0 ], "type": "VEC2" }, { "bufferView": 11, "componentType": 5123, "count": 6, "max": [ 3 ], "min": [ 0 ], "type": "SCALAR" } ], "materials": [ { "pbrMetallicRoughness": { "metallicFactor": 0, "baseColorTexture": { "index": 0 } }, "doubleSided": true, "name": "background" }, { "pbrMetallicRoughness": { "metallicFactor": 0, "baseColorTexture": { "index": 1 } } }, { "pbrMetallicRoughness": { "metallicFactor": 0, "baseColorTexture": { "index": 2 } } } ], "meshes": [ { "primitives": [ { "mode": 4, "attributes": { "POSITION": 0, "NORMAL": 1, "TEXCOORD_0": 2 }, "indices": 3, "material": 0 } ] }, { "primitives": [ { "mode": 4, "attributes": { "POSITION": 4, "NORMAL": 5, "TEXCOORD_0": 6 }, "indices": 7, "material": 1 } ] }, { "primitives": [ { "mode": 4, "attributes": { "POSITION": 8, "NORMAL": 9, "TEXCOORD_0": 10 }, "indices": 11, "material": 2 } ] } ], "extensions": { "MPEG_media": { "media": [ { "alternatives": [ { "mimeType": "video/H265", "uri": "rtp://server.example.com/9879872983/809800/1" } ] }, { "alternatives": [ { "mimeType": "video/H265", "uri": "rtp://server.example.com/9879872983/8034300/2" } ] }, { "alternatives": [ { "mimeType": "video/H265", "uri": "rtp://server.example.com/9879872983/64993/3" } ] } ] } } }