Source:	Samsung Electronics Co., Ltd.

Title:	Proposed corrections on ABNF syntax for ITT4RT

Document for:	Discussion and Agreement

Agenda Item:	4.8 – Others including TEI

- Introduction

At SA4#121, it was agreed to work on the registration of SDP attributes for ITT4RT. More speficifally, we need more work on a complete ABNF syntax, usage level and muxcategory for each attribute to be registred. This contribution provides proposed corrections on ABNX syntaxes of the SDP attributes for ITT4RT.

- Proposed corrections

  - "a=3gpp_360video" attribute

    - Summary of changes

Here is a summary of changes for the syntax of "a=3gpp_360video" attribute in section 2.1.2:

- addition of the syntax for the viewportfb_trigger parameter,

- introducing data elements representing degrees in the unit of 2-16, and

- some editorial changes.

    - Proposed syntax

;Y.6.2.1

att-field =			"3gpp_360video"

att-value =			<payload type>PT [SP FOV] [SP FOV_CENTER] [SP "Stereo"] [SP VDP] [SP viewportfb_trigger]

PT = 1*DIGIT

;sub-rules of FOV from Y.6.2.3

FOV =				"fov=" 1*(fovset)

fovset =			"[x=" azimuthrange ",y=" elevationrange "]"

azimuthrange =		deg-0to360

elevationrange = 	deg-0to180

;sub-rules of FOV_CENTER from Y.6.2.3

FOV_CENTER =		"fov_center=[x=" centerazimuth ",y=" centerelevation "]"

centerazimuth =		deg-minus180to180

centerelevation =	deg-minus90to90

;sub-rules of VDP

VDP =				"VDP" [SP SLVL] [SP Projection] [SP PPM] SP viewport_ctrl SP viewport_size

;from Y.6.2.8

SLVL =				"VL"/"VL,SL"/"SL"

;from Y.6.2.2

Projection =		"projection=" proj-type *("," proj-type)

proj-type =			"ERP" / "CMP"

;from Y.6.2.4

PPM =				"ppm=" ppm-list

ppm-list =			ppm-value *["/"ppm-value]

ppm-value =			"1" / "2" / packing

packing =			"[" PPWHQ "," PPHHQ "," TRHQ "," PPWLQ "," PPHLQ "," TRLQ "]"

PPWHQ	=			pos-integer

PPHHQ =				pos-integer

TRHQ =				transform-value

PPWLQ =				pos-integer

PPHLQ =				pos-integer

TRLQ =				transform-value

transform-value =	"0"/"1"/"2"/"3"/"4"/"5"/"6"/"7"	; transform values as per Table Y.6.1

;from Y.6.2.5

viewport_ctrl =		"viewport_ctrl=" vc-value *2 ("," [SP] vc-value)

vc-value =			"0" / "1" / "2"			; viewport control values as per Table Y.6.2

;from Y.6.2.7

viewport_size =		"viewport=" azimuthrange"x"elevationrange

;from Y.6.2.9

viewportfb_trigger =	"<["D "|" D_azimuth","D_elevation"]>"

D =					deg-0to180minus1

D_azimuth =			deg-minus180to180

D_elevation =		deg-minus90to90

;where D_azimuth, D_elevation are the minimum number of degrees that the viewport may change

;in the horizontal or vertical direction, respectively.

;The values for D shall be in the range 0 to 180 * 2^16 − 1 (i.e., 11 796 479).

pos-integer =		POS-DIGIT *DIGIT

POS-DIGIT = 			%x31-39		;1-9

;from Y.6.2.3

azimuthrange

deg-0to360 =		"0"

/ POS-DIGIT *6DIGIT

/ "1" 7DIGIT

/ "2" ("0"/"1"/"2") 6DIGIT

/ "23" ("0"/"1"/"2"/"3"/"4") 5DIGIT

/ "235" ("0"/"1"/"2"/"3"/"4"/"5"/"6"/"7"/"8") 4DIGIT

/ "2359" ("0"/"1") 3DIGIT

/ "23592" ("0"/"1"/"2"/"3"/"4"/"5"/"6"/"7"/"8") 2DIGIT

/ "235929" ("0"/"1"/"2"/"3"/"4"/"5") DIGIT

/ "23592960"

; 0 to 23 592 960, inclusive

elevationrange

deg-0to180 =		"0"

/ POS-DIGIT *6DIGIT

/ "10" 6DIGIT

/ "11" ("0"/"1"/"2"/"3"/"4"/"5"/"6") 5DIGIT

/ "117" ("0"/"1"/"2"/"3"/"4"/"5"/"6"/"7"/"8") 4DIGIT

/ "1179" ("0"/"1"/"2"/"3"/"4"/"5") 3DIGIT

/ "11796" ("0"/"1"/"2"/"3") 2DIGIT

/ "117964" ("0"/"1"/"2"/"3"/"4"/"5"/"6"/"7") 1DIGIT

/ "11796480"

; 0 to 11 796 480, inclusive

centerazimuth

deg-minus180to180 =		"0"

/ ["-"] POS-DIGIT *6DIGIT

/ ["-"] "10" 6DIGIT

/ ["-"] "11" ("0"/"1"/"2"/"3"/"4"/"5"/"6") 5DIGIT

/ ["-"] "117" ("0"/"1"/"2"/"3"/"4"/"5"/"6"/"7"/"8") 4DIGIT

/ ["-"] "1179" ("0"/"1"/"2"/"3"/"4"/"5") 3DIGIT

/ ["-"] "11796" ("0"/"1"/"2"/"3") 2DIGIT

/ ["-"] "117964" ("0"/"1"/"2"/"3"/"4"/"5"/"6"/"7") 1DIGIT

/ "-11796480"

; -11 796 480 to 11 796 479, inclusive

centerelevation

deg-minus90to90 =	"0"

/ ["-"] POS-DIGIT *5DIGIT

/ ["-"] ("1"/"2"/"3"/"4") 6DIGIT

/ ["-"] "5" ("0"/"1"/"2"/"3"/"4"/"5"/"6"/"7") 5DIGIT

/ ["-"] "58" ("0"/"1"/"2"/"3"/"4"/"5"/"6"/"7"/"8") 4DIGIT

/ ["-"] "589" ("0"/"1"/"2"/"3"/"4"/"5"/"6"/"7") 3DIGIT

/ ["-"] "5898" ("0"/"1") 2DIGIT

/ ["-"] "58982" ("0"/"1"/"2"/"3") DIGIT

/ ["-"] "5898240"

; -5 898 240 to 5 898 240, inclusive

deg-0to180minus1 =	"0"

/ POS-DIGIT *6DIGIT

/ "10" 6DIGIT

/ "11" ("0"/"1"/"2"/"3"/"4"/"5"/"6") 5DIGIT

/ "117" ("0"/"1"/"2"/"3"/"4"/"5"/"6"/"7"/"8") 4DIGIT

/ "1179" ("0"/"1"/"2"/"3"/"4"/"5") 3DIGIT

/ "11796" ("0"/"1"/"2"/"3") 2DIGIT

/ "117964" ("0"/"1"/"2"/"3"/"4"/"5"/"6"/"7") 1DIGIT

; 0 to 11 796 479, inclusive

  - "a=3gpp_overlay" attribute

    - Summary of changes

Here is a summary of changes for the syntax of "a=3gpp_overlay" attribute in section 2.2.2:

- addition of the syntax for the free_overlay and the overlay_overlap_flag parameters,

- addition of sub-rules for the sphere_relative_over_config and the viewport_relative_overlay_config parameters, and

- some editorial changes.

Note: the syntax for the media_alignment in the viewport_relative_overlay_config parameter is given as TBD. It can be a 4-tuple representing the intended scaling and the intended placement of the overlay source, for example <width_percent, height_percent, left_percent, top_percent>. The semantic for the media_alignment is as follows:

Media_alignment: Specifies the default intended scaling of the overlay source  depending on the dimensions of the specified rectangular region and the intended placement of the scaled overlay source relative to the specified rectangular region.

    - Proposed syntax

a = 3gpp_overlay: overlay_id SP type SP (sphere_relative_overlay_config / viewport_relative_overlay_config) [ SP overlay_info]

att-field =			"3gpp_overlay"

att-value =			[free_overlay SP] overlay_id SP type SP (sphere_relative_overlay_config / viewport_relative_overlay_config) [ SP overlay_info] [SP overlay_overlap_flag]

free_overlay =		"free_ovelay=" BIT

overlay_id = 		token

; token is defined in RFC 4566

type =				BIT

overlay_info =	"overlay_info=" 5BIT	;from Y.6.4.3.4

overlay_overlap_flag = "overlap=" BIT

;sub-rules for sphere_relative_overlay_config from Y.6.4.3.2

sphere_relative_overlay_config = Overlay_azimuth "," Overlay_elevation "," Overlay_tilt "," Overlay_azimuth_range "," Overlay_elevation_range "," Overlay_rot_yaw "," Overlay_rot_pitch "," Overlay_rot_roll "," region_depth_minus1 "," timeline_change_flag

Overlay_azimuth =			deg-minus180to180

Overlay_elevation =			deg-minus90to90

Overlay_tilt =				deg-minus180to180

Overlay_azimuth_range =		deg-0to360

Overlay_elevation_range =	deg-0to180

Over_rot_yaw =				deg-minus180to180

Over_rot_pitch =			deg-minus90to90

Over_rot_roll =				deg-minus180to180

region_depth_minus1 =		pct-in2tominus16

timeline_change_flag =		BIT

;sub-rules for viewport_relative_overlay_config from Y.6.4.3.3

viewport_relative_overlay_config = Overlay_rect_left_percent "," Overlay_rect_top_percent "," Overlay_rect_width_percent "," Overlay_rect_height_percent "," Relative_disparity_flag "," (Disparity_in_percent / Disparity_in_pixels) "," media_alignment "," layering_order "," opacity "," overlay_priority”

Overlay_rect_left_percent =		pctIn2ToMinus16

Overlay_rect_top_percent =		pctIn2ToMinus16

Overlay_rect_width_percent =	pctIn2ToMinus16

Overlay_rect_height_percent =	pctIn2ToMinus16

Relative_disparith_flag =		BIT

Disparity_in_percent =			pctIn2ToMinus16

Disparity_in_pixels =			integer

; media_alignment = 	TBD (scaling and placement)

layering_order =				pos-integer

opacity =						"0"/pos-integer

overlay_priority =				pos-integer

integer =						"0"/["-"]pos-integer

pct-in2tominus16 =			"0"

/ POS-DIGIT *4DIGIT

/ "6" ("0"/"1"/"2"/"3"/"4") 3DIGIT

/ "65" ("0"/"1"/"2"/"3"/"4") 2DIGIT

/ "655" ("0"/"1"/"2") 1DIGIT

/ "6553" ("0"/"1"/"2"/"3"/"4"/"5")

; 0 to 65 535, inclusive

  - Other attributes

The ABNF syntaxes for other attributes including the "a=3gpp_fisheye" and the "3gpp-camera-calibration" attributes can be updated to reuse data elements such as pos-integer, POS-DIGIT, deg-minus180to180, deg-minus90to90 and etc.

- Proposal

We propose to use the contents of the section 2 as a basis for a CR on ABNF syntaxes for SDP attributes for ITT4RT and work on the registration of the SDP attributes.