Source:	China Mobile Com. Corporation

Title:	[IBACS] Generic Call Flow for an Audio-Driven Avatar AR Call

Agenda item:	4.5

Document for:	Discussion and Agreement

- Introduction

In the current IBACS PD, the avatars in an AR Call are animated using image-driven methods. However, certain scenarios may arise where users prefer not to turn on their cameras during the call due to privacy concerns or the excessive data usage associated with video streaming. To address this, this contribution introduces a generic call flow for an audio-driven avatar call as a complementary method.

- Audio-Driven Animated Avatar

The 3D model can also be animated based on the user's real-time speech. State-of-the-art methods can take the user’s raw audio as input and generate full body animation (2D video or 3D mesh animation) of the avatar, including facial expression, hand gestures, and body motions. The specific method for generating speech-to-motion data may vary depending on the implementation and is beyond the scope of this work. As an example, one possible method is provided here. The sender or MRF can utilize a pre-trained neutral network model that maps audio signals to the motion data (e.g., 3D vertex coordinates of the face mesh, 2D/3D joints of the hands and body) of the 3D model.

*Figure.1 Audio-driven avatar with full body animation[1].

- Call Flow

Figure 4.2.2.x: Call flow for audio-driven avatar based AR call with MRF assistance for model loading and animation

- The Enhanced MRF will loads a 3D model for UE1 for preparing the avatar call.

- UE1 captures and processes the caller’s speech audio input in real-time (e.g. noise reduction). UE1 will send the processed audio input to the Enhanced MRF as an audio stream.

- The Enhanced MRF generates full body animation for the 3D model, including facial expressions and motion signals (e.g., 3D vertex coordinates, pose parameters, and rotation matrix).

- The Enhanced MRF animates the 3D model based on expression or motion signals.

- The Enhanced MRF delivers the 3D model and audio data to UE2.

- UE2 renders the animated 3D model with user view, and synchronizes avatar animations with the received audio.

- Proposal

We propose to include the first paragraph of section 2 of this document into section 4.2 of the IBACS PD; include section 3 of this document into section 4.2.2 of the IBACS PD.

- References

- Yi, Hongwei and Liang, Hualin and Liu, Yifei and Cao, Qiong and Wen, Yandong and Bolkart, Timo and Tao, Dacheng and Black, Michael J, "Generating Holistic 3D Human Motion from Speech," 2023 IEEE/CVF Conference on Computer Vision and Pattern Recognition (CVPR), Vancouver, Canada, 2023, pp. 469-480.

- Y. Fan, Z. Lin, J. Saito, W. Wang and T. Komura, "FaceFormer: Speech-Driven 3D Facial Animation with Transformers," 2022 IEEE/CVF Conference on Computer Vision and Pattern Recognition (CVPR), New Orleans, LA, USA, 2022, pp. 18749-18758, doi: 10.1109/CVPR52688.2022.01821.