3GPP TSG-SA WG4 Ad-hoc Meeting Post #131-bis-e			S4aR250102

Online, 7th of May 2025

Source:	Lenovo

Title:	[5G_RTP_Ph2] Discussion on handling ETI for RTP-based media delivery

Agenda item:	4.6

Document for:	DISCUSSION

# Introduction

In SA4#131-bis-e the Expedited Transfer Indication (ETI) feature was discussed. One issue identified was the marking requirements on RTP senders for ETI signaling given Stage-2 procedures for Packet Detection Rules (PDR) of ETI and corresponding Packet Detection Information (PDI) used for packet matching. It was deferred to further study whether the presence of ETI marking is always necessary when ETI feature is applied for real-time media delivery over RTP. The corresponding details may be in turn used for optimizing the RTP header extension ETI signal in the user plane.

# PDR requirements for ETI matching

As defined in TS 23.501 [1], clause 5.37.10.3, for enabling ETI with Reflective QoS for non-GBR QoS flows an AF, such as an RTC AF, can request the network (e.g., by NEF/PCF existing APIs) for an AS session with QoS requirements. The request includes two media flows, with same SDF filter, but different QoS requirements corresponding to:

- #1. Elevated QoS requirements mapping to an ETI value of True

- #2. “Non-elevated” QoS requirements mapping to an ETI value of False.

Furthermore, in case of RTP-based traffic, the AF can further provide information to enable the network, i.e., the UPF, detect the ETI signal in user plane at N6. Such auxiliary information for detection of ETI includes extended Protocol Description information, e.g., indicating the presence of ETI in an RTP header extension

ETI is supported only when the UE supports Reflective QoS and otherwise not. When the UE supports Reflective QoS and and ETI is requested by the AF to the network (as summarized above), then two PDRs are configured for the session at the UPF

- #1. PDR with PDI ETI = True and QER set to an elevated QoS flow and reflective QoS enabled

- #2. PDR with PDI ETI = False and QER set to a “non-elevated” QoS flow and reflective QoS optional, up to operator configuration.

When configured with the ETI PDRs, the UPF is required to inspect the N6 DL traffic, detect and route PDUs to the appropriate QoS flow based on the PDR matching. As such, the following applies for each packet at N6 of the flow for which ETI is desired:

- 1. In case a PDU is marked with ETI = True, the UPF matches the PDU to the PDR with ETI = True and routes it to the elevated QoS flow while setting the RQI flag downstream for the matched PDU.

- 2. In case a PDU is marked with ETI = False, the UPF matches the PDU to the PDR with ETI = False and routes it to the “non-elevated” QoS flow.

- 3. In case a PDU is not marked with ETI (either True or False), the UPF does not match the PDU to either PDR with ETI = True or ETI = False. The UPF proceeds with further PDR matching for the PDU as per configuration for the session. The ETI-unmarked PDU may so be matched to a best effort, non-GBR, default QoS flow, which may potentially differ in QoS Enforcement Rule (QER) than the “non-elevated” QoS flows.

The key question herewith is whether the network behavior described above is the expected one from the perspective of an RTP sender/application when ETI is enabled for RTP-based traffic?

The current ETI PDR matching behavior, as above, may therefore present gaps in handling typical RTP application traffic, including multiplexing, as some PDUs cannot be marked with ETI signal. This is for instance the case in practice when RTP media streams are multiplexed on the same 5-tuple with RTCP, STUN, and even DTLS/SCTP when WebRTC is in use (e.g., as per fully bundled mandatory configuration of WebRTC endpoints in IETF RFC 8835). As consequence of multiplexing, ETI-unmarked PDUs will not match any of ETI PDR rules and might be mishandled from a QoS perspective, potentially downgraded to an even lower QoS than the “non-elevated” one.

Observation 1: The current ETI matching behavior including two PDRs, one with matching criterion ETI = True and a second one with matching criterion ETI = False is not well applicable to RTP traffic where on the same 5-tuple multiple RTP streams may be multiplexed together with other protocols (e.g., RTCP, STUN, DTLS/SCTP) that cannot be marked with the ETI signal.

# Desired ETI behaviour

As detailed in Solution 16 of TR 23.700-70 [2], the ETI feature is primarily intended for interactive and conversational XR media services as a dynamic and temporary boost for UL traffic (which is often data rate limited), but also applicable to DL media traffic, if needed. Thus, from an RTC service viewpoint, the ETI feature is intended to provide in user plane a dynamic indication of a temporary and symmetric data boost of UL and DL to a desired elevated QoS such that for RTP traffic:

If RTP HE with ETI marking is present and ETI signal is set to True value,

then data boosting is True and elevated QoS flow is used;

If RTP HE with ETI marking is absent or RTP HE is present and ETI signal is set to False value,

then data boosting is False and “non-elevated”/default QoS flow is used;

NOTE: RTP HE with ETI marking is used as a generic term in the current discussion and does not preclude carrying of the ETI marking as either part of an existing RTP HE defined by TS 26.522 or a separate RTP HE to be further defined. The intention of present document is not to discuss the user plane signaling design of the RTP HE, but establish the basis for doing so, by clarifying issues raised in past meetings.

Observation 2: The desirable behavior from a service provider perspective for ETI needs to be simple and effective for typical RTP traffic configurations in RTC media delivery, such that

If RTP HE with ETI marking is present and ETI signal is set to True value,

then data boosting is enabled and elevated QoS flow is used;

If RTP HE with ETI marking is absent or RTP HE is present and ETI signal is set to False value,

then data boosting is not enabled and “non-elevated”/default QoS flow is used;

# Achieving the desired ETI behaviour

Despite the lack of ETI PDR optimization for typical RTP traffic as per Observation #1, the guiding principle of ETI operation as captured in Observation #2 can still be achieved, such that RTP senders are not required to mark with ETI signal RTP packets that are not to be boosted or packets that cannot be marked.

A potential approach (with minimal impacts on existing Stage 2 agreements [1], [4]) would be in this case to have a third PDR, i.e., a fallback PDR, for the ETI-unmarked packets, i.e., RTP packets that should not be boosted or cannot be marked with RTP HE with ETI marking. To effectively use a fallback PDR, the PDR key operation principles need be preserved, as per clause 5.2.1 of TS 29.244 [3]:

- No exact duplicate PDIs in two or more PDRs

“the CP function shall not provision more than one PDR with the same match fields in the PDI (i.e. with the same set of match fields and with the same value). The CP function may provision PDRs with the same value for a subset of the match fields of the PDI but not all;”

- Overlap of subset of PDIs match fields is allowed

“different PDRs of a same PFCP session may overlap, e.g. the CP function may provision two PDRs which differ by having one match field set to a specific value in one PDR and the same match field not included in the other PDR (thus matching any possible value);”

- Same PDR precedence and packet matching rules apply

“A packet matches a PDR if all the match fields which are identified with different IE type in the PDI of the PDR are matching the corresponding packet header fields unless specified otherwise. If a match field is not included in the PDI, it shall be considered as matching all possible values in the header field of the packet. If the match field is present and does not include a mask, the match field shall be considered as matching the corresponding header field of the packet if it has the same value. If the match field is present and includes a mask (e.g. IP address with a prefix mask), the match field shall be considered as matching the corresponding header field of the packet if it has the same value for the bits which are set in the mask. If a match field has multiple instances, i.e. there are several IEs with the same IE type, a packet matches this match field if any instance is matching the corresponding packet header field.”

The main goal of the fallback PDR is to ensure that ETI-unmarked PDUs are treated by the UPF same to the ETI-marked PDUs with ETI signal set to False. As such the following attributes apply to the fallback PDR:

- Fallback PDR is same to the ETI PDR corresponding to the ETI value of False, except the ETI matching field in the PDI which is missing and the precedence value which is different but higher (i.e., lower precedence) than the precedence value set for the ETI PDR corresponding to the ETI value of True.

Proposal 1: A fallback PDR could be used to packets that are not marked with the ETI signal. The fallback PDR is same to the ETI PDR corresponding to the ETI value of False, except a missing ETI matching field in the PDI and a different precedence, lower than the precedence of the ETI PDR corresponding to the ETI value of True.

NOTE 1: It is highly advisable for ETI support in case of RTP traffic not to enable Reflective QoS control for the ETI PDR corresponding to the ETI value of False. This avoids undesired cancelling of boosted UL packets due to non-elevated Reflective QoS triggering sourced by DL packets marked either with ETI = False or that cannot be marked with RTP HE with ETI marking.

NOTE 2: The precedence condition is fulfilled based on clause 6.1.3.27.9 of TS 23.503 [4] which states that if the PCF does not enable Reflective QoS control in the PCC rule with Expedited Transfer Indication set to FALSE, then the PCF shall set the precedence of the PCC rule to a value higher than the reserved range for PCC rules subject to Reflective QoS. A higher precedence value means a lower precedence given the inverted encoding of precedence.

The usage of the fallback PDR according to the proposal above is in the same vein and complements existing Stage 2 specification of TS 23.501 [1], clause 5.37.10.3 for ETI PDRs applicable to RTP-based media delivery. Furthermore, the fallback PDR principle could be implemented two-fold:

- As Operator-controlled based on policy implementation, where an additional PCC rule is generated as fallback to the PCC rule associated with ETI value set to False.

- Or as ASP-controlled based on an AF request of an additional media flow with the same SDF filter as the ones used for ETI, and same QoS requirements as for ETI set to False, but no ETI configuration flag indication (or alternatively Protocol Description comprising RTP HE with ETI marking). This would map for an RTC AF of the 5G RTC system to configuring a third applicationFlowBinding without an ETI configuration flag for the same SDF filter as the applicationFlowBindings corresponding to ETI set to True and ETI set to False, and same QoS requirements as for the applicationFlowBinding with ETI set to False.

To enable consistent and interoperable system behavior as well as ASP control, the second approach is favorable in the context of 5G RTC subsystem.

Proposal 2: Consider the ASP-controlled additional media flow configuration to trigger the fallback PDR setup at the UPF.

NOTE 3: If the approach above is not agreeable as a way forward and using always only the two ETI PDRs is desired, then either

- an RTP sender is required to mark ETI signal in all packets, or

- an alignment LS to SA2 is necessary to highlight ETI gaps with RTP-based media delivery and request expanding the matching semantics of the PDR with ETI set to False to match both PDUs marked with ETI = False and ETI-unmarked PDUs (i.e., simply put, ETI = False in PDI matches PDUs with either False or missing ETI signal).

# Proposal

It is proposed to discuss and agree a way forward on the principles of operation of ETI for RTP-based media delivery starting from:

Observation 1: The current ETI matching behavior including two PDRs, one with matching criterion ETI = True and a second one with matching criterion ETI = False is not well applicable to RTP traffic where on the same 5-tuple multiple RTP streams may be multiplexed together with other protocols (e.g., RTCP, STUN, DTLS/SCTP) that cannot be marked with the ETI signal.

Observation 2: The desirable behavior from a service provider perspective for ETI needs to be simple and effective for typical RTP traffic configurations in RTC media delivery, such that

If RTP HE with ETI marking is present and ETI signal is set to True value,

then data boosting is enabled and elevated QoS flow is used;

If RTP HE with ETI marking is absent or RTP HE is present and ETI signal is set to False value,

then data boosting is not enabled and “non-elevated”/default QoS flow is used;

Proposal 1: A fallback PDR could be used to packets that are not marked with the ETI signal. The fallback PDR is same to the ETI PDR corresponding to the ETI value of False, except a missing ETI matching field in the PDI and a different precedence, lower than the precedence of the ETI PDR corresponding to the ETI value of True.

Proposal 2: Consider the ASP-controlled additional media flow configuration to trigger the fallback PDR setup at the UPF.

# References

[1] 3GPP TS 23.501: System architecture for the 5G System (5GS); Stage 2, version 19.3.0.

[2] 3GPP TS 23.700-70: Study on architecture enhancement for Extended Reality and Media service (XRM); Phase 2, version 19.0.0.

[3] 3GPP TS 29.244: Interface between the Control Plane and the User Plane Nodes; Stage 3 (Release 19)

[4] 3GPP TS 23.503: Policy and charging control framework for the 5G System (5GS); Stage 2, version 19.3.0

[5] C4-251349: CR960r1 to 29.244 on Expedited Data Transfer with reflective QoS, Status: Agreed