3GPP TSG-SA WG4 Ad-hoc Meeting Post #131-bis-e			S4aR250103

Online, 7th of May 2025

Source:	Lenovo

Title:	[5G_RTP_Ph2] Discussion on marking ETI in RTP header extension

Agenda item:	4.6

Document for:	DISCUSSION

# Introduction

In SA4#131-bis-e the Expedited Transfer Indication (ETI) feature was discussed with a proposal for ETI signal marking in RTP HE for dynamically changing traffic characteristics in S4-250511 [noted]. Consensus could not be reached based on differences in understanding the marking requirements of ETI in RTP HE (i.e., all packets vs. only boosted packets vs. only subset of boosted packets).

This discussion paper presents a concise analysis of approaches in marking ETI in RTP HE and proposes a way forward.

# Analysis of options to mark ETI signal in RTP HE

As detailed in S4aR250102,

- packets that need be boosted by the network always need to carry the ETI signal set to a True codepoint in an RTP HE.

- packets that do not need be boosted by the network are

  - either marked with the ETI signal set to a False codepoint in the RTP HE matching them to the PDR associated with ETI set to False, and mapping to the non-elevated QoS flow

  - or not marked with an ETI signal, thus mapping to a “default” QoS flow.

Following up on offline discussions summarized by S4-250740 [noted], the current options to add ETI signal to RTP HE are either reusing a spare 1 bit in one of the existing RTP HE (i.e., RTP HE for PDU Set marking or RTP HE for dynamically changing traffic characteristics), or creating a new dedicated RTP HE for Expedited Transfer marking.

Table below summarizes the available options with their corresponding pros and cons.

| Option | Pros | Cons |
| --- | --- | --- |
| Option #1: Reuse spare 1 bit in the RTP HE for PDU Set marking | - Low ETI signalling overhead, if PDU Set marking is also in use - No need for IANA registration | - Mandatory dependency of Expedited Data Transfer feature to PDU Set handling feature - Cannot mark with ETI non-RTP packets |
| Option #2.1 (fixed RTP HE): Reuse spare 1 bit in the RTP HE for dynamically traffic characteristics (in current fixed design) | - Low ETI signalling overhead, if both BSize and TTNB are signalled - Simple fixed signalling design - No need for IANA registration of additional RTP HE | - Large ETI signalling overhead, if BSize and/or TTNB are not signalled given the fixed design of the RTP HE and ETI signalling requirements. - Unclear benefits of fixed bundling of the Expedited Data Transfer feature to any of the other dynamic traffic characteristics features, i.e., BSize and TTNB - Frequency/occurrence requirements of signalling ETI (esp. ETI = True) seems to not blend well with recommended frequency/occurrence of signalling BSize and/or TTNB - Cannot mark with ETI non-RTP packets |
| Option #2 (flexible RTP HE): Use a byte field in the RTP HE for dynamically traffic characteristics (with an extensible design) (NOTE 1, NOTE 2) | - Moderate ETI signalling overhead (at most 3 bytes) even if BSize and/or TTNB are not signalled - Frequency/occurrence of signalling ETI (esp. ETI = True) may be decoupled from the frequency/ occurrence of signalling BSize and/or TTNB - No need for IANA registration of additional RTP HE | - More complex signalling design incl. optional fields - Unclear benefits of designing signalling Expedited Data Transfer together with any of other dynamic traffic characteristics features, i.e., BSize and TTNB - Cannot mark with ETI non-RTP packets |
| Option #3 Fixed separate RTP HE for Expedited Transfer marking. | - Moderate ETI signalling overhead (at most 3 bytes) - Simple, standalone and fixed signalling design | - Need for IANA registration of an additional RTP HE - Cannot mark with ETI non-RTP packets |

Table 1: Analysis of pros and cons associated with marking ETI in different RTP HEs.

NOTE 1: An extensible design approach to the RTP HE for dynamically traffic characteristics may be advisable given that the individual dynamic traffic characteristics signals, i.e., data burst size (BSize), time to next burst (TTNB) and expedited transfer indication (ETI) may not share same RTP sender requirements (e.g., frequency/occurence of transmission, time synchronization etc.). In addition, such features may not be practically combined with a clear value add benefit, e.g., ETI signals may not be easily combined with BSize and/or TTNB without making application logic assumptions which should not be object to standardization.

NOTE 2: A possible extensible design of RTP HE for dynamically traffic characteristics can be inspired from RTP HE for PDU Set marking and can preserve current encoding and semantics of existing information fields (i.e., BSize and TTNB). For example, the RTP HE for dynamically traffic characteristics marking can optionally include any of BSize, TTNB or ETI byte fields while the wire format is indicated in the SDP offer/answer procedure by corresponding extension attributes. Some illustrative examples of possible (one-byte) RTP HE for dynamically traffic characteristics marking with various optional fields enabled are presented below:

- RTP HE for dynamically changing traffic characteristics with BSize only

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       0xBE    |    0xDE       |           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  ID   | len   |           BSize                               |

+-+-+-+-+-+-+-+-+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+

- RTP HE for dynamically changing traffic characteristics with TTNB only

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       0xBE    |    0xDE       |           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  ID   | len   |           TTNB                |

+-+-+-+-+-+-+-+-+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+

- RTP HE for dynamically changing traffic characteristics with BSize and TTNB

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       0xBE    |    0xDE       |           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  ID   | len   |           BSize                               |

+-+-+-+-+-+-+-+-+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+

|           TTNB                |

+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+

- RTP HE for dynamically changing traffic characteristics with ETI byte-field only (e.g., B encodes the ETI Boolean value while rest of the bits are reserved)

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       0xBE    |    0xDE       |           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  ID   | len   |      R      |B|

+-+-+-+-+-+-+-+-+.+.+.+.+.+.+.+.+

- RTP HE for dynamically changing traffic characteristics with all BSize, TTNB and ETI

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       0xBE    |    0xDE       |           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  ID   | len   |      R      |B|           BSize

+-+-+-+-+-+-+-+-+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+

|           TTNB                |

+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+

# Proposal

Given Table 1 above, and the fact that data burst size, time to next burst and expedited data transfer are already specified as independent dynamic traffic characteristics features in existent Stage 2, TS 23.501, and corresponding Stage 3 NEF/PCF interfaces (TS 29.122, TS 29.522, TS 29.514), it is proposed to:

- Consider and define in TS 26.522 a standalone fixed RTP HE for Expedited Transfer marking.