Agenda item: 	4.6

Source: 	Qualcomm Inc.

Title: 	[5G_RTP_Ph2] The need for redefining Time to the Next Data Burst based on experimental results

Document for	Agreement

# Introduction

Time to next data burst (TTNB) may be beneficial for UE power saving, but the definition should be also considered from the operation’s perspective – whether there is a way to signal it so that the benefit materializes.

The term of TTNB originated from SA4. Since then, it has been defined in SA2 in TS23.501 as follows:

The time to next data burst, … is the interval between the transmission of the last PDU in the current data burst and the first PDU of the next data burst

Regarding the accuracy of TTNB indication, in R2-2407733 [1], RAN2 stated:

Regarding Question 2 on time to next burst, RAN2 thinks that time to next burst may be useful for the network scheduling for downlink, if it is provided in advance and is reliable and accurate at RAN.

In the 5G_RTP_Ph2 WID, SA4 agreed to indicate TTNB in the RTP header extension for dynamically changing traffic characteristics on the timeliness and accuracy of the indication:

- Conduct normative work on burst size, time to next burst, data boosting indication and the definition of data burst.

NOTE: RAN2 has indicated that TTNB may be useful if provided in time and is reliable. SA4 needs further evaluation before proceeding with normative work.

The timeliness requirement obliges the RTP sender to indicate the TTNB before the next data burst is generated, and this calls for prediction.

There was disagreement within SA4 about the meeting of “reliable and accurate”. SA4 sent an LS to RAN2 in SA4 #131-bis-e meeting to RAN2 for clarification and received the reply in R2-2504812 [2]. In the rest of the paper, we look at the consequence (in terms of prediction errors) if the SA2 definition of TTNB were adopted, using the experimental data from S4-241835 [3], and propose to change the definition of TTNB or use a new term altogether in order to meet the accuracy requirement from RAN2.

# RAN2 requirement on TTNB accuracy

In R2-2504812 [2] RAN2 provided the following feedback on the requirement of the TTNB indication accuracy:

RAN2 thanks SA4 for the question on the preferred accuracy of the TTNB indication.

The TTNB can be used by RAN in its scheduling and configurations to achieve benefits such as UE power savings. Therefore, the more accurate the indicated TTNB is (up to a practical limit and considering the impact of factors such as network jitters), the more benefits (e.g. UE power savings) can be achieved. More specifically, from RAN2’s perspective, the desired accuracy of the TTNB indication equals the shortest PDCCH skipping duration, which is 0.125 msec (i.e. one slot for 120 KHz SCS). If this level of accuracy is not feasible with SA4, then the most accurate indication that SA4 can practically support is acceptable to RAN2.

# RAN2 clearly expects the TTBN indication to be as accurate as possible, up to 0.125msec.

# Prediction results

We present prediction results for the packet traces provided in S4-241835 [3] if the SA2 definition of TTNB were adopted. The prediction uses exponential weighted moving algorithm (EWMA) with the optimal weight found through exhaustive search. The prediction for the kth TTNB is calculated as

pk=αpk-1+(1-α)tk-1

where pk-1 is the previous predicted TTNB, and tk-1is the previous measured TTNB, and α is the weight in the range from 0 to 1.

The Python script in [3] produces two TTNB sequences, saved as a_b_bursts_windows_duplex.csv and b_a_bursts_windows_duplex.csv. In what follows, we present the predictions for them separately.

2.1 The first TTNB sequence (a_b_bursts_windows_duplex.csv)

The optimal weight of EWMA predictor is found by searching for the weight that minimizes the mean absolute prediction error over the entire first TTN sequence. The optimal EWMA is 0.82, as shown in Figure 1.

Figure 1 Mean absolute prediction error vs the WEMA weight for the first TTNB sequence

The measured TTNB (i.e., actual TTNB) vs the predicted TTNB for the first TTNB sequence is shown in Figure 2. It is seen that although the predicted TTNB well tracks the trends of the measured TTNB, it often over-estimates or under-estimates the measured TTNB with a significant prediction error.

Figure 2 The measured TTNBs and the predicted TTNBs for the first TTNB sequence

The cumulative distribution function (CDF) of the prediction error, i.e., the predicted TTNB minus the measured TTNB, is shown in Figure 3. It is seen that the prediction over-estimates by 14.7ms 10% of the time, and under-estimates by 19ms 30% of the time.

Figure 3 CDF of the prediction error for the first TTNB sequence

2.2 The second TTN sequence (b_a_bursts_windows_duplex.csv)

For the second TTN sequence, the optimal EWMA weight is 0.99, as shown in Figure 4. The measured TTNB and the predicted TTNB is shown in Figure 5. Figures show that the prediction over-estimates by 12.4ms 20% of the time, and under-estimates by 13.4ms 20% of the time.

Figure 4 Mean absolute prediction error vs the EWMA weight for the second TTNB sequence

Figure 5 The measured TTNBs and the predicted TTNBs for the second TTNB sequence

Figure 6 CDF of the prediction error for the second TTNB sequence

Given that RAN2 expects the error to be 1/8 msec, and the errors from the prediction observed here, we have the following observation:

Observation: The definition TTNB in TS23.501 could result in prediction errors that are 100x larger than the desired error expected from RAN2.

# Conclusion

Based on the observation in clause 3, we have the following proposal:

Proposal 1: SA4 redefines TTNB or replace it with a new term with the associated definition and inform other working groups including SA2, RAN2 and CT.

In S4-250356 [4], the idle period was proposed to replace TTNB. The idle period can be considered as the TTNB currently defined in TS23.501 minus the (magnitude of) the prediction error.

Proposal 2: Agree

Alternative 1: Replace the TTBN definition currently in TS23.501 with the following new definition:

TTNB is the duration of a time interval from the transmission of the last packet of the current data burst to a time before the transmission of the first packet of the next data burst during which the traffic source is guaranteed not to send delay-sensitive packets.

or

Alternative 2: keep the TTBN definition currently in TS23.501 but replace TTNB from the RTP header extension for dynamically changing traffic characteristics in TS26.522 with the idle period defined as follows:

Idle period is the duration of a time interval from the transmission of the last packet of the current data burst to a time before the transmission of the first packet of the next data burst during which the traffic source is guaranteed not to send delay-sensitive packets.

# References

[1] S4-250606, LS on Indicating Time to the Next Data Burst (TTNB), 3GPP TSG SA WG4 #131-bis-e, online, April 14-18, 2025.

[2] R2-2504812, Reply to LS on Indicating Time to the Next Data Burst (TTNB), 3GPP TSG RAN WG2 #130, Malta, 19~23 May 2025.

[3] S4-241835, Additional statistics for RTP Senders and Receivers in Solution #13, KI#12 for dynamic traffic, SA WG4#130, November 2024.

[4] S4-250356, On the ending time of the Time To the Next Data Burst (TTNB), 3GPP SA4 #131 Meeting, Geneva, Switzerland, 17-21 February 2025.