Agenda item: 	7

Source: 	Qualcomm Inc.

Title: 	[AvCall-MED]  Data Channel Transport of Avatar Data

Document for	Discussion and Agreement

# Introduction

In this contribution, we provide specification text for the section on Data Channel Transport of Avatar Data.

# Proposed Changes

## 8.4		Data Channel Transport of Avatar Data

### 8.4.1 General

An AR-MTSI client that supports avatars shall use an application data channel established per TS 26.264 for the transport of avatar control and animation data. The data channel sub-protocol for avatar metadata, including negotiation and animation, shall be 3gpp-ar-metadata and the payload shall follow the UTF-8 JSON AR-metadata envelope defined in clause 6.2. The data channel shall operate reliable and in-order. The SDP attribute a=3gpp_armetadata_types shall enumerate all avatar-related message URNs that the endpoint is prepared to send or receive on that ADC. This clause specifies the message families, transport behavior, and media type bindings for avatar negotiation, base-avatar delivery over the data channel, animation streams, and optional avatar QoE telemetry.

When scene description is used as the entry point, the Avatar ADC co-exists with the scene-description stream. The scene description continues to be transported as specified for IBACS and may reference avatar nodes and external media while the Avatar ADC carries the operational selections and runtime animation data for those nodes. This preserves the single-graph model established for IBACS scene description and avoids introducing a parallel signaling plane.

### 8.4.2 Transport of Base Avatar

When a base avatar or selected assets are delivered in-session over a data channel, the sender and receiver shall establish a second application data channel whose sub-protocol is 3gpp-arf. Payloads carried on 3gpp-arf shall be ARF containers with media types model/vnd.mpeg.arf+zip or model/vnd.mpeg.arf+mp4 as defined by ISO/IEC 23090-39. The 3gpp-arf channel shall be reliable and in-order and shall rely on the data-channel transport for segmentation and reassembly. Session-scoped authorization for dereferencing or transferring ARF content shall follow the BAR access policy; where an auth token is required, it shall be conveyed in the Avatar negotiation payload alongside the BAR URI and not as an out-of-band header on 3gpp-arf. The receiver shall validate the token prior to accepting ARF content.

### 8.4.3	Transport of Animation Streams

Avatar animation streams shall be transported over the Avatar ADC using the AR-metadata envelope and the message type urn:3gpp:ar:v2:avatar:animation. The payloads shall carry animation samples as specified by ISO/IEC 23090-39 and by the selected animation framework URN. The subtype field identifies the animation family; value 1 indicates facial blendshape samples, value 2 indicates skeletal joint samples, and value 3 indicates 2D landmark samples. No compression scheme is defined at this release for animation samples on the ADC.

# Proposal

We propose to agree the content of section 2 and integrate it as part of the base CR.