Agenda item: 	6

Source: 	Qualcomm Inc.

Title: 	[5G_RTP_Ph2] Over-provisioning in PDU Set size and data burst size indication

Document for	Agreement

# Introduction

Regarding the accuracy of the PDU Set size and data burst size indication, RAN2 reply LS R2-2504811/S4-251434 states that:

From RAN2’s perspective, in some use cases, the more accurate the indications on PDU Set size and data burst size are (up to a practical limit), the more efficiently RAN may use them in its scheduling and configurations. But higher accuracy may not always guarantee higher efficiency for RAN.

Note that the highlighted is clearly only about scheduling efficiency. The LS reply does not consider delay resulting from inaccurate PDU Set size and data burst size indication. It has been agreed in TR26.822 that under-provisioning the PDU Set size may result in increased end-to-end delay. But for XR applications, delay is an important performance metric.

The CR S4-251527, which was SA4 133-e meeting, tried to reflect the RAN2 reply LS. However, it left out scheduling efficiency and accidentally broadened the scope of the reply LS, as seen below:

A more accurate PDU Set Size indication is preferred. There may be some practical limits in indicating the PDU Set size accurately by the sender for use at the receiver due to network operations unknown to the RTP sender; in practice, a limited accuracy within a margin (of a few percents) can still be acceptable.

TR26.822 6.23 proposed several ways to do over-provisioning, which can be done by the RTP sender, UPF or RAN. It is best done by the RTP sender for two reasons:

- The RTP sender knows whether low latency is important for the application and whether there is a need for reducing the end-to-end latency.

- RTP sender’s behavior is within the scope of SA4, and the UPF and RAN’s behavior is outside the scope of SA4, and yet SA2 and RAN2 have not adopted any guidelines on over-provisioning the PDU Set size or data burst size. This makes the RTP sender the only place to do over-provisioning.

# Proposal

It is proposed to agree the following proposal.

Proposal: Add guidelines on over-provisioning in PDU Set size and data burst size indication to TS26.522.

A companion paper S4aR250149 proposes a CR to implement the above proposal.