Agenda item: 	4.10

Source: 	Qualcomm Incorporated

Title: 	[FS_5GVideo] Proposed Updates to Storage formats

Document for	Discussion and Agreement

# Introduction

Annex B of TR 26.955 documented some details on metrics and storage formats. Initially it was considered to use ISO BMFF based raw storage. However, it seems that this specification will not be ready and fully supported for some time. Hence, we proposed to change this to a json format for

- Reference Sequences

- Anchor and reference streams

- For metric computation

This document provides an update based on the discussion last week.

# Raw Video Format

## 2.1	Requirements

Based on TR 26.955 v0.4.0, Annex B, the following aspects of the raw data need to be provided:

1)	Spatial Resolution width

2)	Spatial Resolution height

3)	Chroma Format

4)	Chroma Sub-Sampling

5)	Picture Aspect ratio

6)	Frame rate

7) Bit depth

8) Color space format (be default set to 1 for BT.709)

9)	Transfer characteristics (be default set to 1 for BT.709)

10) Frame count

11) Duration (optional)

In addition, the following aspects were collected that should be added:

11)	Video storage format

12)	MD5 of raw video

13)	Reference to sequence (original website or 3GPP repository)

14)	Copyright statement

15)	Contact Person

16)	packing and storage format

17)	thumbnail preview

18)	mp4 preview

19)	Name of sequence

20)	Sequence key

21)	Some background information

Some information may be optional, others may be optionally defaulted.

## 2.3	Proposed JSON Schema

The proposed JSON schema for the raw format is here: https://github.com/haudiobe/5G-Video-Content/blob/main/3gpp-raw-schema.json.

## 2.4	Diagram

A diagram can be generated here: https://ogaoga.github.io/json-visual-editor/

## 2.5	Example

{ "Sequence": { "Name": "Sequence", "Background": "The Sequence is generated by someone and was provided by the study and shows a 3GPP meeting.", "Scenario": "FullHD", "Key": "S1-R1", "URI": "./sequence.yuv", "md5": "824fe384be7b0095c391ff6c5a5acc53", "thumbnail": "./sequence.jpg", "preview": "./sequence.mp4" }, "Properties": { "width": 1280, "height": 720, "format": "yuv", "packing": "interleaved", "scan": "progressive", "subsampling": "420", "bitDepth": 8, "frameRate": 30, "colourPrimaries": 1, "transferCharacteristics": 1, "matrixCoefficients": 1, "sampleAspectRatio": 1, "duration": 10, "frameCount": 30 }, "copyRight": "This material has been recorded by the Thomas Stockhammer, who owns the copyright. Thomas Stockhammer (licensor) has licensed it under the Creative Commons Attribution 4.0 license (https://creativecommons.org/licenses/by/4.0/). In short, this means you can freely reuse and distribute this content, also commercially, for as long you provide a proper attribution.", "Contact": { "Name": "Thomas Stockhammer", "Company": "Qualcomm", "e-mail": "tsto@qti.qualcomm.com" } }

## 2.6	Usage in TR 26.955

The information of the above sequence is added to Annex C as follows.

- Sequence is added under a Scenario according to Sequence.Scenario.

- The title of the sequence is Sequence.Name.

- The text from Sequence.Background is added to the introduction.

- A reference is added to a figure that adds the Sequence.thumbnail image.

- A table is added to document the Properties and details

  - All properties and values from Properties.

  - MD5: Sequence.md5

  - A reference to the sequence: Sequence.URI.

  - A reference to the preview: Sequence.preview.

- A new clause is added for the copyright statement.

- A new clause is added that references this json sequence

# Encoded Video Format

## 3.1	Requirements

Based on TR 26.955 v0.4.3, the following aspects of the encoded data/anchor need to be provided:

1)	Explanation what this anchor is important for

2)	Reference sequence

3)	Reference encoder

4)	Encoder configuration

5)	Reference to bitstream

6) MD5 of bitstream

7) Metrics

8) MD5 of reconstructed sequence

9) Encoder log

10) Copyright statement

11) Contact Person

Some information may be optional, others may be optionally defaulted.

## 3.3	Proposed JSON Schema

The proposed JSON schema for the encoded format is here: https://github.com/haudiobe/5G-Video-Content/blob/main/3gpp-bin-schema.json.

## 3.4	Diagram

A diagram can be generated here: https://ogaoga.github.io/json-visual-editor/

## 3.5	Example

{ "Bitstream": { "URI": "./sequence.bin", "md5": "824fe384be7b0095c391ff6c5a5acc53", "key": "S1-R1-264-QP27", "codecs": "avc1.64002a" }, "Generation": { "sequence": "./sequence.json", "encoder": "JM19.0", "config-file": "./encoder.cfg", "config-variant": "-qp 27", "log-file": "./decoder.log" }, "Reconstruction": { "decoder": "JM19.0", "log-file": "./decoder.log", "md5": "824fe384be7b0095c391ff6c5a5acc53" }, "Metrics": { "Bitrate": 4.32, "PSNR": 27.2, "SSIM": 14.8, "VMAF": 18.7 }, "copyRight": "This material has been recorded by the Thomas Stockhammer, who owns the copyright. Thomas Stockhammer (licensor) has licensed it under the Creative Commons Attribution 4.0 license (https://creativecommons.org/licenses/by/4.0/). In short, this means you can freely reuse and distribute this content, also commercially, for as long you provide a proper attribution.", "Contact": { "Name": "Thomas Stockhammer", "Company": "Qualcomm", "e-mail": "tsto@qti.qualcomm.com" } }

## 3.6	Usage in TR 26.955

The detailed usage is still tbd.

# Proposed Text Updates

NEXT CHANGE

## 5.2	Reference Sequences

This specification provides reference sequences that are used in order to provide anchors and are also made available in order to generate test results for other codecs. Reference sequences are selected to represent scenarios.

Reference sequences are collected described in Annex C of this document among along with their properties and their licenses. A format for raw reference sequences based on a JSON schema is defined in clause B.2.

Annex E describes how to upload and access reference sequences.

## 5.3	Anchors

Anchors provide a baseline that a tested method can be compared against. Anchors defined in this specification use a codec/profile/level that exists in an existing 3GPP specification as introduced in clause 4.

Anchor tuples are collected to address different quality and bitrates that can then be used for evaluation over a larger set of operation points.

The following principle apply to anchor definitions:

-	Each scenario typically has several well-defined anchors

-	An anchor is a combination of:

-	Explanation what this anchor is important for

-	Reference sequence

-	Reference encoder

-	Encoder configuration

-	Encoding complexity estimation, if available

-	Variable encoder configuration to create multiple quality/bitrate variants (using for example QP variations or other bitrate/quality evaluation tools).

-	Anchor tuples creating multiple variants, each including

-	Anchor bitstream

-	Anchor Metrics

-	Additional recommended anchor information includes

-	MD5 check sum of the complete reconstructed yuv file (anchor sequence)

-	Output picture log from reference encoder

-	Anchors and anchor tuples are an integral part of this document

Anchor tuples should be created over a wide range of parameters in order to provide sufficient data and overlap with expected test results to support the generation of characterization results (see clause 5.7).

The generation of anchor tuples is shown in Figure 5.3-1.

Figure 5.3-1: Anchor Tuple Generation Framework and Anchor Tuple Metrics Generation

Anchors are provided according to the defined binary format as defined in Annex B.3.

NEXT CHANGE

- Annex B: Data Formats and Metrics

# B.1	Introduction

This Annex provides a detailed overview on data formats and their usage for metrics computation.

# B.2	Raw Video Sequences

### B.2.1 Overview

For a raw video sequence used in the context of this Technical Report, the following metadata is expected to be provided. :

1)	Spatial Resolution width

2)	Spatial Resolution height

3)	Chroma Format

4)	Chroma Sub-Sampling

5)	Picture Aspect ratio

6)	Frame rate

7) Bit depth

8) Color space format (default is 1 for BT.709)

9)	Transfer characteristics (default is 1 for BT.709)

10) Frame count (and optionally the duration)

In addition, the following aspects were collected that should be added:

11)	Video storage format

12)	MD5 of raw video

13)	Reference to bitstream (URL to weblink where the original sequence is available)

14)	Copyright statement

15)	Contact Person

16)	packing and storage format

17)	thumbnail preview

18)	mp4 preview

19)	Name of sequence

20)	Sequence key

21)	Some background information

Some information may be optional, others may be optionally defaulted.

A JSON scheme is defined in clause B.2.2 for this matter. An example is provided in clause B.2.3.

### B.2.2 JSON Schema

JSON schema for the raw format is here: https://github.com/haudiobe/5G-Video-Content/blob/main/3gpp-raw-schema.json.

NOTE: this will be copied in here once agreed

### B.2.3 Example

{

"Sequence": {

"Name": "Sequence",

"Background": "The Sequence is generated by someone and was provided by the study and shows a 3GPP meeting.",

"Scenario": "FullHD",

"Key": "S1-R1",

"URI": "./sequence.yuv",

"md5": "824fe384be7b0095c391ff6c5a5acc53",

"thumbnail": "./sequence.jpg",

"preview": "./sequence.mp4"

},

"Properties": {

"width": 1280,

"height": 720,

"format": "yuv",

"packing": "interleaved",

"scan": "progressive",

"subsampling": "420",

"bitDepth": 8,

"frameRate": 30,

"colourPrimaries": 1,

"transferCharacteristics": 1,

"matrixCoefficients": 1,

"sampleAspectRatio": 1,

"frameCount": 30

},

"copyRight": "This material has been recorded by  Thomas Stockhammer, who owns the copyright. Thomas Stockhammer (licensor) has licensed it under the Creative Commons Attribution 4.0 license (https://creativecommons.org/licenses/by/4.0/). In short, this means you can freely reuse and distribute this content, also commercially, for as long you provide a proper attribution.",

"Contact": {

"Name": "Thomas Stockhammer",

"Company": "Qualcomm",

"e-mail": "tsto@qti.qualcomm.com"

}

}

# B.3	Encoded Video Sequences

### B.2.1 Overview

For encoded video sequences used in the context of this Technical Report, the following metadata is expected to be provided:

1)	Explanation what this anchor is important for

2)	Reference sequence

3)	Reference encoder

4)	Encoder configuration

5)	Reference to bitstream

6) MD5 of bitstream

7) Metrics

8) MD5 of reconstructed sequence

9) Encoder log

10) Copyright statement

11) Contact Person

Some information may be optional, others may be optionally defaulted.

A JSON scheme is defined in clause B.2.2 for this matter. An example is provided in clause B.2.3.

### B.3.2 JSON Schema

JSON schema for the encoded format is here: https://github.com/haudiobe/5G-Video-Content/blob/main/3gpp-bin-schema.json.

NOTE: this will be copied in here once agreed

### B.3.3 Example

{

"Bitstream": {

"URI": "./sequence.bin",

"md5": "824fe384be7b0095c391ff6c5a5acc53",

"key": "S1-R1-264-QP27",

"codecs": "avc1.64002a"

},

"Generation": {

"sequence": "./sequence.json",

"encoder": "JM19.0",

"config-file": "./encoder.cfg",

"config-variant": "-qp 27",

"log-file": "./decoder.log"

},

"Reconstruction": {

"decoder": "JM19.0",

"log-file": "./decoder.log",

"md5": "824fe384be7b0095c391ff6c5a5acc53"

},

"Metrics": {

"Bitrate": 4.32,

"PSNR": 27.2,

"SSIM": 14.8,

"VMAF": 18.7

},

"copyRight": "This material has been recorded by the Thomas Stockhammer, who owns the copyright. Thomas Stockhammer (licensor) has licensed it under the Creative Commons Attribution 4.0 license (https://creativecommons.org/licenses/by/4.0/). In short, this means you can freely reuse and distribute this content, also commercially, for as long you provide a proper attribution.",

"Contact": {

"Name": "Thomas Stockhammer",

"Company": "Qualcomm",

"e-mail": "tsto@qti.qualcomm.com"

}

}

NEXT CHANGE

# C.1	Introduction

This annex provides a summary of candidate reference sequences that where discussed to be potentially suitable for one or multiple of the scenarios introduced in clause 6 of this Technical Report. For each candidate reference sequence, at least the following information is provided.

-	A summary of the sequence characteristics

-	A screenshot of the sequence

-	Source sequence properties

-	Information where the source sequence is hosted

-	Copyright and license information

The content is provided in JSON files here: https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequence. The format of the reference sequences follows the proposed format in Annex B.

NEXT CHANGE

# E.2	Reference Sequences

## E.2.1 	Hosting

All reference sequences are hosted at

https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequence.

Below this folder, for each reference sequence a dedicated folder is created. The folder includes at least the following information:

-	JSON file for raw video according to schema in clause C.1.2.

In addition, the folder may include the following information

- 	the raw video sequence, if hosted on the server

-	a preview file

-	a thumbnail image

## E.2.2 	Uploading

For uploading a new reference sequence, please create a new issue here

- https://github.com/haudiobe/5G-Video-Content/issues/new?assignees=&labels=request&template=reference_sequence.md&title=

Please specify all required information that needs to be added to the JSON file for raw video sequences as defined in clause C.1.2.

# E.3		Anchors

## E.3.1 	Hosting

All anchors are hosted at:

https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Anchors.

Below this folder, a hierarchy is created

-	Scenario

-	Codec

-	Anchor Tuple

-	Anchor

A dedicated folder is created.

Below this folder, for each anchor a dedicated folder is created.

The folder includes at least the following information:

-	JSON file for binary video according to schema in clause E.3.4.

-	binary video bitstream

## E.3.2 	Uploading

For uploading anchors, please create a new issue here

- https://github.com/haudiobe/5G-Video-Content/issues/new?assignees=&labels=request&template=anchor.md&title=

Please specify all required information that needs to be added to the JSON file for encoded video sequences as defined in clause C.1.2

## E.3.3 	Downloading

Anchors can be downloaded for free from the above folders. Licensing terms must be obeyed. Downloaded files should be MD5 verified.