SA4-e (AH) Video SWG post 114-e	S4aV210751

3rd August 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.955 | CR | <CR#> | rev | - | Current version: | 1.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | pCR on reference metrics computation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GVideo |  |  |  |  |  | Date: |  |  | 2021-01-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduce reference for metrics computation to simplify cross-check and validation of results. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Additional clarifications on metrics computation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

===== CHANGE 1 =====

### 5.5.7	Reference computation of SDR metrics

Computation of PSNR(Y), PSNR(U) and PSNR(V) metrics as defined in clause 5.5.3 is performed with HDRMetics tool with the following parameters specified:

- EnablePSNR=1

- EnableJVETPSNR=1

Computation of MS-SSIM metric as defined in clause 5.5.4 is performed with HDRMetics tool with the following parameters specified:

- EnableMSSSIM=1

- EnableLogSSIM=1

- SSIMBlockDistance=1

- SSIMBlockSizeX=11

- SSIMBlockSizeY=11

Computation of VMAF using FFMPEG build with integrated libvmaf (such as available at https://www.gyan.dev/ffmpeg/builds/) is performed using the following command line:

.\ffmpeg -s $WxH -pix_fmt $PIX_FMT -r $FRAME_RATE  -i ref.yuv -s $WxH -pix_fmt yuv420p10le -r $FRAME_RATE -i test.yuv -lavfi libvmaf=model_path=$PATH_TO_MODEL -f null –

$WxH: specifies resolution of the video e.g., 1920x1080, 3840x2160,

$PIX_FMT: specifies pixel format e.g., yuv420p10le,

$FRAME_RATE: specifies video frame rate e.g., 30, 60,

$PATH_TO_MODEL: specifies path to VMAF model.

For 4K video resolutions vmaf_4k_v0.6.1.json model should be used. For HD and below HD video resolutions vmaf_v0.6.1.json should be used.

Additional libvmaf parameters phone_model and enable_transform should be set to [0].

===== END OF CHANGE 1 =====

===== CHANGE 2 =====

### 5.5.8	Reference computation of HDR metrics

Computation of wPSNR(Y), wPSNR(U) and wPSNR(V) metrics as defined in clause 5.5.5 is performed with HDRMetrics tool with the following parameters specified:

- EnableJVETPSNR=1,

- EnableWTPSNR=1,

- WeightTableFile="hdrTable.txt",

where, hdrTable.txt is available in cfg/hdrTable.txt of HDRTools repository.

Computation of DeltaE100 and PSNRL100 metrics requires conversion from YUV to linear light RGB data which is performed with the use of HDRConvert and reference config file cfg/JCTVC_CTC_cfgFiles/YCbCr/HDRConvertYCbCr420ToEXR2020.cfg

After the conversion for reference and decoded video clips is done, computation of DeltaE100 and PSNRL100 metric as defined in caluse 5.5.5 is performed with HDRMetrics tools with the following parameters specified:

- MaxSampleValue=10000.0,

- WhitePointDeltaE1=100.0,

- EnableDELTAE=1,

- DeltaEPointsEnable=1

===== END OF CHANGE 2 =====

===== CHANGE 3 =====

## 5.7	Characterization

Characterization is the comparison of a codec under test with an anchor based on the framework introduced in this clause. Characterization in this report is based on Bjöntegard-Delta (BD)-rate information according to [44].

A full characterization of a codec for a scenario against a 3GPP codec shall provide at least the following metrics

-	The BD-rate gain for each defined anchor tuple and each required metric

-	The average BD-rate gain across all anchors of the scenario for each required metric.

Figure 5.7-1: Characterization Framework

BD-Rate is computed according to the CTC method used in JVET and specified in [44] from the tools publicly available: Reference codec software, Excel file available in [57] for SDR and in [58] for HDR.

The Excel files include the VBS script bdrate( ) to compute the BD-Rate performance between a test codec and a reference from four or five rate-distortion points.

These excel files have been extended in the Random-Access and low delay tabs to contain new columns for the new metrics: VMAF and MS-SSIM, in the SDR case only. The “SA4 extended excel files” for SDR and HDR are attached as S4-template-HDR.xlsx and S4-template-SDR.xlsx.

Editor’s Note: Need to decide what to do if a metric does not have a monotonic behavior for a particular sequence.

Editor’s Note: At this stage the BD-Rate is defined as result of the above excel sheets. In an updated version of the TR, an independent metric computation tool may be provided that allows to generate the BD-Rate values based on the metrics results. The reference for an extended version with 5 points is still tbd.

Another optional method is provided, as described in Annex F, to compute the metrics automatically.  BD-Rate computation is supported by a script that uses anchor tuple and test tuple metrics to provide the characterization results as shown in Figure 5.7-1.

For details on BD-Rate computation, refer to [44].

===== END OF CHANGE 3 =====

===== CHANGE 4 =====

### metrics computation:

HDRTools

HDRMETRICS_TOOL=/path/to/HDRTools/build/bin/HDRMetrics

VMAF

VMAF_EXE=/path/to/vmaf/libvmaf/build/tools/vmaf

VMAF_MODEL=path=/path/to/vmaf/model/vmaf_v0.6.1.json  #HD and below #HD resolution video

VMAF_MODEL_4K=path=/path/to/vmaf/model/vmaf_4k_v0.6.1.json  #4K resolution video

===== END OF CHANGE 4 =====