3GPP TSG SA4-e (AH) Video SWG post 127-bis-e                            	  S4aV240007

Online, 7th May, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.956 | CR | <CR#> | rev | <Rev#> | Current version: | 0.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_Beyond2D] Summary of VR Video Profiles from TS 26.118 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile Com. Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Beyond2D |  |  |  |  |  | Date: |  |  | 05-07-2024 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TR 26.956, clause 5 aims to provide a summary of the existing beyond 2D video capabilities in 3GPP, from at least TS.26.119 and TS.26.118. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add Video profiles for 360 VR streaming services, as defined in TS 26.118. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete overview of existing beyond 2D video capabilities in 3GPP. |  |  |  |  |  |  |  |  |
| c |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.3(New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGE #1

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	Allied Market Research, “3D Technology Market Size, Share, Competitive Landscape and Trend Analysis Report by Product, Application : Global Opportunity Analysis and Industry Forecast, 2021-2030.”,  www.alliedmarketresearch.com/3d-technology-market.

[3]	Mordor Intelligence, “Mobile 3D Market Size & Share Analysis - Growth Trends & Forecasts (2024 - 2029).”,  https://www.mordorintelligence.com/industry-reports/mobile-3d-market.

[4]	Grand View Research, “Immersive Technology Market Size, Share & Trends Analysis Report By Component (Hardware, Software, Services), By Technology, By Application, By Industry, By Region, And Segment Forecasts, 2023 - 2030.”,  https://www.grandviewresearch.com/industry-analysis/immersive-technology-market-report.

[5]	3GPP TS 26.119: "Media Capabilities for Augmented Reality".

[6]	3GPP TS 26.118: "Virtual Reality (VR) profiles for streaming applications".

CHANGE #2

5	Overview of existing "Beyond 2D" Video Capabilities in 3GPP

5.3 	VR Video Profiles

The VR profiles for streaming services are defined in TS 26.118 [6], specifying the coded representation and media profile of 360 VR distribution signals. Table 5.3-1 provides an overview of the 360 VR relevant formats considered in the context of 3GPP VR Profiles.

For restrictions on source formats such as resolution and frame rates, content generation and encoding guidelines, refer to TS 26.118 [6], Annex A.

Table 5.3-1: High-level Summary of Operation Points

| Operation Point name | Decoder | Bit depth | Typical Original Spatial Resolution | Frame Rate | Colour space format | Transfer Characteristics | Projection | Rotation | RWP | Stereo |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Basic H.264/AVC | H.264/AVC HP@L5.1 | 8 | Up to 4k | Up to 60 Hz | BT.709 | BT.709 | ERP w/o padding | No | No | No |
| Main H.265/HEVC | H.265/HEVC MP10@L5.1 | 8, 10 | Up to 6k in mono and 3k in stereo | Up to 60 Hz | BT.709 BT.2020 | BT.709 | ERP w/o padding | No | Yes | Yes |
| Flexible H.265/HEVC | H.265/HEVC MP10@L5.1 | 8, 10 | Up to 8k in mono and 3k in stereo | Up to 120 Hz | BT.709 BT.2020 | BT.709, BT.2100 PQ, BT.2100 HLG | ERP w/o padding CMP | No | Yes | Yes |
| Main 8K H.265/HEVC | H.265/HEVC MP10@L6.1 | 10 | Up to 8k in mono and 6k in stereo | Up to 60 Hz for 8K and 120 Hz for 4k | BT.709 BT.2020 | BT.709, BT.2100 PQ, BT.2100 HLG | ERP w/o padding | No | Yes, but restricted to coverage | Yes |

Table 5.3-2 summarizes the video operation point, sample entry, and DASH integration associated with each video media profiles defined in clause 5.2 of TS 26.118 [6].

Table 5.3-2 Video Media Profiles

| Media Profile | Operation Point | Sample Entry | DASH Integration |
| --- | --- | --- | --- |
| Basic Video | Basic H.264/AVC | resv avc1 | Single Adaptation Set Single Representation streaming |
| Main Video | Main H.265/HEVC or Main 8K H.265/HEVC | resv hvc1 | Single or Multiple independent Adaptation Sets offered Single Representation streaming |
| Advanced Video | Flexible H.265/HEVC | resv hvc1, hvc2 | Single or Multiple dependent Adaptation Sets offered Single or Multiple representation streaming |

END OF CHANGES