3GPP TSG-SA4 Meeting (AH) Video SWG post #131-bis-e	S4aV250043

Online, 29 April – 8 May 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.265 | CR | pseudo | rev | - | Current version: | 1.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [VOPS] pCR On terminology |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | VOPS |  |  |  |  |  | Date: |  |  | 2025-05-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This changes derives from discussion around S4-250601. Creating the an intermediate concept between bitstream and CVS such this concept can accommodate layer in case of layer coding. The proposal is to reuse elementary stream as known from system integration, cf analysis below. - Analysis of terminology: - HEVC: - Elementary stream (only 3 occurrences in the HEVC spec): - A sequence of one or more bitstreams: - A sequence of bits, in the form of a NAL unit stream or a byte stream, that forms the representation of coded pictures and associated data forming one or more coded video sequences (CVSs): - A sequence of access units that consists in a given order. - (general) A set of NAL units […] and contain exactly one coded picture with nuh_layer_id equal to 0. - (multiview) A set of NAL units [...] and contain at most one coded picture with any specific value of nuh_layer_id. - System (14496-1/ISOBMFF/DASH/CMAF) - Elementary stream - (part 1) consecutive flow of mono-media data from a single source entity to a single destination entity on the compression layer. More than one elementary stream may be connected to a single decoder (e.g., in a decoder of a scalable audio-visual object) - (NALU FF) sequence of one or more bitstreams of the applicable video standard - access units: - (part 1) smallest individually accessible portion of data within an elementary stream to which unique timing information can be attributed - (DASH) unit of a media stream (3.1.29) with an assigned Media Presentation time - (ISOBMFF) media data pertaining to a particular composition time in a media stream, usually carried in one sample of a media track. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Access Unit: Smallest individually accessible portion of data within a Bitstream to which unique timing information can be attributed.

Bitstream: A sequence of bits that forms the representation of any coded pictures and associated data. This sequence of bits is formed by one or more coded video sequences (CVSs) where the CVS share identical metadata.

Coded Video Sequence: A sequence of bits that consists of a series of coded frames and any associated metadata (required for decoder and rendering initialization) and conforms to a specific video encoding format and aligns with a certain Operation Point, as defined in this document. Such coded video sequence (CVS) has no decoding dependency on any other prior CVS and consists, in decoding order, of information specifying the characteristics or format of the encoded video data, a single intra random access coded frame followed by zero or more dependent, on the intra random access coded frame, coded frames, and a series of associated coded metadata.

Chroma: a sample array or single sample representing one of the two colour difference signals related to the primary colours, represented by the symbols Cb and Cr.

Elementary Stream: One or more Coded Video Sequences that conform to a specific video encoding format and aligns with a certain Operation Point.

Hero Eye: The default eye in a stereo (stereoscopic) video pair, often determined by tags set by the cameras used to capture the video.

Luma: a sample array or single sample representing the monochrome signal related to the primary colours (denoted with the symbol Y),

Operation Point: A collection of discrete combinations of different video representation formats, including spatial and temporal resolutions, colour mapping, transfer functions, and the encoding format.

Receiver: A device capable of decoding and rendering any bitstream that is conforming to a certain Operation Point.

* * * End of Changes * * * *