Source:	Sony Mobile Communications

Title:	Suggestions regarding the loudness tool in FS_EVS_FCNBE

Document for:	Agreement

Agenda Item:	3

# Introduction

Within “Study on EVS Float Conformance Non Bit-Exact”, the loudness estimation portion of ITU-R BS.1387-1 music quality predictor PEAQ [1] has been proposed and documented in TR 26.483 clause 5.3.3 Maximum Loudness Difference [2]. This is a useful addition to the toolset in the TR, but some improvements are suggested in the present document.

Absolute level calibration is essential for loudness models. At SA4 #98, there was no information about which presentation level was assumed in the tests. The PEAQ specification uses a default scaling according to the following:

Figure 1 Excerpt from BS.1387-1.

We may assume that “full scale input” should in this context be understood as the instantaneous max level is Lmax.

Table 1 Acoustic and digital file levels

|  | Acoustic peak level | Acoustic RMS level | Digital RMS level |
| --- | --- | --- | --- |
| Max pure tone | Lmax dBSPLpeak | Lmax-3 dBSPL | -3 dBov |
| -26 dBov “nominal” level |  | Lmax-26 dBSPL | -26 dBov |
| 1 sone pure tone |  | 40 dBSPL | 40-Lmax dBov |

Using default values:

Table 2 Acoustic and digital file levels, example of Lmax=92 dBSPL

|  | Acoustic peak level | Acoustic RMS level | Digital RMS level |
| --- | --- | --- | --- |
| Max pure tone | 92 dBSPLpeak | 89 dBSPL | -3 dBov |
| -26 dBov “nominal” level |  | 66 dBSPL | -26 dBov |
| 1 sone pure tone |  | 40 dBSPL | -52 dBov |

For the EVS codec standardization, tests were carried out at a variety of input levels to the encoder but -26 dBov was the nominal input level. Assuming the conformance test file corpus will retain this nominal level, we can note the assumed presentation level of 66 dBSPL, if using the default Lmax of 92 dBSPL for the loudness model. This is fairly low for the purposes of FS_EVS_FCNBE. One consequence of setting a too low Lmax value is that audible low-level signals are estimated as inaudible. Within the present work item, it means the tool could be less sensitive.

To select a presentation level, we note:

- Commonly used reference levels which were also used in the EVS development:

  - monotic presentation 79 dBSPL

  - diotic presentation 73 dBSPL

- For a mobile phone in handset or headset mode at maximum volume (assuming wideband RLR=-13dB and flat diffuse-field corrected frequency repsonse), the expected level is 95.5 dBSPL, for nominal -16 dBm0 input, see section 3.2 of S4-180055 [3]. This will however be perceived as very loud in the case of diotic presentation. Also, significantly raised volume controls is often associated with background noise which would work as a masker.

- For a mobile phone in headset mode at nominal volume (RLR=8 dB) the expected level is 74.5 dBSPL, for nominal -16 dBm0 input

If we restrict to assuming diotic presentation and nominal level, it seems suitable to select 74.5 dBSPL. If we include a “safety margin” and assume a somewhat raised level above nominal (while still realistic for close-to-silent condition listening), we could select 74.5+X dBSPL, where X is e.g. 6 dB.

Table 3 Acoustic and digital file levels, example of Lmax=106.5 dBSPL

|  | Acoustic peak level | Acoustic RMS level | Digital RMS level |
| --- | --- | --- | --- |
| Max pure tone | 106.5 dBSPLpeak | 103.5 dBSPL | -3 dBov |
| -26 dBov “nominal” level |  | 80.5 dBSPL | -26 dBov |
| 1 sone pure tone |  | 40 dBSPL | -66.5 dBov |

It is the understanding of the source that the bulk of calculations in PEAQ are independent between left and right channels, and no loudness left/right summation is performed. It is not clear how it has been implemented, but assuming that PEAQ applies the referenced Zwicker model “as is” to each channel, we should not sum the loudness contributions from left and right channels.

# Suggested procedure for determining Lmax

To avoid uncertainties of the scaling:

- Select assumed acoustic presentation level 80.5 dBSPL for -26 dBov

- Construct a mono calibration file with a 1 kHz pure tone representing 40 dBSPL (r.m.s. -66.5 dBov)

- Repeat until the loudness model gives 1 sone for each channel:

  - Measure the loudness with the calibration file

  - Adjust Lmax

Use this Lmax  for testing and document it together with results.

# Proposals

Documentation

As a minimum, it should be documented in TR 26.483:

- which Lmax value that was used to obtain the results of the TR

- the procedure in clause 2

- a recommended Lmax value

- how the implementation of the loudness portion of PEAQ shall be done, especially w.r.t. treatment of left and right signals, potential separate loudness values obtained for each channel, and in general avoid any ambiguity

Example results in TR, using appropriate Lmax

Even better, the tests documented in the TR clause 5.3.3 could be re-run with an Lmax value obtained as described in section 2. It is not clear how large differences would be observed, but it would seem appropriate to address this issue now, rather than later.

Availability and unambiguity of the tool

According to the understanding of the source, there is no code, nor executable available for those that would want to use the loudness-based tool. The TR explains how to modify a specific implementation of a portion of the ITU-R Recommendation which has in the first place no published source code. Ideally, the loudness predictor should be changed to ISO 532-1, since

- it is up-to-date

- it has published source code

- we would avoid the ambiguity associated with referring to a portion of PEAQ + modifications

Specific loudness tool

Depending on how sensitive a tool is desired for a certain purpose, an additional tool could be added, based on specific loudness, as a way to perform critical band-based analysis.

The already described method could be used, but instead of aggregating to a total loudness value, each critical band is used separately, adding one dimension (time, critical band, spec. loudness). This would enable more sensitive testing, similar to the spectral distortion tool, but avoid flagging differences that would be imperceptible due to masking (if desirable).

Interpretation of results

The tools could be used to address different kinds of questions:

- Bit-exact: “Is there any difference at all in the resulting signal?”

- SNR, RMS error, spectral distortion: “How large is the difference?”

- Specific loudness: “Do we predict any audible difference?”

- MOS-LQO: “Do we predict any quality difference”?

- Loudness: “Do we predict any loudness difference”?

It can also be noted that segment-by-segment analysis is applied but the smallest possible size of segments differs between methods.

# References

[1]	ITU-R BS.1387-1, Method for objective measurements of perceived audio quality

[2]	S4-180610 - Draft TR 26.843 Study on Non Bit-Exact Conformance Criteria and Tools for Floating-Point EVS Codec, v. 1.0.0

[3]	S4-180055 - Investigation and suggestion regarding test method for RAOT

Strictly speaking, the question about audible difference is of course more complicated than just comparing specific loudness segment-per-segment. But the critical-band analysis is at least a perceptually motivated method.