3GPP TSG-SA4 EVS SWG Conference Call on FCNBE	AHEVS-481

September 26, 2019

Agenda item: 	3

Source: 	Fraunhofer IIS, Intel, Apple

Title: 	MOS-LQO speech database verification

Document for	Information

# Introduction

This contribution presents verification data for the database for the MOS-LQO test used in the course of the study [1] and work item [2] with the suggested McGill TSP speech database (see also [3] and [4]).

# Test Files

## P.501

The speech test files used in the course of the study [1] and work item [2] are based on ITU-T P.501 Annex B & C. The 30 files representing various talkers and languages. The files are listed below:

an1f1s1 => AnnexC//P501_C_chinese_f1_FB_48k.wav

an1f1s2 => AnnexC//P501_C_chinese_f2_FB_48k.wav

an1f1s3 => Speech and Noise Signals Clause B/Dutch_FB_clause_B.3.2//female 1.wav

an1f1s4 => Speech and Noise Signals Clause B/Dutch_FB_clause_B.3.2//female 2.wav

an1f1s5 => AnnexC//P501_C_english_f1_FB_48k.wav

an1f2s1 => AnnexC//P501_C_english_f2_FB_48k.wav

an1f2s2 => AnnexC//P501_C_finnish_f1_FB_48k.wav

an1f2s3 => AnnexC//P501_C_finnish_f2_FB_48k.wav

an1f2s4 => AnnexC//P501_C_french_f1_FB_48k.wav

an1f2s5 => AnnexC//P501_C_french_f2_FB_48k.wav

an1f3s1 => AnnexC//P501_C_german_f1_FB_48k.wav

an1f3s2 => AnnexC//P501_C_german_f2_FB_48k.wav

an1f3s3 => AnnexC//P501_C_italian_f1_FB_48k.wav

an1f3s4 => AnnexC//P501_C_italian_f2_FB_48k.wav

an1f3s5 => AnnexC//P501_C_japanese_f1_FB_48k.wav

an1m1s1 => AnnexC//P501_C_chinese_m1_FB_48k.wav

an1m1s2 => AnnexC//P501_C_chinese_m2_FB_48k.wav

an1m1s3 => Speech and Noise Signals Clause B/Dutch_FB_clause_B.3.2//male 1.wav

an1m1s4 => Speech and Noise Signals Clause B/Dutch_FB_clause_B.3.2//male 2.wav

an1m1s5 => AnnexC//P501_C_english_m1_FB_48k.wav

an1m2s1 => AnnexC//P501_C_english_m2_FB_48k.wav

an1m2s2 => AnnexC//P501_C_finnish_m1_FB_48k.wav

an1m2s3 => AnnexC//P501_C_finnish_m2_FB_48k.wav

an1m2s4 => AnnexC//P501_C_french_m1_FB_48k.wav

an1m2s5 => AnnexC//P501_C_french_m2_FB_48k.wav

an1m3s1 => AnnexC//P501_C_german_m1_FB_48k.wav

an1m3s2 => AnnexC//P501_C_german_m2_FB_48k.wav

an1m3s3 => AnnexC//P501_C_italian_m1_FB_48k.wav

an1m3s4 => AnnexC//P501_C_italian_m2_FB_48k.wav

an1m3s5 => AnnexC//P501_C_japanese_m1_FB_48k.wav

## McGill TSP Speech Database

In contrast to ITU-T P.501, which contains eight languages with two sentence pairs (male/female) per language, the McGill TSP Speech database contains a single language with over 1400 utterances by 23 speakers (eleven male, twelve female), reading multiple Harvard sentences. The data was recorded in an anechoic room. The database includes the original samples (48 kHz sampling rate), and also the data filtered and subsampled to different sample rates.

For creating a speech database for the MOS-LQO test, sentence pairs were created from the available samples, aiming at covering as many talker and Harvard Sentences as possible. Due to the structure of the recording, no single phonetically balanced list from the Harvard Sentences was recorded by all talkers, thus the first two sentences in each list were used to create the sentence pairs.

For generating P.800 style sentence pairs, the active speech of the first sentence is usually preceded by some initial silence (i.e. the recording’s idle noise), followed by a silent gap before the second sentence, and finally a silent trailing silent period. Unfortunately, the sentences of the TSP database are cut with only ~100ms guard silence.

Since P.863 requires a certain temporal structure in the analyzed speech samples (see section 8.1.2 of P.863) following the P.800 style, the sentence pairs were composed of 0.5s silence, followed by the first sentence, 1s of silence, the second sentence, and finally 0.5s silence.

This described procedure of item selection and temporal structure resulted in the following 30 sentence pairs:

FA02_01.wav, FA02_02.wav -> an1f1s1.wav         6.410s

FA02_03.wav, FA02_04.wav -> an1f1s2.wav         7.165s

FB08_01.wav, FB08_02.wav -> an1f1s3.wav         7.162s

FB08_03.wav, FB08_04.wav -> an1f1s4.wav         7.209s

FC14_01.wav, FC14_02.wav -> an1f1s5.wav         6.891s

FC14_03.wav, FC14_04.wav -> an1f2s1.wav         5.834s

FD20_01.wav, FD20_02.wav -> an1f2s2.wav         6.759s

FE26_01.wav, FE26_02.wav -> an1f2s3.wav         7.118s

FF32_01.wav, FF32_02.wav -> an1f2s4.wav         7.875s

FG38_01.wav, FG38_02.wav -> an1f2s5.wav         5.859s

FH44_01.wav, FH44_02.wav -> an1f3s1.wav         6.882s

FI50_01.wav, FI50_02.wav -> an1f3s2.wav         7.345s

FJ56_01.wav, FJ56_02.wav -> an1f3s3.wav         7.269s

FK62_01.wav, FK62_02.wav -> an1f3s4.wav         6.589s

FL68_01.wav, FL68_02.wav -> an1f3s5.wav         6.544s

MA01_01.wav, MA01_02.wav -> an1m1s1.wav         6.712s

MA01_03.wav, MA01_04.wav -> an1m1s2.wav         6.419s

MB07_01.wav, MB07_02.wav -> an1m1s3.wav         6.772s

MB07_03.wav, MB07_04.wav -> an1m1s4.wav         6.838s

MC13_01.wav, MC13_02.wav -> an1m1s5.wav         5.702s

MC13_03.wav, MC13_04.wav -> an1m2s1.wav         6.259s

MD19_01.wav, MD19_02.wav -> an1m2s2.wav         6.608s

MD19_03.wav, MD19_04.wav -> an1m2s3.wav         7.335s

MF31_01.wav, MF31_02.wav -> an1m2s4.wav         6.202s

MG37_01.wav, MG37_02.wav -> an1m2s5.wav         6.570s

MH43_01.wav, MH43_02.wav -> an1m3s1.wav         6.457s

MI49_01.wav, MI49_02.wav -> an1m3s2.wav         7.515s

MJ55_01.wav, MJ55_02.wav -> an1m3s3.wav         6.183s

MK61_01.wav, MK61_02.wav -> an1m3s4.wav         6.677s

ML67_01.wav, ML67_02.wav -> an1m3s5.wav         5.758s

# Test Description

A subset of the MOS-LQO test was conducted with all verification implementations (see Table 1).

Table 1. List of implementations used for Verifications

| Name | Code | Platform | Compiler | Optimization | OS | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| gcc-7 x86_64_O3 | CB0 | x86_64 | GCC-7 | O3 | Linux |  |
| 26.442 | CC0 | x86_64 | Visual-C++ 14.10 | None | Win32 | Fixed-point code |
| FLC_Test1A | CB0 | x86_64 | GCC-7 | O3 | Linux | Code change |
| FLC_Test2 | CB0 | x86_64 | GCC-7 | O3 | Linux | Decoder code change |
| icc16_x86_32_O2source | CB0 | x86_32 | ICC-16 | O2, source | Linux |  |
| icc16_x86_32_O3source | CB0 | x86_32 | ICC-16 | O3, source | Linux |  |
| icc16_x86_32_O2Fast | CB0 | x86_32 | ICC-16 | O2, fast | Linux | Unsafe optimization |
| icc19_x86_64_avx2Fast | CB0 | x86_64 | ICC-19 | O2, fast | Linux | Unsafe optimization |
| icc19_x86_64_O3source | CB0 | x86_64 | ICC-19 | O3 source, high precision | Linux |  |
| aarch64_ggc-8_Ofast | CB0 | ARMv8a | GCC-8 | O fast | Linux | Unsafe optimization |
| gcc-7_x86_0fast | CB0 | x86_64 | GCC-7 | Ofast | Linux | Unsafe optimization |
| Clang-6_mavx2_O3 | CB0 | x86_64 | Clang-6 | O3 | Linux |  |
| 26.443_d20 | D20 | x86_64 | Visual-C++ 9.0 |  | Win32 | Obsolete code version |

The 30 input speech items of both the P.501 and the TSP database were processed for the 55 SWB clean speech, clean channel conditions that are part of the MOS-LQO test, proposed for 26.444, resulting in MOS-LQO difference scores for the cases A-B (looking at both IuT enc/dec), A-C (looking at IuT dec), A-D (looking at IuT enc). Processed material was ~200s per condition, with 55 conditions for 13 IuTs, which is ~40 hours in processed material that was analyzed using P.863.

# MOS-LQO Test Results

Figure 1: FX/FX vs. IuT/IuT

Figure 2: FX/FX vs. FX/IuT

Figure 3: FX/FX vs. IuT/FX

From the results one can observe that the MOS-LQO differences for the P.501 and the TSP speech databases lead to different results, which confirms the expectation, that thresholds would only be applicable to a set database. Still, the average scores are in the same order of magnitude, with one case sticking out, which is the code change 1A, that show less variation with the TSP database.

# Summary

Based on the above results it is clear that the definition of the thresholds depend on database used. The results indicate that both databases still provide similar results. The P.501 items seem to better identify the code change 1A.

The sources like to note that this is only an excerpt of the set of experiments of the full MOS-LQO test, which is ~20x in size.

Based on these results, the sources suggest to keep the current database, based on P.501, for the MOS-LQO test, as it covers more languages and there are already significant amount of results obtained with this databse.

# References

[1] 3GPP TR 26.843 “Study on Non Bit-Exact Conformance Criteria and Tools for Floating-Point EVS Codec”

[2] S4-190922 “EVS Float Conformance Verification”, Fraunhofer IIS, Intel, Apple, August 2019

[3] AHEVS-463 “Database for the Proposed MOS-LQO Test”, Qualcomm, May 2019

[4] S4-190911 “On the Proposed EVS FLC Method”, Qualcomm, August 2019