3GPP TSG-SA4 MTSI SWG Telco on E2E_DELAY	S4-AHM445

18 December 2018, 16:00-18:00 CET

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.114 | CR | XXXX | rev |  | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DBI Signaling Recommendations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | E2E_DELAY |  |  |  |  |  | Date: |  |  | 2018-12-15 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | IANA registration of the new RTCP-FB message type for DBI is complete and value ‘10’ has been assigned. This needs to be reflected in the spec. Moreover, inclusion of further recommendations on the use of DBI signalling is desirable. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include the new IANA registration value. In addition, add further recommendations on the use of DBI signaling. Also make a few editorial enhancements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 7.3.8, R.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First Change

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply:

NOTE:	An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AC	Alternating Current

AL-SDU	Application Layer - Service Data Unit

AMR	Adaptive Multi-Rate

AMR-NB	Adaptive Multi-Rate - NarrowBand

AMR-WB	Adaptive Multi-Rate - WideBand

AMR-WB IO	Adaptive Multi-Rate - WideBand Inter-operable Mode, included in the EVS codec

ANBR	Access Network Bitrate Recommendation

ANBRQ	Access Network Bitrate Recommendation Query

APP	APPlication-defined RTCP packet

ARQ	Automatic repeat ReQuest

AS	Application Server

ATCF	Access Transfer Control Function

ATGW	Access Transfer GateWay

AVC	Advanced Video Coding

BFCP	Binary Floor Control Protocol

CCM	Codec Control Messages

CDF	Cumulative Distribution Function

cDRX	Connected Mode DRX

CMR	Codec Mode Request

cps	characters per second

CS	Circuit Switched

CSCF	Call Session Control Function

CTM	Cellular Text telephone Modem

CVO	Coordination of Video Orientation

DBI	Delay Budget Information

DRX	Discontinuous Reception

DTMF	Dual Tone Multi-Frequency

DTX	Discontinuous Transmission

ECN	Explicit Congestion Notification

ECN-CE	ECN Congestion Experienced

ECT	ECN Capable Transport

eNodeB	E-UTRAN Node B

E-UTRAN	Evolved UTRAN

EVS	Enhanced Voice Services

FECC	Far End Camera Control

FIR	Full Intra Request

FLR	Frame Loss Rate

FoIP	Facsimile over IP

GIP	Generic IP access

GOB	Group Of Blocks

H-ARQ	Hybrid - ARQ

HEVC	High Efficiency Video Coding

HSPA	High Speed Packet Access

ICM	Initial Codec Mode

IDR	Instantaneous Decoding Refresh

IFP	Internet Facsimile Protocol

IFT	Internet Facsimile Transfer

IMS	IP Multimedia Subsystem

IP	Internet Protocol

IPv4	Internet Protocol version 4

IRAP	Intra Random Access Point

ITU-T	International Telecommunications Union - Telecommunications

JBM	Jitter Buffer Management

MGCF	Media Gateway Control Function

MGW	Media GateWay

MIME	Multipurpose Internet Mail Extensions

MO	Management Object

MPEG	Moving Picture Experts Group

MRFC	Media Resource Function Controller

MRFP	Media Resource Function Processor

MSMTSI	Multi-Stream Multimedia Telephony Service for IMS

MSRP	Message Session Relay Protocol

MTSI	Multimedia Telephony Service for IMS

MTU	Maximum Transfer Unit

NACK	Negative ACKnowledgment

NNI	Network-to-Network Interface

NTP	Network Time Protocol

PCM	Pulse Code Modulation

PDP	Packet Data Protocol

PLI	Picture Loss Indication

PLR	Packet Loss Ratio

POI	Point Of Interconnect

PSTN	Public Switched Telephone Network

PTZF	Pan, Tilt, Zoom and Focus

QCI	QoS Class Identifier

QMC	QoE Measurement Collection

QoE	Quality of Experience

QoS	Quality of Service

QP	Quantization Parameter

RoHC	Robust HeaderCompression

ROI	Region of Interest

RR	Receiver Report

RTCP	RTP Control Protocol

RTP	Real-time Transport Protocol

SB-ADPCM	Sub-Band Adaptive Differential PCM

SC-VBR	Source Controlled VBR

SDP	Session Description Protocol

SDPCapNeg	SDP Capability Negotiation

SID	SIlence Descriptor

SIP	Session Initiation Protocol

SR	Sender Report

SRVCC	Single Radio Voice Call Continuity

TFO	Tandem-Free Operation

TISPAN	Telecoms and Internet converged Services and Protocols for Advanced Network

TMMBN	Temporary Maximum Media Bit-rate Notification

TMMBR	Temporary Maximum Media Bit-rate Request

TrFO	Transcoder-Free Operation

UDP	User Datagram Protocol

UDPTL	Facsimile UDP Transport Layer (protocol)

UE	User Equipment

VoIP	Voice over IP

VOP	Video Object Plane

Second change

### 7.3.8	Delay Budget Information (DBI) Signaling

RAN delay budget reporting is specified in TS 36.331 [160] for E-UTRA and TS 38.331 [163] for NR. RAN delay budget reporting through the use of RRC signalling to eNB / gNB allows UEs to locally adjust air interface delay. Based on the reported delay budget information, a good coverage UE on the receiving end (i.e., the UE that contains the MTSI receiver) can reduce its air interface delay, e.g., by turning off CDRX or via other means. This additional delay budget can then be made available for the sending UE (i.e., the UE that contains the MTSI sender), and can be quite beneficial for the sending UE when it suffers from poor coverage. When the sending UE is in bad coverage, it would request the additional delay from its local eNB / gNB, and if granted, it would utilize the additional delay budget to improve the reliability of its uplink transmissions in order to reduce packet loss, e.g., via suitable repetition or retransmission mechanisms, and thereby improve end-to-end delay and quality performance.

While RAN-level delay budget reporting as defined in TS 36.331 [160] and TS 38.331 [163] allows UEs (i.e., MTSI sender and MTSI receiver) to locally adjust air interface delay, such a mechanism does not provide coordination between the UEs on an end-to-end basis. To alleviate this issue, this clause defines RTCP signalling to realize the following capabilities on signalling of delay budget information (DBI) across UEs: (i) an MTSI receiver can indicate available delay budget to an MTSI sender, and (ii) an MTSI sender can explicitly request delay budget from an MTSI receiver.

More specifically, the RTCP-based signalling of DBI is composed of a dedicated RTCP feedback (FB) message type to carry available additional delay budget during the RTP streaming of media, signalled from the MTSI receiver to the MTSI sender. In addition, the defined RTCP feedback message type may also be used to carry requested additional delay budget during the RTP streaming of media, signalled from the MTSI sender to the MTSI receiver.

A corresponding dedicated SDP parameter on the RTCP-based ability to signal available or requested additional delay budget during the IMS/SIP based capability negotiations is also defined, as described in clause 6.2.8.

Such RTCP-based signaling of DBI can also be used by an MTSI receiver to indicate delay budget availability created via other means such as jitter buffer size adaptation as mentioned in clause 8.2.1.

The signalling of available or requested additional delay budget information (DBI) shall use RTCP feedback messages as specified in IETF RFC 4585 [40]. The RTCP feedback message is identified by PT (payload type) = RTPFB (205) which refers to RTP-specific feedback message. FMT (feedback message type) shall be set to the value '10' for delay budget information (DBI). The RTCP feedback method may involve signalling of available or requested additional delay budget in both of the immediate feedback and early RTCP modes.

As such, the RTCP feedback message shall be sent from the MTSI receiver to the MTSI sender to convey to the sender the available additional delay budget from the perspective of the receiver. The recipient UE of the RTCP feedback message (i.e., the UE containing the MTSI sender) may then use this information in determining how much delay budget it may request from its eNB / gNB over the RAN interface, e.g. by using RRC signalling based on UEAssistanceInformation as defined in TS 36.331 [160] and TS 38.331 [163].

The FCI (feedback control information) format shall be as follows. The FCI shall contain exactly one instance of the available additional delay budget information, composed of the following parameters:

-	Available additional delay budget delay - specified in milliseconds (16 bits)

-	Sign 's' for the additional delay budget delay and whether this is positive or negative– specified as a Boolean (1 bit)

-	Query 'q' for additional delay budget – specified as a Boolean (1 bit)

The sign value, 's' may be positive, indicated by ‘1’ or negative, indicated by ‘0’. Essentially, when the additional delay parameter takes on a positive value, the UE indicates that there is additional delay budget available. In case the additional delay parameter takes on a negative value, the UE indicates that the available delay budget has been reduced. A sequence of RTCP feedback messages may be sent by the UE to report on the additional delay budget availability in increments.

When the MTSI receiver sends RTCP feedback messages indicating the available delay budget for the received RTP stream, the query parameter shall be to be set to '0'. When the MTSI sender sends RTCP feedback messages indicating the requested delay budget for the RTP stream sent from the MTSI sender to the MTSI receiver, the query parameter shall be set to '1'. In this case, the value of delay indicates the additional delay budget requested by the sender of the RTCP feedback message (i.e., the MTSI sender) for the RTP stream sent from the MTSI sender to the MTSI receiver.

The FCI for the proposed RTCP feedback message shall follow the following format where (i) 's' stands for the single-bit message on the sign of the additional delay parameter and (ii) 'q' stands for the single-bit message on query:

0                1                   2                   3
 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
|            delay              |s|q|     zero padding          |
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

The high byte of delay shall be followed by the low byte, where the low byte holds the least significant bits.

An MTSI receiver shall not indicate available delay budget to an MTSI sender via DBI signalling more frequently than once every N round-trip times (RTT). [N is TBD]. If an MTSI receiver indicates available delay budget to an MTSI sender via DBI signalling, it shall commit to ensuring that the indicated delay budget amount is available to the MTSI sender for at least the duration of N round-trip times (RTT). An MTSI sender shall not request delay budget from an MTSI receiver via DBI signalling more frequently than once every N round-trip times (RTT).

Timing-wise, it is possible that DBI signalling may happen concurrently or asynchronously between the MTSI sender and MTSI receiver, i.e., the MTSI receiver may indicate available delay budget to the MTSI sender, while the MTSI sender may request delay budget from an MTSI receiver.

If the MTSI sender receives available delay budget information from an MTSI receiver via DBI signaling, it shall consider this delay budget to be available for its uplink over the duration of at least N round-trip times (RTT).  Thus, if an MTSI receiver has already indicated available delay budget to the MTSI sender via DBI signalling, reception of a DBI request from the MTSI sender during any time within the time window of N round-trip times (RTT) shall not trigger any further DBI signalling from the MTSI receiver to the MTSI sender on the available delay budget at any time sooner than N round-trip times (RTT).

Once the period of N round-trip times is over, if the available delay budget has changed, the MTSI receiver shall inform the MTSI sender on the new delay budget availability (as a relative value as explained above) using DBI signalling. If the MTSI sender does not receive any new DBI signalling on the available delay budget from the MTSI receiver after the period of N round-trip times (RTT) is over, this shall mean that the MTSI receiver is still committed to providing the same amount of available delay budget indicated to the MTSI sender earlier via DBI signalling.

Likewise, if the MTSI sender no longer needs the additional delay budget it has requested earlier or has a delay budget request that is different from what it had requested earlier, it shall inform the MTSI receiver about the new delay budget request (as a relative value as explained above) via DBI signalling. If the MTSI receiver does not receive any new DBI signalling on the requested delay budget from the MTSI sender after the period of N round-trip times (RTT) is over, this shall mean that the MTSI sender is still requesting the same amount of delay budget indicated to the MTSI receiver earlier via DBI signalling.

Third change

# R.2	Delay Budget Information (DBI)

The new RTCP feedback types for Delay Budget Information (DBI) can be registered with IANA as follows:

Value name: 3gpp-delay-budget

Long name: Available or requested delay budget specified in milliseconds

Reference: 3GPP TS 26.114.

The following value can be registered as one FMT value in the "FMT Values for RTPFB Payload Types" registry http://www.iana.org/assignments/rtp-parameters:

Name: DBI

Long name: Delay Budget Information (DBI)

Value: 10

Reference: 3GPP TS 26.114.

End of document