3GPP TSG-SA4 MTSI / MBS Adhoc	S4-AHM453

17th January 2019	revision of S4-181292

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.238 | CR | 0003 | rev | 1 | Current version: | 15.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Remote Control Interface |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | E-FLUS |  |  |  |  |  | Date: |  |  | 13.11.2018 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR contains the implementation of the agreements from TD S4-181173 into the architecture section of TS 26.238. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of the FLUS remote control function |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.1, 4.2.3, 4.4.1, 5.3.2.2a, 7.5a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** First Change ***

### 4.2.1	General

Figure 4.2-1 depicts the architecture showing the relevant entities for providing an uplink streaming service.

Figure 4.2-1: FLUS architecture

The uplink streaming service architecture is based on a FLUS source located in a UE and a FLUS sink located in either another UE or in the network.

The FLUS source receives media content from one or more capture devices. In the context of this specification, the capture devices are considered as parts of a UE or are connected to it.

When the FLUS sink is located in a UE, the FLUS sink shall forward media content to a decoding and rendering function.

When the FLUS sink is located in the network, the FLUS sink may forward media content to a processing or distribution sub-function. The processing and distribution sub-functions are not in scope of the present specification. The FLUS sink may act as a Media Gateway Function (MGW) and/or an Application Function (AF).

The F reference point connects a FLUS source and a FLUS sink. The F reference point enables the FLUS source to establish and control a single FLUS session.

The F reference point also enables the FLUS sink and the FLUS source to mutually authenticate and authorize each other.

FLUS source and sink are each decomposable into FLUS media (F-U end-point), FLUS control (F-C end-point) and FLUS remote control ( F-RC end-point) subfunctions. The FLUS remote control function is optional. For MTSI-based FLUS, the FLUS remote control function is not present.

Regarding colocation of FLUS Source subfunctions:

- The FLUS control function (F-C end-point) could be collocated with the FLUS media function (F-U end-point) on the same node (the UE) or reside in a different entity.

- The FLUS remote control subfunction (when present) shall be collocated with the FLUS media subfunction on the same UE.

- The FLUS source may connect to multiple FLUS remote control subfunctions.

Regarding colocation of FLUS Sink subfunctions:

- The FLUS control and the FLUS media function should be collocated on the same node.

- The FLUS remote control function, when present, may operate independently from the FLUS media function.

The F reference point shall support security function for confidentiality protection for all, the FLUS control (F-C) FLUS media (F-U) and FLUS remote control (F-RC).

Details of the FLUS control, FLUS remote control and FLUS media functions are shown in Figure 4.2-2. The arrows indicate the main information flow.

Figure 4.2-2: Sub-functions of FLUS

Note that in the above diagram, hatched-filled boxes represent FLUS control plane functionality, solid line boxes represent mandatory functionality, and dashed-line boxes corresponds to optional functionality. Also, note that F-C, F-RC and F-U denote FLUS control, FLUS remote control and FLUS user plane functionalities, respectively, and do not represent reference points.

FLUS control and F-C: FLUS control plane functionality including the associated processing by FLUS sink of the uploaded media for subsequent downstream distribution, plus FLUS media instantiation selection. F-C may also support configuration of static metadata for the session.

FLUS remote control and F-RC: FLUS remote control functionality provides instructions to the FLUS Source. The behavaiour of the FLUS source depends on the FLUS remote control instruction. Example instructions are start or stop of the FLUS media upstreaming process in FLUS Source.

FLUS media and F-U: FLUS user plane functionality which includes setup of one or more media sessions and subsequent media data transmission via media streams. In some cases, a media session establishment protocol (e.g. IMS session set-up for MTSI-based FLUS instantiation) is necessary.

NOTE: F-C is not needed when the FLUS sink is an MTSI client that is only capable of rendering. In such event, logical control plane functions such as media and session descriptions and support for FLUS session establishment are encapsulated in FLUS user plane functionality.

F-C is used to establish and control the FLUS session. F-C allows the FLUS source to

- select a FLUS media instantiation,

- provision static metadata associated with each media session present in the FLUS session,

- select and configure the processing and distribution sub-functions.

The FLUS media instantiation is defined as part of the FLUS session. The user plane (F-U) may also contain the media stream establishment procedures when needed. Multiple media streams may be established for one FLUS session.

A media stream may contain media components of a single content type, e.g., audio, or media components of different content types, e.g., audio and video. A FLUS session may be composed of more than one media stream containing the same content type, e.g., multiple media streams of video.

*** Last Change ***

### 4.2.3	Uplink streaming for PSS-based distribution

The architecture of uplink streaming for subsequent PSS distribution is depicted in Figure 4.2-4.

Figure 4.2-4: Uplink streaming for PSS

The PSS Content Source is located on the UE side and contains the FLUS source.

*** Next Change ***

### 4.4.1	General

A FLUS session may include one or more media streams. Media streams are time bounded to the FLUS session they belong to. When a media stream is active, the FLUS source can send media content to the FLUS sink.

The establishment and / or teardown of a media session can be remoted controlled using FLUS remote control instruction functionality.

Figure 4.4-1 depicts an example relationship of a FLUS session with a single media session containing two media streams.

Figure 4.4-1: FLUS, media sessions and media streams

When the FLUS sink is located in a UE and the UE renders the received media content directly, the FLUS session may be implicitly present, e.g. it may be realized through IMS/MTSI.

When the FLUS sink is located on the network side and provides Media Gateway functionality, the FLUS session is used to select the media session instantiation and to configure any processing and distribution related sub-functions.

*** Next Change ***

#### 5.3.2.2a	FLUS remote control session establishment

When FLUS remote control is needed, the FLUS source is provisioned with the relevant information to establish one or more FLUS remote control sessions (i.e. the HTTPS URL). The FLUS source creates the remote control session and then listens for incoming remote control instructions.

Figure 4.4-2: FLUS F-RC Session creation

1. The FLUS remote control session is created.

2. On successful creation, the FLUS source receives a positive acknolwedge.

The FLUS Source starts waiting for FLUS remote control instructions.

3. The FLUS Source receives a F-RC commands (here a start command instruction) and acts accordingly

*** Next Change ***

## 7.5a	Remote control session establishment

This paragraph defines the establishment of a FLUS remote control session using WebSocket Protocol as specified in RFC 6455.

To establish the WebSocket based FLUS remote control session, the FLUS source shall use the HTTPS GET method on the provisioned resource as follows:

-	the request URI with the “path” can be set to e.g. “/flus/v1.0/rc/session/{session}”. The URL for the remote control session is provided via provisioning.

-	the Host field is set to the FQDN of the remote controlling host.

-	when the Upgrade fiels is present and contains “websocket” as value, the client establishes a web socket connection.

The message format for the remote control commands shall be json formated.

*** Next Change ***

# 7a	Uplink Streaming Remote Control Interface

## 7a.1	Introduction

A FLUS remote control session allows sending remote control instructions to a FLUS Source. The FLUS source behaviour depends on the FLUS remote control instruction.

This clause defines the FLUTE remote control instructions and the resulting behaviour of the FLUS source.

## 7a.2	General Instruction Format

Remote control instruction are formated in JavaScript Notation (json). The minimal remote control instuction message contains a message_type value pair and a timing json object. The message_type value is a string. The timing object contains at least a creation_timestamp, which represents the timestamp at which the instruction message was created. The timing object may contain an action_timestamp, which represents the timestamp at which the instruction shall be executed. All timestamps within the timing object shall be ISO 8601 formated.

Example:

{

"timing": {

"creation_timestamp": "2019-01-14T13:28:06.419Z",

"action_timestamp": "2019-01-14T13:30:00.000Z"

},

"message_type": "start"

}

## 7a.3	Start Instruction Message

The start remote control instruction represents a trigger instruction to start the live uplink streaming process. The FLUS source shall start streaming uplink data. The value of the message_type property shall be set to “start”.

When the timing object does not contain an action_timestamp property, then the FLUS source shall perform the start of the uplink streaming process at instruction message reception.

When the timing object contains an action_timestamp property, then the FLUS source shall perform the start of the uplink streaming process according to the action_timestamp value. The F-U media plane should be established before.

## 7a.4	Stop Instruction Message

The stop remote control instruction represents a trigger instruction to stop the live uplink streaming process. The FLUS source shall stop streaming uplink data. The value of the message_type property shall be set to “stop”.

When the timing object does not contain an action_timestamp property, then the FLUS source shall perform the stop of the uplink streaming process at instruction message reception.

When the timing object contains an action_timestamp property, then the FLUS source shall perform the stop of the uplink streaming process according to the action_timestamp value.

*** Last Change ***