3GPP TSG-SA4 MTSI Adhoc on E_FLUS	S4-AHM469

# Telco, 16th May 2019																revision of S4-190512

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (draft) CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.238 | CR | 0003 | rev | 6 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Remote Assist / Control Interface |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM, Sony Europe B.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | E-FLUS |  |  |  |  |  | Date: |  |  | 22/03/2019 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR contains the implementation of the agreements from TD S4-181173 into the architecture section of TS 26.238 regarding the FLUS assistance (F-A) and FLUS remote control interface functions (F-RC). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of the FLUS assistance and FLUS remote control function |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.2.1, 4.4.1, 4.5.6 (new), 5.3.2.1, 5.3.2.2, 5.3.2.3, 5.3.2.4, 5.3.2.5, 5.3.2.6, 5.3.2.7, 5.3.2.8 (new), 5.3.2.9 (new), 5.3.2.10 (new), 5.3.2.11 (new), 5.3.2.12 (new), 5.3.2.13 (new), 5.3.2.14 (new), 5.3.2.15 (new), 5.3.2.16 (new), 5.3.4 (new), 5.3.5 (new), 5.3.5 (new), 5.3.6 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** First Change ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	Recommendation ITU-R BS.2051-1 (06/2017): "Advanced Sound System for Programme Production".

[3]	ISO/IEC DIS 23090-2: "Omnidirectional Media Format. N16824".

[4]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction".

[5]	3GPP TS 26.235: "Transparent end-to-end Packet-switched Streaming Service (PSS); Protocols and codecs".

[6]	3GPP TS 24.147: "Conferencing using the IM Multimedia (IM) Core Network (CN) subsystem; Stage 3”.

[7]	3GPP TS 23.003: "Numbering, addressing and identification".

[8]	3GPP TS 33.203: "3G security; Access security for IP-based services".

[9]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security".

[10]	3GPP TS 33.328: "IP Multimedia Subsystem (IMS) media plane security".

[11]	IETF RFC 7231: "Hypertext transfer protocol (HTTP/1.1): Semantics and Content".

[12]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[13]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[14]	3GPP TS 23.682: " Architecture enhancements to facilitate communications with packet data networks and applications".

[15]	3GPP TS 23.501: "System Architecture for the 5G System".

[16]	ISO/IEC CD 23090-8, MPEG-I Part 8: "Network-Based Media Processing".

*** Next Change ***

### 4.2.1	General

Figure 4.2-1 depicts the architecture showing the relevant entities for providing an uplink streaming service.

Figure 4.2-1: FLUS architecture

The uplink streaming service architecture is based on a FLUS source located in a UE and a FLUS sink located in either another UE or in the network.

The FLUS source receives media content from one or more capture devices. In the context of this specification, the capture devices are considered as parts of a UE or are connected to it.

When the FLUS sink is located in a UE, the FLUS sink shall forward media content to a decoding and rendering function.

When the FLUS sink is located in the network, the FLUS sink may forward media content to a processing or distribution sub-function. The processing and distribution sub-functions are not in scope of the present specification. The FLUS sink may act as a Media Gateway Function (MGW) and/or an Application Function (AF).

The F reference point connects a FLUS source and a FLUS sink. The F reference point enables the establishment and control a single FLUS session.

The F reference point also enables the FLUS sink and the FLUS source to mutually authenticate and authorize each other.

FLUS source and FLUS sink are each split into

-	Media Source and Sink (F-U end-points),

-	Control Source and Sink (F-C end-points),

-	Remote Controller and Remote Control Target (F-RC end-points) and

-	Assistance Sender and Receiver (F-A end-points).

The UE, the FLUS Source and the FLUS sink are considered to be logical devices. Functions are not required to be located in the same physical device. Functions can be spread over multiple physical devices and interconnected via other interfaces.

There may be multiple F-A and F-RC end-points for a single FLUS Source. F-A and F-RC end-points are independent from a FLUS Sink and depend on the offered service.

The F reference point shall support security function for confidentiality protection for all subfunctions.

Details of the functions are shown in Figure 4.2-2.

Figure 4.2-2: Sub-functions of FLUS

Note that in the above diagram, hatched-filled boxes represent FLUS control plane functionality, solid line boxes represent mandatory functionality, and dashed-line boxes corresponds to optional functionality. Also, note that F-C and F-U denote FLUS control and FLUS user plane functionalities, respectively, and do not represent reference points.

Control Source and Sink (F-C): FLUS control plane functionality including the associated processing by FLUS sink of the uploaded media for subsequent downstream distribution, plus FLUS media instantiation selection. F-C may also support configuration of static metadata for the session.

Assistance Sender and Receiver (F-A): The Assistance Receiver on the FLUS Source receives assistance messages. The messages affect the behaviour of the FLUS media function in the FLUS Source. Examples of remotely-provided assist information may pertain to network related conditions, viewership or engagement information from content recipients, or user preference data. An example recommendation issued by the UE-based assistance to the Media Source is to only upload the first 5 seconds of video to the network FLUS sink, due to current absence of viewership of the live uplink streaming content.

Remote Controller and Remote Control Target (F-RC): The Remote Control Target on the FLUS Source receives control messages. The messages affect the behaviour of the Media Source in the FLUS Source. Examples of commands issued to the Remote Control Target are start or stop of the media upstreaming process in FLUS Source.

Media Source and Sink (F-U): Media plane functionality which includes setup of one or more media sessions and subsequent media data transmission via media streams. In some cases, a media session establishment protocol (e.g. IMS session set-up for MTSI-based FLUS instantiation) is necessary.

NOTE: F-C is not needed when the FLUS sink is an MTSI client that is only capable of rendering. In such event, logical control plane functions such as media and session descriptions and support for FLUS session establishment are encapsulated in FLUS user plane functionality.

F-C and optionally F-RC are used to establish and control the FLUS session. F-C allows the Control Source to

- select a FLUS media instantiation,

- provision static metadata associated with each media session present in the FLUS session,

- select and configure the processing and distribution sub-functions.

The FLUS media instantiation is defined as part of the FLUS session. The user plane (F-U) may also contain the media stream establishment procedures when needed. Multiple media streams may be established for one FLUS session.

A media stream may contain media components of a single content type, e.g., audio, or media components of different content types, e.g., audio and video. A FLUS session may be composed of more than one media stream containing the same content type, e.g., multiple media streams of video.

*** Next Change ***

### 4.4.1	General

A FLUS session may include one or more media streams. Media streams are time bounded to the FLUS session they belong to. When a media stream is active, the FLUS source can send media content to the FLUS sink.

The establishment and / or teardown of a media session can be remotely controlled / influenced using remote control functionality.

Figure 4.4-1 depicts an example relationship of a FLUS session with a single media session containing two media streams.

Figure 4.4-1: FLUS, media sessions and media streams

When the FLUS sink is located in a UE and the UE renders the received media content directly, the FLUS session may be implicitly present, e.g. it may be realized through IMS/MTSI.

When the FLUS sink is located on the network side and provides Media Gateway functionality, the FLUS session is used to select the media session instantiation and to configure any processing and distribution related sub-functions.

*** Next Change***

### 4.5.6	Media production FLUS source system

In the media production use case for FLUS, typically it is the media production centre that controls the FLUS source devices that deliver media streams for the media production process. The media production centre typically is behind the FLUS sink, which acts as a sort of Media Gateway Function, and the Remote Controller function which acts as proxy for the control of the FLUS source devices.

When a FLUS source is set up and prepared for the event, it proceeds, either by manual trigger or autonomously, to register with the FLUS sink, the Remote Controller function and remote production centre. Once a FLUS source is known to the media production center, the remote production centre can take full control of that device and the FLUS sessions involving that FLUS source is given to the FLUS sink, as a proxy for the media production centre. This includes the initiation, adaptation and termination of FLUS sessions.

*** Next Change ***

#### 5.3.2.1	FLUS sink capability discovery

This procedure allows a Control Source to discover capabilities of the FLUS sink.

Figure 5.3.2.1-1: Get current FLUS sink capabilities

1. The Control Source sends the FLUS sink capability request to a known URL of the FLUS sink.

2. The Control Sink provides a list of FLUS sink capabilities in response.

#### 5.3.2.2	FLUS session establishment on FLUS Sink

It is assumed that the Control Source has selected a FLUS sink and acquired the necessary information to establish an F-C connection to that the Control Sink of the FLUS sink (i.e. the HTTPS URL).

The procedure allows a Control Source to create a new FLUS session on the FLUS Sink. Session configuration properties and in particular FLUS media instantiation selection is added in subsequent procedures.

Figure 5.3.2.2-1: FLUS session creation

1. The FLUS session is created. The Control Source provides a valid access token.

2. On successful creation, the Control Sink of the FLUS sink responds with the resource id of the session. FLUS session properties are fetched and modified with subsequent transactions.

### 5.3.2.3	Get FLUS session properties on FLUS Sink

The procedure allows a Control Source to fetch the current FLUS Session configuration of a FLUS session.

Figure 5.3.2.3-1: Get current FLUS session properties

1. The Control Source sends along with the session property request, the access token and the resource id of the FLUS session to the Control Sink of a FLUS Sink.

2. The Control Sink provides the FLUS session properties in response.

#### 5.3.2.4	FLUS session update on FLUS Sink

The procedure allows a Control Source to update the current configuration of the FLUS session.

Figure 5.3.2.4-1: FLUS session update

The Control Source may first fetch the current FLUS session configuration using the Get FLUS session properties procedure.

1. The Control Source modifies the properties of the session resource. The procedure may allow modification of individual properties or all properties.

2. The Control Sink updates the resource identified by the id of the session.

#### 5.3.2.5	FLUS session termination

The Control Source may explicitly terminate a FLUS session and all its provisioned and active media sessions. Alternatively, the FLUS session is automatically terminated, when the last media session of the FLUS session is terminated. The procedure allows a Control Source to terminate a FLUS session. All media streams will be terminated automatically with the termination of the FLUS session.

Figure 5.3.2.5-1: FLUS session termination

1. The Control Source sends the terminate FLUS session command. The access token and the resource id of the FLUS session is provided as input.

2. The Control Sink terminates the FLUS session, including all active media streams and acknowledges the reception of the request.

#### 5.3.2.6	Void

#### 5.3.2.7	Void

#### 5.3.2.8	Session establishment for FLUS Assistance

When assistance is needed, the Assistance Receiver of the FLUS source is provisioned with the relevant information to establish one or more F-A sessions. The Assistance Receiver creates the F-A session and then listens for incoming messages.

Figure 5.3.2.8-1: FLUS F-A Session creation

1. The F-A session is established triggered by the Assistance Receiver in the FLUS Source.

2. On successful creation, the Assistance Receiver receives a positive acknolwedge.

The Assistance Receiver starts waiting for assistance related messages.

#### 5.3.2.9	Receive Assistance Message

This procedure allows receiving of assistance messages.

Figure 5.3.2.9-1 shows the sequence diagram of sending assistance information using the F-A.

Figure 5.3.2.10-1: Send Assistance Message

1. The Assistance Sender sends the assistance message to the Assictance Receiver of the FLUS source.

2. The Assistance Receive acknowledges the reception.

3. The Assistance Receiver triggers the appropriate action.

#### 5.3.2.10	Session establishment for remote control

When remote control is needed, the Remote Control Target in the FLUS Source is provisioned with the relevant information to establish one or more F-RC sessions. The Remote Control Target creates the F-RC sessions and then listens for incoming messages.

Figure 5.3.2.10-1: FLUS F-RC Session creation

1. The F-RC session is established triggered by the Remote Control Target. Properties.

2. On successful creation, the Remote Control Target receives a positive acknolwedge.

The Remote Control Target starts waiting for remote control related messages.

#### 5.3.2.11	FLUS source capability discovery

This procedure allows discover capabilities of the FLUS source.

Figure 5.3.2.11-1 shows the sequence diagram of the FLUS source capability discovery message exchange using the F-RC.

The set of FLUS source capabilities is specified in clause 5.3.5.

Figure 5.3.2.11-1: Get current FLUS source capabilities

1. The Remote Controller functions sends the FLUS source capability request the Remote Control Target of the FLUS source.

2. The Remote Control Target provides a list of FLUS source capabilities in response.

#### 5.3.2.12	Remote FLUS session establishment

When the FLUS Source has established a F-RC session, the Remote Controller function can establish a FLUS session on the FLUS Source by providing relevant FLUS Sink parameters. Session configuration properties and in particular FLUS media instantiation selection is added in subsequent procedures.

Figure 5.3.2.12-1 shows the sequence diagram of the remote FLUS session establishment message exchange.

Figure 5.3.2.12-1: Remote FLUS session creation

1. The FLUS session is created. The Remote Controller function provides a valid access token.

2. On successful creation, the Remote Control Target acknowledges the creation. FLUS session properties (such as the FLUS Session information to identify the FLUS Sink resources) are fetched and modified with subsequent transactions.

### 5.3.2.13	Get Remote FLUS session properties

This procedure allows a Remote Controller function to obtain the current configuration of the FLUS session from the FLUS source.

Figure 5.3.2.13-1 shows the sequence diagram of the FLUS session properties retrieval message exchange.

Figure 5.3.2.13-1: Get current FLUS session properties

1. The Remote Controller function sends along with the session property request, the access token.

2. The Remote Control Target of the FLUS source provides the FLUS session properties in response. When no FLUS session is defined on the FLUS Source, an empty FLUS session property list is responded.

#### 5.3.2.14	Remote FLUS session update

The procedure allows a Remote Controller function to update the current configuration of the FLUS session of a FLUS Source.

Figure 5.3.2.14-1 shows the sequence diagram of the Remote FLUS session update message exchange.

Figure 5.3.2.14-1: Remote FLUS session update

1. The Remote Controller function modifies any or all of the properties of the existing FLUS session resource.

2. The FLUS source updates the resource identified by the id of the session.

#### 5.3.2.15	FLUS source status report request

This procedure allows a Remote Controller function to obtain the current status of the FLUS source. The current status is reported as a list of its capabilities with the current status of each one. Each parameter may be elaborated with more information, e.g. when on battery power the charge level in per-cent may be reported.

Figure 5.3.2.15-1 shows the sequence diagram of the FLUS source status report request message exchange.

Figure 5.3.2.15-1: Get current FLUS source status report

1. The Remote Controller function sends the FLUS source status request to the FLUS source.

2. The Remote Control Target of the FLUS source provides its current status as a list of its capabilities with the status of each one in response.

#### 5.3.2.16	Remote FLUS session termination

This procedure allows a Remote Controller function to terminate a FLUS session in a FLUS Source. All active media sessions and streams will be terminated automatically with the termination of the FLUS session.

Figure 5.3.2.16-1 shows the sequence diagram of the Remote FLUS session termination message exchange.

Figure 5.3.2.16-1: Remote FLUS session termination

1. The Remote Controller function sends the terminate FLUS session command. The access token is provided as input.

2. The Remote Control Target of the FLUS source terminates the FLUS session, including all active media streams. The Remote Control Target may discard the FLUS session configuration.

3. The Remote Control Target acknowledges the execution of the request.

*** Next Change ***

*** Next Change ***

### 5.3.4	List of FLUS sink capabilities

The FLUS sink capabilities should include the features that are listed under 4.4.4.

### 5.3.5	FLUS source characterisation and capabilities

FLUS source devices are characterised so that the basic FLUS-related functionality can be made known to the FLUS sink.

The following FLUS source properties are specified:

•	Available video format(s);

•	Available audio format(s);

•	Available ancillary stream format(s): subtitles / captions, content metadata – for that which is not embedded in the media stream(s);

•	Connectivity: RAN, wired – and which system/s;

•	Remote control - one of: manual, remote, none – and which system/s;

•	Mobility - one of: fixed, on foot, ground vehicle, airborne vehicle, surface water vehicle, underwater;

•	Power – one of: Battery, battery with autonomous charging, mains.

The Remote Controller function uses the FLUS source capability discovery message exchange as specified in sub-clause 5.3.2.11 to retrieve the capabilities of the FLUS source and its attached media capture device(s).

The Remote Controller function uses the FLUS source status report request message exchange as specified in sub-clause 5.3.2.15 to retrieve the current status of the FLUS source device. The status is returned in the form of the capabilities of the FLUS source populated with the applicable setting to indicate the current state of the corresponding parameter.

Some parameters can conatin a particular capability setting, e.g. the current media format being transmitted. Other parameters contain one of the possible settings, possibly adding auxiliary information, e.g. when running on battery power, the current charge level and estimated operating time before needing to re-charge is indicated. When charging, the charge level and estimated time to full charge could be indicated.

#### 5.3.6	List of FLUS session properties

All FLUS session properties, except for the resource id, are always carried in an HTTP message body. The access-token is always carried as part of HTTP headers. Except for the FLUS session creation request (where the id is not present), the resource id shall be present in the URL of all requests that relate to a specific FLUS session.

In the table below, the following assertions are made:

-	Table header: C stands for Create FLUS session procedure, G is for Get FLUS session properties procedure, U is for Update FLUS session properties procedure and T is for Terminate FLUS session procedure. “I”, and “O” respectively denote “request” (going Into the FLUS sink), and response (going Out of the FLUS sink).

-	Optional (“O”) means that the property may or may not be sent/received during a REST transaction. It does not necessarily mean that the property is optional. It is possible, for example, that a session is not yet active because the FLUS source has not set the property in any previous update transaction using the PUT or PATCH HTTP method, as opposed to representing a hint on the importance of the property for the FLUS sink.

-	A property marked as optional (O) in a request message may be present in the request. When not present in the request body, the property, if present in the FLUS sink, will not be updated.

-	A property marked as optional (O) in a response message is only present in the response when a value is assigned or changed by the FLUS sink.

-	A property marked as mandatory (M) in a response message is always present in the response. The FLUS sink provides defaults, which may be modified subsequently by the content provider.

-	A blank cell in the table means “forbidden” (the property cannot be added to the request or returned by the FLUS sink, depending on the transaction direction).

Table 5.3.2.7-1: List of FLUS session properties

| Property Name | Property Description | C I | C O | G I | G O | U I | U O | T I |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| id | Identifier of the FLUS session resource. Note, the id is only provided within an HTTP body during the Create FLUS session response. Otherwise, the id is present in the message URL to identify the resource in the FLUS sink. Type; Unit; Default. Integer; None; N/A. |  | M |  |  |  |  |  |
| fu_instantiation | Identifier of the FLUS media instantiation that is used by this FLUS session. Vendor specific enumeration values shall start with ‘vnd-‘ followed by a unique vendor name and optionally followed by additional characters. The F-U instantiation shall be provided as a globally unique URN. Type; Unit; Default. URI; None; All. |  |  |  | M | O |  |  |
| entrypoint_URL | Entry point URL information (e.g., SIP URL) for establishing the F-U connection to start the media streaming. Details on the Entrypoint URL is F-U instantiation specific. |  |  |  |  |  |  |  |
| processing_description | This object provides a media processing description document that defines the post processing pipeline that the FLUS sink shall apply to received media components. The pipeline description may also set the distribution target (incl FLUS sink storage) for the media. The Object has the following properties: - type: the MIME type of the workflow description document - document: the media processing document may be embedded in this element. The document shall be base64 encoded. - url: the URL to the media processing document. The type and either the document property or the url property shall be provided. The following formats are supported: - The MPEG NBMP Workflow Description Document, as defined in [16], which describes the requested media processing and the desired distribution mechanism after the processing has been performed. The type field shall be set to “application/mpeg-media-processing+json” |  |  |  | O | O |  |  |

*** Last Change ***