3GPP TSG-SA4 Meeting #107 	S4-AHM506

# MTSI SWG Telco, RTCPVer #1,  Dec 4, 2019, 16 - 18 CET

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.139 | CR | xxx | rev | - | Current version: | 0.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Basic RTCP Interval |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | RTCPVer |  |  |  |  |  | Date: |  |  | 2019-11-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The basic RTCP interval is typically constant (non-randomized) in most point-to-point implementations, violating IETF RFC 3550. However, using a constant RTCP interval has no negative impact in RTP sessions with few participants and/or few RTP streams, which motivates introducing an exception. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Separating the basic RTCP interval test to one mandatory, allowing a non-randomized interval, and one conditionally mandatory test for multi-party / multi-stream scenarios aligned with IETF RFC 3550 that requires interval randomization. The text below assumes that changes to 6.2.3.2 in S4-AHMxxx are accepted. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Almost every deployed 3GPP RTP implementation would fail this very basic RTCP interval test. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4-AHM506 Initial pCR |  |  |  |  |  |  |  |  |

First Change

#### [6.2.3.2	RTCP Basic Interval Test

Purpose:	Test if the non-reduced, minimum time interval with which RTCP packets are sent complies with IETF RFC 3550 [2], excluding prescribed randomization of the time interval.

Status:	Mandatory

Preconditions:	1) The system under test is pre-configured to not send any RTP packets, e.g. set as receive-only.
2) The system under test is pre-configured with default RTCP timing (based on 5% RTCP bandwidth), e.g. neither "b=RS" nor "b=RR" are included in the SDP answer.
3) The system under test is set to not use the reduced minimum RTCP interval for high bandwidth sessions (see test 6.2.3.4).
4) The system under test is set to use an RTP session bandwidth of at least 1000 kbit/s to ensure that the minimum, non-reduced RTCP interval is used, e.g. pre-configured or negotiated in SDP through "b=AS:1000" being included in the SDP answer. If that high RTP session bandwidth cannot be set, the maximum possible RTP session bandwidth should instead be set.
5) The data injection to the system under test is set to not send any RTP packets, such that the system under test does not receive any RTP packets (that is, no RTP packets will be sent to or from the system under test).

Test procedure:	1) Observe the system under test output.
2) Note the reception time (ti) of each RTCP packet sent from the system under test.
3) Count the number of RTCP packets, n, sent from the system under test.
4) For each pair of subsequent RTCP packets, calculate the difference in reception time (called the interval), Ii=ti-ti-1, iϵ2..n.
5) Calculate the average RTCP interval I=1n-1i=2nIi.

Stop condition:	At least 10 minutes, or 100 RTCP packets are received, whichever is less.

Pass criteria:	1) 2 seconds≤miniIi≤2.5 seconds2 seconds≤miniIi≤I. 
2) 5.5 seconds≤maxiIi≤7 secondsI≤maxiIi≤7 seconds. 
3) 4.5 seconds≤I≤5.5 seconds.

Comments:	This test allows an exception to IETF RFC 3550 [2] and the RTP testing strategies in [3] stating that an implementation fails the test if the RTCP packets are sent with a constant interval (not randomized).

#### 6.2.3.2a	RTCP Multiparty and Multistream Interval Test

[Editor’s note: This and subsequent clauses to be re-numbered.]

Purpose:	Test if the non-reduced, minimum time interval with which RTCP packets are sent complies with IETF RFC 3550 [2], including prescribed randomization of the time interval.

Status:	Conditionally Mandatory (precondition 1)

Preconditions:	1) The system under test is targeted for RTP multi-party and / or RTP multi-stream scenarios (see test 6.2.3.2 for other scenarios).
2) The system under test is pre-configured to not send any RTP packets, e.g. set as receive-only.
3) The system under test is pre-configured with default RTCP timing (based on 5% RTCP bandwidth), e.g. neither "b=RS" nor "b=RR" are included in the SDP answer.
4) The system under test is set to not use the reduced minimum RTCP interval for high bandwidth sessions (see test 6.2.3.4).
5) The system under test is set to use an RTP session bandwidth of at least 1000 kbit/s to ensure that the minimum, non-reduced RTCP interval is used, e.g. pre-configured or negotiated in SDP through "b=AS:1000" being included in the SDP answer. If that high RTP session bandwidth cannot be set, the maximum possible RTP session bandwidth should instead be set.
6) The data injection to the system under test is set to not send any RTP packets, such that the system under test does not receive any RTP packets (that is, no RTP packets will be sent to or from the system under test).

Test procedure:	1) Observe the system under test output.
2) Note the reception time (ti) of each RTCP packet sent from the system under test.
3) Count the number of RTCP packets, n, sent from the system under test.
4) For each pair of subsequent RTCP packets, calculate the difference in reception time (called the interval), Ii=ti-ti-1, iϵ2..n.
5) Calculate the average RTCP interval I=1n-1i=2nIi.

Stop condition:	At least 10 minutes, or 100 RTCP packets are received, whichever is less.

Pass criteria:	1) 2 seconds≤miniIi≤2.5 seconds.
2) 5.5 seconds≤maxiIi≤7 seconds.
3) 4.5 seconds≤I≤5.5 seconds.

Comments:	The RTP testing strategies in [3] recommends the test to run for at least 20 minutes and highlights that an implementation fails the test if the RTCP packets are sent with a constant interval (not randomized).

End of document