Source:	Ericsson LM

Title:	Characterization of 6 DoF rendering and extended metadata for ISM

Agenda Item:	2.4

Document for:	Agreement

Introduction

The IVAS decoder and renderer operation supports head-tracking of the listener, such that the head orientation is taken into account in the spatial rendering. Rendering the listener orientation, 3 degrees of freedom (DoF), greatly enhances the immersive experience in the rendered audio scene. The discrete ISM operation further permits moving the listener by injecting a listener position, resulting in 6 DoF rendering. When operating IVAS ISM at bit rates from 64 kbps and up, an extended metadata set is natively supported in the IVAS bitstream, adding object orientation and distance from the origin to each audio object. In combination with a directivity pattern set by a renderer configuration, the immersive experience of object audio rendering is further enhanced. This contribution proposes an experiment to be included in the test plan (IVAS-8b) [1] to characterize this feature, addressing the "6 degrees-of-freedom (DoF) and directivity” item of aspects to test according to clause 4.2 of [1].

Discussion

The IVAS renderer for discrete ISM has an inherent support for 6 DoF rendering, by allowing the listener position to move. Since the HR filters are generated based on the relative position of the listener and the audio objects, adding the listener movement comes with practically no additional computational complexity. The source directivity and distance dependent attenuation provide a simple but effective way to improve the sense of being immersed in the audio scene.

The source believes a relevant reference for comparison would be to run the IVAS decoder and renderer in 3 DoF mode, to characterize the additional feature. This is consistent with the proposed experiment setup for the combined format Objects + MASA (OMASA) and Objects + SBA (OSBA), where the combined format is compared to two instances of IVAS providing the corresponding combination.

The IVAS 3 DoF mode would be implemented by disabling the extended metadata in the IVAS encoder, and by setting the listener position to zero in the head-tracking input used for this condition in the IVAS decoder.

To emphasize the feature of 6 DoF rendering, the input material is proposed to use 4 objects including both speech and other sound effects, both with and without synthetic reverb enabled. Both static objects and moving objects should be included, and suitable directivity patterns and reverb parameters would complement each test item. The material is assumed to be designed and contributed to the Material Collection entity, where the source volunteers to contribute to such material.

Proposal

It is proposed to insert the new clause 4.3.4 below after clause 4.3.3, clause G.19 below into Annex G of IVAS-8b.

- 4.3.4	6-DoF characterization MUSHRA Test

A MUSHRA test characterizing the 6-DoF and directivity feature is setup with the following conditions:

| Label | Condition | Bitrate [kbps] | DTX | Extended MD | Listener position |
| --- | --- | --- | --- | --- | --- |
| c01 | Reference | - | - | Yes | Yes |
| c02 | LP 3.5 kHz | - | - | Yes | Yes |
| c03 | IVAS | 64.0 | Off | No | No (orientation only) |
| c04 | IVAS | 512.0 | Off | No | No (orientation only) |
| c05 | IVAS | 64.0 | Off | Yes | Yes |
| c06 | IVAS | 512.0 | Off | Yes | Yes |

- G.19	Experiment BS1534-14: ISM 6 DoF (4 obj.)

Conditions (BS1534-14 Generic Audio)

| Main Codec Conditions |  |
| --- | --- |
| Codec under Test (CuT) | IVAS [FX/FL] operated with ISM (4 obj.) audio input with extended metadata enabled at 64 kbps, 512 kbps DTX off at 0% FER Rendering via the IVAS-internal rendering |
| Codec references |  |
| Codec references | IVAS operation with extended metadata disabled in the encoder and listener position set to zero in the decoder. 64 kbps, 512 kbps DTX off at 0% FER Rendering via the IVAS-internal rendering |
| Other references |  |
| Reference | Direct signal, non-quantized (direct) extended metadata, rendered with IVAS external renderer |
| Hidden Reference | Direct signal, non-quantized (direct) extended metadata, rendered with IVAS external renderer |
| LP3k5 anchor | 3.5 kHz lowpass filtered signal, non-quantized (direct) extended metadata, rendered with IVAS external renderer |
| Common Conditions |  |
| Test item generation | According to material collection procedure for IVAS selection BS.1534 tests. |
| Audio sampling frequency/bandwidth | 48 kHz/FB |
| Input frequency mask | 20KBP |
| Nominal output loudness | -26 LKFS |
| Listening Level | Adjusted by listener |
| Listeners | Experienced Listeners |
| Randomizations | Individual per listeners |
| Rating Scale | Continuous BS.1534 scale from 0-100 |
| Listening System | High-quality headphone for diotic presentation, in accordance with clause XXX |
| Listening Environment | No room noise |

References

[1]	S4-240367, Pdoc IVAS-8b: Test plan for Characterization Phase, v.0.2.0