Source:	Rapporteur

Title:	ISAR Pdoc on Testing Aspects for Phase/Track 2/a

Version:	v0.1.1

Document for:	Discussion and Agreement

Agenda Item:	2.8

Document History:

| v0.1.0 | 2 February 2024 | Initial version (high-level) |
| --- | --- | --- |
| v0.1.1 | 1 March 2024 | First detailed version for presentation at Audio SWG call, 1 March 2024 |

Introduction

This Permanent Document describes test plan aspects for the selection testing of IVAS specific ISAR solutions targeted in Phase/Track 2/a of the ISAR work [1]. It covers the organization of the selection tests and the relevant processing and test plan aspects.

References, Conventions, and Contacts

Related ISAR Documents

The following documents provide additional information on the selection of IVAS specific ISAR solutions targeted in Phase/Track 2/a .

| Document | Title |
| --- | --- |
| TR 26.865 | Immersive Audio for Split Rendering Scenarios; Requirements |
| S4-240403 | Work Plan for ISAR |
| This document | Pdoc: Testing Aspects for Phase/Track 2/a |
| S4-240398 | Pdoc: Selection Deliverables for Phase/Track 2/a |
| S4-240397 | Pdoc: Selection Rules for IVAS Specific ISAR Solutions |

Reference Documents

[1]	Tdoc S4-240403: Work Plan for the ISAR v0.5.0

[2] 	Tdoc S4-240254: Trajectory Nullification for Binaural Renderer Evaluation, Fraunhofer IIS

[3]	Tdoc S4aA230086: IVAS Permanent Document IVAS-8a: Test Plan for Selection Phase, v.1.1.0

[4]	Recommendation ITU-R BS.1534 (10/2015): Method for the subjective assessment of intermediate quality level of audio systems.

[5]	3GPP TR 26.865: Immersive Audio for Split Rendering Scenarios; Requirements

[6]	Recommendation ITU-R BS.1770-4 (10/2015): Algorithms to measure audio programme loudness and true-peak audio level.

[7]	Tdoc S4aA230087: IVAS Permanent Document IVAS-7a: Processing plan for selection phase, v.1.1.0

Key Acronyms

CLL		Cross-check Listening Laboratory

CuT		Codec under Test

FB		Full Band

HOA3		Higher-Order Ambisonics, 3rd order

IVAS		Immersive Voice and Audio Services

ISM		Independent Streams with Metadata

LL		Listening Laboratory

MASA		Metadata-Assisted Spatial Audio

MUSHRA	Multi Stimulus test with Hidden Reference and Anchor

PCO		Processing Cross-check Organization

SP		Solution Proponent

SPL 		Sound Pressure Level

TC		Transport Channel

Organization of tests

Overview of the Selection Test Process

The selection tests of the IVAS specific ISAR solution will be organized as in-house tests by Solution Proponents Laboratories. The execution of these subjective tests is under the responsibility of the solution proponents participating in the selection and other volunteering organizations.

The selection test experiments will be duplicated and additionally run by suitable cross-checkers, Cross-check Listening Laboratories, with no stake in the candidate solutions under test.

The selection test results will be reported by the testing organizations to SA4 with suggested statistical result analysis. SA4 will review these analyses and, if found valid, confirm them to base its selection decision on them.

The processing of the selection test material is under the responsibility of the solution proponents participating in the selection. It is based on commonly available processing scripts, solution candidate executables, original sound material and head-tracker trajectories available to the solution proponents and other organizations who volunteer to carry out cross-checks.

Responsibilities

Solution Proponents (SP)

The specific responsibilities of each SP are:

Make executable of solution candidate publicly available or share with other SPs and Volunteering Processing Cross-check Organizations.

Develop common processing scripts using the condition lists defined in this document and the processing steps defined in the processing plan.

Process the test material using commonly available processing scripts, the shared solution candidate executables, original sound material and head-tracker trajectories.

Communicate with other SPs and Volunteering Processing Cross-check Organizations to verify correct processing.

Solution Proponents Laboratory (SPL)

Carry out selection tests according to the requirements of this test plan.

Carry out statistical result analysis according to the requirements of this test plan and provide test report including analysis to SA4.

Obligations as SPL:

The testing shall be caried out in a blinded fashion not revealing the conditions to the subjects.

No test subjects must be used that were actively involved in developing the split rendering features in the systems under test that are exposed by the experiments.

The test report shall describe how the listening lab ensured unbiased testing.

Volunteering Cross-check Listening Laboratories (CLL)

Carry out selection tests according to the requirements of this test plan.

Carry out statistical result analysis according to the requirements of this test plan and provide test report including analysis to SA4.

Obligations as CLL:

The testing shall be caried out in a blinded fashion not revealing the conditions to the subjects.

The CLL shall not be contributor of the split rendering features in the systems under test that are exposed by the experiments.

The test report shall contain a statement confirming that the listening lab has met the obligations.

Listening Laboratories (LL) (both SPL and CLL)

Provide a listening environment meeting the listening conditions for BS.1534 testing [4].

Volunteering Processing Cross-check Organizations (PCO)

Process the test material using commonly available processing scripts, the shared solution candidate executables, original sound material and head-tracker trajectories.

Communicate with Solution Proponents to verify their correct processing.

SA4

Review selection test analyses received from LLs and determine their validity.

Selection of Candidate Solution according to selection rules for IVAS specific ISAR solutions targeted in Phase/Track 2/a of the ISAR work [1].

Statistical analysis of results

The statistical result analysis reports shall present the results of the Terms of Reference (ToR) tests using Student’s Dependent Groups t-test (single-sided at 95% confidence level). Results of the Requirement ToR tests for each experiment shall be presented containing all relevant data allowing to verify proper execution of the Student’s Dependent Groups t-test.

In the for Requirement ToR tests this should lead to the following indications:

Requirement ToR tests that are passed, (i.e., CuT “not worse than” Requirement) are indicated by CuT NWT Ref.

Requirement ToR tests that are exceeded, (i.e., CuT “better than” Requirement) are indicated by CuT BT Ref.

Requirement ToR tests that are failed (i.e., CuT “worse than” Requirement) are indicated by CuT WT Ref.

The statistical result analysis reports shall also present the results a test comparing the two solution candidates (CuT1 and CuT2) against each other using Student’s Dependent Groups t-test (two-sided at 95% confidence level). Results of the comparative test for each experiment shall be presented containing all relevant data allowing to verify proper execution of the Student’s Dependent Groups t-test.

In the CuT Comparison tests this should lead to the following indications:

CuT1 BT CuT2.

CuT1 WT CuT2.

CuT1 and CuT2 are equivalent.

Identities

In the following, the identities of the involved organizations are listed:

SP1 (proponent of CuT1):

Proponent companies:
Dolby Sweden AB, Ericsson LM, Fraunhofer IIS, Nokia Corporation, NTT, Orange, Panasonic Holdings Corporation, Philips International B.V., Qualcomm Incorporated, VoiceAge Corporation

Main contributor(s) to CuT1 whose split rendering features are exposed in the experiments:
[Dolby Sweden AB, Fraunhofer IIS]

SP2 (proponent of CuT2):

Proponent companies:
Huawei

Main contributor(s) to CuT2 whose split rendering features are exposed in the experiments:
[Huawei]

SPLs:

Dolby Sweden AB, Fraunhofer IIS, Huawei

CLLs:

[tba]

Processing Cross-check Organizations (PCO)

[tba]

Information relevant to all Experiments

General Technical Notes

Any and all deviations from the specifications contained in this document must be documented and submitted to SA4 along with the test reports.

General Consideration of Experiments

Difference scenario between assumed and actual end-device poses

- For the evaluation of ISAR split rendering solution, a primary focus should be the testing with relevant difference scenarios between assumed and actual end-device poses. To cover relevant cases, the used head-tracker trajectory files should be taken from the following categories:

Static within range: +-20 degrees

Dynamic within range: +-20 degrees

Sinusoidal: 0.25 Hz

Triangular: 0.5 Hz

Real, i.e., derived from real head tracker trajectories with movements giving rise to substantial differences (>15 degrees) between assumed and actual end-device poses.

DOF

- Another aspect of interest is the ability to deal with differences of assumed and actual end-device poses around different axes, i.e., the number of degrees of freedom (1-3) which the candidate solutions can cope with. Accordingly, the head-tracker trajectory files shall cover the following DOF scenarios:

1-DOF with pose deviations in yaw

2-DOF with pose deviations in yaw and pitch

3-DOF with pose deviations in yaw, pitch and roll

Rendering simulation

- Two different rendering simulation methodologies shall be covered in the tests.

Trajectory nullification [2]
This simulation methodology is based on the concept that an immersive audio scene is pre-rotated prior to IVAS encoding while the head-tracked rendering compensates for the pre-rotation. In the ideal case and under certain conditions, this compensation can be perfect. This simulation methodology exposes the ability of a split rendering system to compensate for the pre-rotation despite the differences between assumed and actual end-device poses.

Unguided end-device pose
This simulation methodology is based on rendering a decoded an immersive audio scene according to a given head-tracker trajectory. This simulation methodology exposes the ability of a split rendering system to follow the actual head-tracker trajectory despite the mere availability of the divergent assumed head-tracker trajectory available at the pre-renderer.

Input formats

- According to the ISAR requirements, the tests shall cover the following IVAS codec input formats:

SBA (HOA3)

MASA (2 TCs)

Multi-channel (7.1.4)

Objects (ISM-4)

Methodology

BS.1534 test methodologies shall be used in the ISAR selection tests. High-level configuration of the experiments is outlined below.

BS.1534

Number of items per experiment: 12

8[-12] experienced listeners

Total number of conditions: 5 (assuming two solution candidates)

Number of anchor conditions: 2

Direct

7 kHz low-pass anchor

Note: the exact number of conditions depends on the number of solution candidates to be evaluated.

Head-tracker trajectories

All head-tracker trajectory files shall follow the convention imposed by the IVAS specifications [ref IVAS TS 26.258(?)], i.e., shall be usable by the IVAS decoder/renderer.

Head-tracker trajectory categories and DOF

- Head-tracker trajectories shall meet the above-defined category and DOF specifications.

Head-tracker trajectory availability and selection

- Head-tracker trajectories of the above-defined categories and DOF will be publicly collected from volunteering organizations. After collection and checking suitability, a list of available trajectories will be generated. The collection and checking will be done jointly by the involved organizations of the ISAR selection, i.e., the SPs, the LLs and the PCOs.

- In a second step, these organizations will jointly select up to [N] suitable trajectories for each test. In case this number of trajectories is not available, a smaller number is selected where a given trajectory may be reused across different tests or within a test.

- The involved organizations will document their trajectory selection and assignment to tests for inclusion of this information into this document.

Audio Material

All audio material shall be sampled at 48 kHz with Full Band (FB) content and formatted as 16-bit little endian WAVE format files [25].

Audio categories

- To cover a broad range of conceivable audio categories, the test items should be taken from the categories clean and noisy speech, music, critical audio. However, the ability of the system to deal with different audio categories is only a secondary focus of the selections tests and full coverage of these categories may not be possible.

Test Item availability and selection

- Audio material of the above categories has been collected as part of the IVAS codec selection phase. Details of this material are available in the IVAS test plan [3]. A subset of this material is either publicly available or at least available to the involved organizations of the ISAR selection, i.e., the SPs, the LLs and the PCOs. These organizations will in a first step create a list of commonly available test items.

- In a second step, these organizations will jointly select up to 12 suitable original test items for each test. In case this number of test items is not available, a smaller number is selected where a given test item may be reused across different tests or even within a test if the applied head-tracker trajectories, DOF or simulation methodology is different.

- The involved organizations will document their test item selection and assignment to tests for inclusion of this information into this document.

Training material

No dedicated training material will be made available for use in a potential training phase in which the subjects may familiarize with the testing methodology and environment.

Such a training phase is voluntary and upon the own responsibility of the involved LLs. No items from the main tests shall be used for training. A training phase shall be executed as a separate short BS.1534 session.

Listening Systems and Listening Environments

The ISAR Selection Test will use the following listening systems:

High-quality stereo headphones for binaural listening, e.g.:

Sennheiser HD 650

Subjective Experiments

The purpose of the 4 experiments (Experiments BS1534-1 – BS1534-4) is to evaluate the performances of the IVAS specific ISAR solution candidates with respect to the performance requirements and objectives defined in ISAR TR 26.865 [5].

The details provided in this section and in corresponding Annexes are those that are specific to each particular experiment. Generic information can be found in Section 4. Therefore, the LLs should use the information in Section 4 in conjunction with the information given in this section and Annexes.

Table 4 shows high-level overview of the experiments.

Detailed conditions for each subjective experiment are defined in Annex B.

Table 4: High-level overview of ISAR selection experiments

| Exp | Input format | Source material | Listening environment | Bitrates kbps |
| --- | --- | --- | --- | --- |
| BS1534-1 | SBA (HOA3) | Generic Audio | Headphones | IVAS: 512, CuT: 768 |
| BS1534-2 | Multi-channel 7.1+4 | Generic Audio | Headphones | IVAS: 512, CuT: 768 |
| BS1534-3 | Objects (ISM-4) | Generic Audio | Headphones | IVAS: 512, CuT: 768 |
| BS1534-4 | MASA (2 TC) | Generic Audio | Headphones | IVAS: 512, CuT: 768 |

Selection Testing Timeline

Table H.1: Testing Timeline

| Month | Meeting/date | Task | Active Parties |
| --- | --- | --- | --- |
| March-2024 | March 1 | 3GPP SA4 Audio SWG telco | Audio SWG |
|  | March 9 | Collection, checking and selection of head-tracker trajectories and assignment to tests completed | SPs, LLs, PCOs |
|  | March 9 | Identification and selection of test items and assignment to tests completed | SPs, LLs, PCOs |
|  | March 9 | Provision of processing scripts | SPs, PCOs |
|  | March 15 | Selection test processing completed including cross-checking | SPs, LLs, PCOs |
|  | March 18 | 3GPP SA4 Audio SWG telco | Audio SWG |
|  | March 19 | Start of listening tests | LLs |
| April-2024 | April 2 | Delivery of LL reports | LLs |
|  | April 8-12 | 3GPP SA4 meeting #127bis-e – IVAS specific ISAR solution Selection meeting | SA4 |

BS.1534 Experiments

Experiment BS1534-1: SBA (HOA3)

Table F.1.1: Conditions (BS1534-1 Generic Audio)

| Main Conditions |  |
| --- | --- |
| Condition under Test (CuT1) | IVAS operated with HOA3 audio input at 512 kbps rendered to pre-renderer pose, pose corrected to post renderer pose with ISAR candidate 1 operated at 768 kbps |
| Condition under Test (CuT2) | IVAS operated with HOA3 audio input at 512 kbps rendered to pre-renderer pose, pose corrected to post renderer pose with ISAR candidate 2 operated at 768 kbps |
| References (Hidden and open) |  |
| Reference | (Native coding system) IVAS operated with HOA3 audio input at 512 kbps rendered to post-renderer pose |
| Other references |  |
| LP7 anchor | 7 kHz lowpass filtered reference, nominal level |
| 0-DOF native transcoding reference | IVAS operated with HOA3 audio input at 512 kbps rendered to pre-renderer pose, binaural output transcoded with IVAS stereo coded@256kbps |
| Common Conditions |  |
| Test item generation | According to material collection procedure for ISAR selection BS.1534 tests. |
| Audio sampling frequency/bandwidth | 48 kHz/FB |
| Input frequency mask | 20KBP |
| Nominal output loudness | -26 LKFS [6] |
| Listening Level | Adjusted by listener |
| Listeners | Experienced Listeners |
| Randomizations | Individual per listeners |
| Rating Scale | Continuous BS.1534 scale from 0-100 [4] |
| Listening System | High-quality headphone for diotic presentation, in accordance with clause 4.6 |
| Listening Environment | No room noise |

Table F.1.2: Test conditions for Experiment BS1534-1

| Label | Condition | Bitrate [kbps] | ToR |
| --- | --- | --- | --- |
| c01 | Reference | 512 | - |
| c02 | LP7 anchor | 512 | - |
| c03 | 0-DOF | 512, 256 | - |
| c04 | CuT1 | 512, 768 | NWT c03 |
| c05 | CuT2 | 512, 768 | NWT c03 |

Experiment BS1534-2: Multi-channel 7.1+4

Table F.2.1: Conditions (BS1534-2 Generic Audio)

| Main Conditions |  |
| --- | --- |
| Condition under Test (CuT1) | IVAS operated with 7.1+4 audio input at 512 kbps rendered to pre-renderer pose, pose corrected to post renderer pose with ISAR candidate 1 operated at 768 kbps |
| Condition under Test (CuT2) | IVAS operated with 7.1+4 audio input at 512 kbps rendered to pre-renderer pose, pose corrected to post renderer pose with ISAR candidate 2 operated at 768 kbps |
| References (Hidden and open) |  |
| Reference | (Native coding system) IVAS operated with 7.1+4 audio input at 512 kbps rendered to post-renderer pose |
| Other references |  |
| LP7 anchor | 7 kHz lowpass filtered reference, nominal level |
| 0-DOF native transcoding reference | IVAS operated with 7.1+4 audio input at 512 kbps rendered to pre-renderer pose, binaural output transcoded with IVAS stereo coded@256kbps |
| Common Conditions |  |
| Test item generation | According to material collection procedure for ISAR selection BS.1534 tests. |
| Audio sampling frequency/bandwidth | 48 kHz/FB |
| Input frequency mask | 20KBP |
| Nominal output loudness | -26 LKFS [6] |
| Listening Level | Adjusted by listener |
| Listeners | Experienced Listeners |
| Randomizations | Individual per listeners |
| Rating Scale | Continuous BS.1534 scale from 0-100 [4] |
| Listening System | High-quality headphone for diotic presentation, in accordance with clause 4.6 |
| Listening Environment | No room noise |

Table F.2.2: Test conditions for Experiment BS1534-2

| Label | Condition | Bitrate [kbps] | ToR |
| --- | --- | --- | --- |
| c01 | Reference | 512 | - |
| c02 | LP7 anchor | 512 | - |
| c03 | 0-DOF | 512, 256 | - |
| c04 | CuT1 | 512, 768 | NWT c03 |
| c05 | CuT2 | 512, 768 | NWT c03 |

Experiment BS1534-3: Objects

Test with 4 Objects (ISM-4) in BS1534-3 at high bitrate.

Table F.3.1: Conditions (BS1534-3 Generic Audio)

| Main Conditions |  |
| --- | --- |
| Condition under Test (CuT1) | IVAS operated with 4 Objects audio input at 512 kbps rendered to pre-renderer pose, pose corrected to post renderer pose with ISAR candidate 1 operated at 768 kbps |
| Condition under Test (CuT2) | IVAS operated with 4 Objects audio input at 512 kbps rendered to pre-renderer pose, pose corrected to post renderer pose with ISAR candidate 2 operated at 768 kbps |
| References (Hidden and open) |  |
| Reference | (Native coding system) IVAS operated with 4 Objects audio input at 512 kbps rendered to post-renderer pose |
| Other references |  |
| LP7 anchor | 7 kHz lowpass filtered reference, nominal level |
| 0-DOF native transcoding reference | IVAS operated with 4 Objects audio input at 512 kbps rendered to pre-renderer pose, binaural output transcoded with IVAS stereo coded@256kbps |
| Common Conditions |  |
| Test item generation | According to material collection procedure for ISAR selection BS.1534 tests. |
| Audio sampling frequency/bandwidth | 48 kHz/FB |
| Input frequency mask | 20KBP |
| Nominal output loudness | -26 LKFS [6] |
| Listening Level | Adjusted by listener |
| Listeners | Experienced Listeners |
| Randomizations | Individual per listeners |
| Rating Scale | Continuous BS.1534 scale from 0-100 [4] |
| Listening System | High-quality headphone for diotic presentation, in accordance with clause 4.6 |
| Listening Environment | No room noise |

Table F.3.2: Test conditions for Experiment BS1534-3

| Label | Condition | Bitrate [kbps] | ToR |
| --- | --- | --- | --- |
| c01 | Reference | 512 | - |
| c02 | LP7 anchor | 512 | - |
| c03 | 0-DOF | 512, 256 | - |
| c04 | CuT1 | 512, 768 | NWT c03 |
| c05 | CuT2 | 512, 768 | NWT c03 |

Experiment BS1534-4: MASA

Table F.4.1: Conditions (BS1534-4 Generic Audio)

| Main Conditions |  |
| --- | --- |
| Condition under Test (CuT1) | IVAS operated with MASA (2 TC) audio input at 512 kbps rendered to pre-renderer pose, pose corrected to post renderer pose with ISAR candidate 1 operated at 768 kbps |
| Condition under Test (CuT2) | IVAS operated with MASA (2 TC) audio input at 512 kbps rendered to pre-renderer pose, pose corrected to post renderer pose with ISAR candidate 2 operated at 768 kbps |
| References (Hidden and open) |  |
| Reference | (Native coding system) IVAS operated with MASA (2 TC) audio input at 512 kbps rendered to post-renderer pose |
| Other references |  |
| LP7 anchor | 7 kHz lowpass filtered reference, nominal level |
| 0-DOF native transcoding reference | IVAS operated with MASA (2 TC) audio input at 512 kbps rendered to pre-renderer pose, binaural output transcoded with IVAS stereo coded@256kbps |
| Common Conditions |  |
| Test item generation | According to material collection procedure for ISAR selection BS.1534 tests. |
| Audio sampling frequency/bandwidth | 48 kHz/FB |
| Input frequency mask | 20KBP |
| Nominal output loudness | -26 LKFS [6] |
| Listening Level | Adjusted by listener |
| Listeners | Experienced Listeners |
| Randomizations | Individual per listeners |
| Rating Scale | Continuous BS.1534 scale from 0-100 [4] |
| Listening System | High-quality headphone for diotic presentation, in accordance with clause 4.6 |
| Listening Environment | No room noise |

Table F.4.2: Test conditions for Experiment BS1534-4

| Label | Condition | Bitrate [kbps] | ToR |
| --- | --- | --- | --- |
| c01 | Reference | 512 | - |
| c02 | LP7 anchor | 512 | - |
| c03 | 0-DOF | 512, 256 | - |
| c04 | CuT1 | 512, 768 | NWT c03 |
| c05 | CuT2 | 512, 768 | NWT c03 |

Processing plan

The processing of the test material for the ISAR selection tests is conceptually following the IVAS selection processing [7]. In particular, pre-processing steps and post-processing steps are identical to those described in respective sections 4.3 and 4.10 of [7].

The main processing is described below.

Note: Irrespective of this processing plan, the processing is defined by the publicly available processing scripts. In case of deviations between this plan and the scripts, the implementation of the script will prevail.

Reference conditions

This condition is the IVAS codec operated with the audio input in the format of the given experiment, encoded at 512 kbps and rendered to post-renderer pose.

Assuming a head-tracker trajectory file at the lightweight device called ‘post.csv’, the respective command lines are:

Experiment BS.1534-1: SBA

ivas_cod.exe -sba 3 512000 48 hoa3.wav enc_out.pkt

ivas_dec.exe -t post.csv BINAURAL 48 enc_out.pkt ref.wav

Experiment BS.1534-2: Multi-channel 7.1+4

ivas_cod.exe -mc 7_1_4 512000 48 mc714.wav enc_out.pkt

ivas_dec.exe -t post.csv BINAURAL 48 enc_out.pkt ref.wav

Experiment Experiment BS.1534-3: Objects

ivas_cod.exe -ism 4 ism_Obj1.csv ism_Obj2.csv ism_Obj3.csv ism_Obj4.csv 512000 48  ism4.wav enc_out.pkt

ivas_dec.exe -t post.csv BINAURAL 48 enc_out.pkt ref.wav

Experiment Experiment BS.1534-4: MASA

ivas_cod.exe -masa 2 masa.metadata  512000 48 masa2ch.wav enc_out.pkt

ivas_dec.exe -t post.csv BINAURAL 48 enc_out.pkt ref.wav

7 kHz anchor conditions

The 7 kHz anchors are obtained by 7 kHz lowpass filtering of the processed reference conditions before post processing.

0-DOF native transcoding reference conditions

As under C.1 with the difference that the IVAS decoder command lines are replaced by

ivas_dec.exe -t pre.csv BINAURAL 48 enc_out.pkt out_0dof.wav

ivas_cod.exe -stereo 256000 48 out_0dof.wav  stereo_0dof.pkt

ivas_dec.exe STEREO 48 stereo_0dof.pkt  native_ref_0dof.wav

Note: the used head-tracker trajectory file ‘pre.csv’ is the assumed head-tracker pose at the pre-renderer.

CuT conditions

As under C.1 with the difference that the IVAS decoder command lines are replaced by

IVAS decoder (in split pre rendering mode):

ivas_dec.exe -render_config  split_renderer_config_768_3dof_cldfbpc.txt -t pre.csv BINAURAL_SPLIT_CODED 48 enc_out.pkt split_out.pkt

IVAS ext renderer(in split post rendering mode):

ivas_rend.exe -i split_out.pkt -if BINAURAL_SPLIT_CODED -o out.wav -of BINAURAL -fs 48 -t post.csv

Note: The two SPs are expected to coordinate about how to use these command lines and may agree on modifications.

Stefan Bruhn, Dolby Sweden AB; email: stefan.bruhn@dolby.com