3GPP TSG-SA4 Meeting #101	S4-181324

# Busan, Korea (Republic Of), 19th Nov 2018 - 23rd Nov 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.247 | CR | 0160 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Quotation correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile Com. Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2018-11-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The QoE mertics is defined in 10.2, not in 10.3 and 10.4 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the reference of QoE metrics |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion exists |  |  |  |  |  |  |  |  |
| Clauses affected: |  | K.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### ========================= Start of change ==========================

The mapping between QoE metrics and input to ITU-T P.1203 [49] Mode 0 is provided in table K-1 to K-4 below.

Table K-1 mapping between QoE metrics and input in ITU-T P.1203 mode 0 model for video stream

| Video Metrics needed for ITU-T P.1203[49] model |  | QoE metrics defined in clause 10.2 |  | Remark |
| --- | --- | --- | --- | --- |
| Description | Value | Metric | Description |  |
| Target Video bit-rate | Bit-rate in kbps. | Mpdinfo | Provides the MPD information for the representation or subrepresentation identified by representationid and subreplevel, if present. Related attributes: @bandwidth, @width, @height, and @codecs. Note: codec attribute includes video codec profile information and video frame rate information | Target Video bit-rate has been supported by @bandwidth attribute. Video frame-rate and Video codec and profile have been supported by @ codec attribute. Video encoding resolution has been supported by @width and @height attributes. |
| Video frame-rate | Frame rate in frames per second. |  |  |  |
| Segment duration | Duration in seconds |  |  |  |
| Video encoding resolution | Number of pixels (WxH) in transmitted video |  |  |  |
| Video codec and profile | One of: H264-baseline, H264-high, H264-main |  |  |  |
|  |  | Play List | Provides information on when and how long different representations or subrepresentations have been used during the session. | Note that the actual timing of the required input for the P.1203 quality model may be influenced by actions such as play/pause. |

Table K-2 mapping between QoE metrics and input in ITU-T P.1203 mode 0 model for audio stream

| Audio Metrics needed for ITU-T P.1203 model |  | QoE metrics defined in clause 10.2 |  | Remark |
| --- | --- | --- | --- | --- |
| Description | Value | Metric | Description |  |
| Target Audio bit-rate | Bit-rate in kbps. | Mpdinfo | Provides the MPD information for the representation or subrepresentation identified by representationid and subreplevel, if present. Related attributes: @bandwidth, and @codecs. Note: codec attribute includes audio codec profile information, audio sampling frequency and Number of audio channels information | Target Audio bit-rate has been supported by @bandwidth attribute. Audio codec, Audio sampling frequency and Number of audio channels have been supported by @ codecs attribute. |
| Segment duration | Duration in seconds |  |  |  |
| Audio codec | One of: AAC-LC, AAC-HEv1, AAC-HEv2, AC3 |  |  |  |
| Audio sampling frequency | In Hz |  |  |  |
| Number of audio channels | 2 |  |  |  |
|  |  | Play List | Provides information on when and how long different representations or subrepresentations have been used during the session. | Note that the actual timing of the required input for the P.1203 quality model may be influenced by actions such as play/pause. |

Table K-3 mapping between QoE metrics and input in ITU-T P.1203 mode 0 model for stalling

| Metrics needed for ITU-T P.1203 model |  | QoE metrics defined in clause 10.2 |  | Remark |
| --- | --- | --- | --- | --- |
| Description | Value | Metric | Description |  |
| Buffering event start | The start time of the buffering/stalling event in seconds relative to the start of the original video clip, expressed in media time (not wall clock time) Note: This is 0 for initial buffering. | Play List | The start time of a buffering/stalling event is equal to the end time of a trace entry with the stop reason equal to "rebuffering” | - |
| Event duration | The duration of the buffering/stalling event in seconds. | Play List | The duration of each buffer is calculated from the PlayList according to the example described in the Play List metric clause. | - |

Table K-4 mapping between QoE metrics and input in ITU-T P.1203 mode 0 model for device information

| Metrics needed for ITU-T P.1203 model |  | QoE metrics defined in clause 10.2 |  | Remark |
| --- | --- | --- | --- | --- |
| Description | Value | Metric | Description |  |
| The resolution of the image displayed to the user | Number of pixels (WxH) in displayed video | Device Information | videoWidth and videoHeight | . |
| The device type on which the media is played | pc or mobile | Device Information | screenWidth x pixelWidth, screenHeight x pixelHeight | P.1203 classifies devices up to 12" as mobiles |
| The size of the video shown | WxH in mm | Device Information | videoWidth x pixelWidth, videoHeight x pixelHeight |  |