3GPP TSG-SA4 Meeting #101	S4-181367

# Busan, Korea,  19.11. – 23.11.2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.929 | CR | <CR#> | rev | <Rev#> | Current version: | 0.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Impact of Content Complexity on QoE for VR Videos |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Deutsche Telekom AG |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_QoE_VR |  |  |  |  |  | Date: |  |  | 15.9.2017 |
| Category: | C |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | During SA4#100 it was agreed to consider the proposed content in S4-181114 to be added to TR26.929 |  |  |  |  |  |  |  |  |
| Summary of change: |  | The proposed section elucidate the influence of the content on the Quality of Experience and substantiats the need for QoE Metric for content cpmplexity |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Essential aspect for VR content impact on QoE missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### **** First Change ****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	3GPP TR 26.918: "Virtual Reality (VR) media services over 3GPP".

[3] 	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[4]	ISO/IEC 23009-1: 2014/Amd. 1:2015/Cor.1:2015" Information technology -- Dynamic adaptive streaming over HTTP (DASH) -- Part 1: Media presentation description and segment formats".

[5]	ITU-T P.1203: "Parametric bitstream-based quality assessment of progressive download and adaptive audiovisual streaming services over reliable transport".

[6]	A. Singla, S. Fremerey, W. Robitza, A. Raake, “Measuring and comparing QoE and simulator sickness of omnidirectional videos in different head mounted displays”, in 9th International Conference on Quality of Multimedia Experience (QoMEX), May 2017.

[7]	A. Singla, A. Raake, W. Robitza, P. List, B. Feiten, “AhG8: Subjective Quality Evaluation for Omnidirectional (360°) Videos”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-G0152, 7th Meeting, July 2017.

[8]	ITU-T P.910, “Subjective video quality assessment methods for multimedia applications”,                   International Telecommunication Union, April 2008.

[9]	ITU-R BT.500-13, “Methodology for the subjective assessment of the quality of television pictures”, International Telecommunication Union, Jan. 2012.

[10]	R. S. Kennedy, N. E. Lane, K. S. Berbaum, and M. G. Lilienthal, “Simulator sickness questionnaire: An enhanced method for quantifying simulator sickness,” The International Journal of Aviation Psychology, 1993.

[11]	E. Asbun, Y. He, Y. He, “AHG8: InterDigital Test Sequences for Virtual Reality Video Coding”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-D0039, 4th Meeting, Oct. 2016.

[12]	S. Schwarz et. al., “Tampere pole vaulting sequence for virtual reality video coding”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-D0143, 4th Meeting, Oct. 2016.

[13]	W. Sun, R. Guo, “Test Sequences for Virtual Reality Video Coding from LetinVR”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-D0179, 4th Meeting, Oct. 2016.

### **** Second Change, adding section 7.2  ****

## 7.2	Impact of Content Complexity on QoE

### 7.2.1	Introduction

QoE provided by the immersive technologies such 360-degree videos play an important role how much users are going to interact with the technology. Therefore, there is a need to assess the QoE of the new emerging technology, as QoE is one of the contributing factors in making the technology successful.

In this work, the influence of resolutions, camera motion, motion in the content, and simulator sickness on QoE is investigated. Some of the users are prone to simulator sickness, therefore, it is of interest to investigate how the simulator sickness interacts with the QoE and vice-versa.

### 7.2.2	Preparation of datasets

The dataset was downloaded from the Internet in two resolutions, 4K and FHD (see Table 7.1). Choosing 4K and FHD was motivated by the resolution limitation of the HMDs. The resolution of both devices is 2160×1200. The reason for downloading the dataset from the Internet was that the duration of these video sequences could be chosen much longer compared to the standard dataset [11–13]. We also wanted to use off-the-shelf contents. Table 7.1 provides an overview of the content. We downloaded the H.264/AVC encoded video sequences, which were then cut to the duration of 60-65 seconds with XMedia Recode without changing the quality of the video [6,7]. The video files are publicly available at https://zenodo.org/record/571065#.WVkpR4iGOUl.

### 7.2.3	Technical setup and equipment

Two HMDs were used from two different companies – named HMD1and HMD2  here. The resolution and field of view (FOV) for both devices are 2160×1200 and 110° respectively. Whirligig player (version 3.89) was used in order to display the 360° videos in both HMDs. The HMDs were connected to a desktop PC equipped with an NVIDIA GTX980 graphics card and an Intel Core i7 processor. The names of the HMDs were hidden to the subjects to decrease contextual effects [6,7].

Table 7.1: Description of the Dataset [6,7]

| No. | Name | Resolution | FPS | Bit-rate (in mbps) | Timestamp |
| --- | --- | --- | --- | --- | --- |
| 1 | Mega Coaster | 3840×2048 | 30 | 17.7 | 00.10 – 01:10 |
|  |  | 1920×1080 |  | 3.7 |  |
| 2 | Project 360 | 3840×2160 | 30 | 17.4 | 01:36 – 02:40 |
|  |  | 1920×1080 |  | 3.4 |  |
| 3 | 360 Cockpit View | 3840×1920 | 25 | 9.8 | 07:35 – 08:40 |
|  |  | 1920×1080 |  | 3.6 |  |
| 4 | Sky Diving in 360 | 3840×1920 | 29 | 16.4 | 03:22 – 04:25 |
|  |  | 1920×1080 |  | 4.5 |  |
| 5 | Reimagine – Etihad A380 | 3840×2048 | 29 | 8.5 | 03:29 – 04:31 |
|  |  | 1920×1080 |  | 2.4 |  |
| 6 | Surrounded by Wild Elephants | 3840×2048 | 30 | 13.0 | 01:19 – 02:24 |
|  |  | 1920×1080 |  | 3.5 |  |

### 7.2.4	Test Method

For assessing the QoE of 360° videos, the Absolute Category Ratings (ACR) [8] method was used. Subjects were asked to rate the “Integral quality of VR representation”, and a paper-based scale was provided after each sequence. Instructions were given to the subjects not to consider stitching and ghosting artifacts while rating the videos. A total of 28 subjects participated in the subjective test. Out of 28, 15 were female and 13 were male, with an average age of 26.25 and a median of 25. Prior to the experiment, subjects were screened for correct visual acuity using Snellen charts (20/20) and for color vision using Ishihara charts. A short training was performed at the beginning of each test session to familiarize the subjects with the test procedure, and to help adjusting the HMD(s) according to their head size and inter-ocular distance [6,7].

### 7.2.5	Results

#### 7.2.5.1	Audiovisual quality of the entire VR session

Outlier detection was performed on the raw scores of the subjects based on ITU-R Rec. BT.500-13 [9]. In this experiment, no outliers were found. Mean Opinion Score (MOS) along with the associated 95% Confidence Interval (CI) were computed for each test stimulus.

From Figure 7.1, it is clear that subjects were able to find a difference between the two resolutions irrespective of the device and content. Figure 7.2 shows the difference in the MOS for HMD2 and HMD1 for 4K and FHD resolution. For 4K resolution, HMD1 was slightly preferred over HMD2 for all the contents except content #5. For the FHD resolution, in tendency, the HMD2 was preferred for the contents #1, #3 and #5, but the difference was not significant. For contents #2 and #6, the difference in the MOS is considerable and for content #2 the difference is statistically significant [6,7].

It is interesting to note that video sequence “Project 360” (Content #2) which has the highest amount of motion due to the motion in the content and camera motion provides the least QoE irrespective of the device and resolution. Whereas, the video sequence “Surrounded by Wild Elephants” (Content #6) which has the least motion in the content provides the highest QoE irrespective of device at 4K resolution.

Figure 1. MOS with CIs obtained all devices and resolutions [6,7].

A one-way ANOVA was carried out on the individual ratings to analyze the impact of the resolution, device, and content on the users’ judgements. Results show that content and resolution have a significant impact on the users’ ratings with all p - values < 0.01, whereas the device has a slight but non-significant effect (p = 0.07) [6,7].

#### 7.2.5.2	Assessment of simulator sickness

While watching the 360° videos in HMD, users may experience symptoms of simulator sickness. Therefore, assessing the simulator sickness is a relevant additional step. Simulator sickness is a sub-category of motion sickness and symptoms comprise fatigue, sweating, vertigo, nausea, etc.

Figure 2. Difference in the MOS between devices for different resolutions [6,7].

[6,10]. The most popular Questionnaire for assessing the simulator sickness is Simulator Sickness Questionnaire (SSQ) published in 1993 [10], which we used for our experiment.

SSQ was derived from the motion sickness questionnaire (MSQ) [10], selecting 16 out of the original 28 symptoms for analysis. These symptoms are further classified into three sub-categories: nausea (N), oculomotor (O), and disorientation (D). Not all 16 symptoms are used for calculating N, O, D, and unit weights are assigned in each category. For obtaining the scores, a 4-point scale (0, 1, 2, 3) is used and weighted values are added to get the scores for each category. N, O, D, and Total Score (TS) are then calculated using the method shown in [10].

From Fig. 3 it is clear that for HMD1 for 4K resolution, content 2 leads to the highest simulator sickness scores. It is worth to note that content 2 lead to the lowest quality scores. Inversely, content 6 has the lowest simulator sickness scores and the highest QoE. For HMD2 for FHD resolution, content 2 lead to the highest simulator sickness scores among all devices and resolutions as well, and was judged to have the lowest quality among all devices and resolutions. These observations indicate that simulator sickness interacts with quality when 360° videos are watched in HMDs, or may be the cause for lower quality scores.

Figure 3. Simulator Sickness Scores for all devices and resolutions [6,7].

In order to find the influence of resolution on the simulator sickness scores, the difference between the simulator sickness scores of 4K and FHD have been computed for HMD1 and HMD2, as shown in Figure 4. It can be seen from the graph that – in case of HMD1– users are generally more prone to simulator sickness in FHD resolution as compared to 4K resolution. For HMD2, contents 3 and 6 show slightly lower simulator sickness scores for FHD [6,7].

An ANOVA was carried out on the Total Score of simulator sickness with content, resolution and gender as factors. Results indicate that content, resolution and gender as random factors. Results indicate that content, resolution and gender have a significant impact on the simulator sickness scores with all p-values < 0.01.

Figure 4.Difference in Simulator Sickness Scores depending on resolution [10].

### 7.2.6	Conclusion

In this contribution, we have investigated the influence of resolutions, camera motion, motion in the content, and simulator sickness on QoE. Results show that 4K/UHD provides better QoE as compared to FHD resolution. In addition, we have shown that camera motion and motion in the content interacts with the QoE. Simulator sickness has also an impact on the QoE and vice-versa. Content that has the lowest simulator sickness scores provides the maximum QoE to the users and vice-versa.

Hence it is essential to include detection of features for camera motion and motion in the content for deriving metrics that help to estimate simulator sickness and consequently the influence of simulator sickness on the VR QoE.